# Digital IPOS - Sensor-Hub

- Snapshot: snap-b2e8101b00dc6909feaed885

Author: Alessandro Lucio IANNUZZI

### 0.1 Table of contents

## 0. Document Navigation

- 1. Block overview

- 2. Source I/O

- 3. Block requirements

### 0.2 Document control

#### Table 1. Version history

| Version | Date | Description | Author |
| --- | --- | --- | --- |
| 0.1 | 2026-10-01 | Snapshot snap-b2e8101b00dc6909feaed885 digital IPOS block baseline | Alessandro Lucio IANNUZZI |

### 0.3 Requirement navigation

| Requirement | Internal link |
| --- | --- |
| IPOS-SENSOR-HUB-001 | Go to requirement |
| IPOS-SENSOR-HUB-002 | Go to requirement |
| IPOS-SENSOR-HUB-003 | Go to requirement |
| IPOS-SENSOR-HUB-004 | Go to requirement |
| IPOS-SENSOR-HUB-005 | Go to requirement |
| IPOS-SENSOR-HUB-006 | Go to requirement |
| IPOS-SENSOR-HUB-007 | Go to requirement |
| IPOS-SENSOR-HUB-008 | Go to requirement |
| IPOS-SENSOR-HUB-009 | Go to requirement |
| IPOS-SENSOR-HUB-010 | Go to requirement |
| IPOS-SENSOR-HUB-011 | Go to requirement |
| IPOS-SENSOR-HUB-012 | Go to requirement |
| IPOS-SENSOR-HUB-013 | Go to requirement |
| IPOS-SENSOR-HUB-014 | Go to requirement |
| IPOS-SENSOR-HUB-015 | Go to requirement |
| IPOS-SENSOR-HUB-016 | Go to requirement |
| IPOS-SENSOR-HUB-017 | Go to requirement |
| IPOS-SENSOR-HUB-018 | Go to requirement |

## 1. Block overview

### 1.1 Functionality

The Sensor-Hub block serves to operate the Sensor Hub I2C master for external targets, including trigger/control, pad enable, data collection, and FIFO multi-mode coordination.

### 1.2 Supported functions and scope

- The Sensor-Hub block accepts Primary sensing input. The Sensor-Hub block produces Analog input signal.

- The last 8-bits of the data stored in FIFO. The last 8-bits of the data stored in FIFO contains the TAG field.

- SLAVEx NUMOP bit-fields of SLAVEx CONFIG registers. SLAVEx NUMOP bit-fields of SLAVEx CONFIG registers defines how many bytes per slave are read as per effect of an I2C operation (x from 1 to 12).

- The output port sleep master. The output port sleep master is set to 1 when an I2C operation is not on-going.

- Only I2C target addressed through SLAVE0 DEV ADD register. Only I2C target addressed through SLAVE0 DEV ADD register is the destination of an I2C Write Operation.

- Each data stored in FIFO. Each data stored in FIFO is composed by 4 words (16bytes).

- All the SENSOR HUB x registers. All the SENSOR HUB x registers is reset every time a new I2C operation starts.

- 3 words (12bytes) of the data stored in FIFO. 3 words (12bytes) of the data stored in FIFO contains the data from each I2C slave.

- 24-bits of the data stored in FIFO. 24-bits of the data stored in FIFO is reserved at ‘0’.

- any dvalid triggered (HW or SW). When Quokka MasterBlaze µP runs (sleep master signal = 0), any dvalid triggered (HW or SW) is ignored.

- The trigger of sensor hub. The trigger of sensor hub is possible in both ways, HW via dvalid masterblaze ext signal or SW using DVALID REGISTER.

- SLAVE0 NUMOP bitfield. SLAVE0 NUMOP bitfield is ignored if an I2C Write operation is issued to the I2C Slave 0.

- All the values from 1 to 12. All the values from 1 to 12 is valid for SLAVEx NUMOP bit-fields of SLAVEx CONFIG registers.

- The AHB Write to registers has no effect if the IP masterblaze processor is busy (I2C operation on-going); the registers value. The AHB Write to registers has no effect if the IP masterblaze processor is busy (I2C operation on-going); the registers value is not updated.

## 2. Source I/O

The following approved source I/O entries are owned by this block.

| Port name | Direction | Type / details | Table | Source page |
| --- | --- | --- | --- | --- |
| scan_mode | input |  | Sensor Hub I/O List | 104 |
| scan_enable | input |  | Sensor Hub I/O List | 104 |
| scan_rstn | input |  | Sensor Hub I/O List | 104 |
| HCLKM | input |  | Sensor Hub I/O List | 104 |
| HCLKS | input |  | Sensor Hub I/O List | 104 |
| sys_clk | input |  | Sensor Hub I/O List | 104 |
| HRESETn | input |  | Sensor Hub I/O List | 104 |
| DEBUG_MODE_i | input |  | Sensor Hub I/O List | 104 |
| scl_master_in | input |  | Sensor Hub I/O List | 104 |
| sda_master_in | input |  | Sensor Hub I/O List | 104 |
| scl_master_out | output |  | Sensor Hub I/O List | 104 |
| sda_master_out | output |  | Sensor Hub I/O List | 104 |
| dvalid_masterblaze_ext | input |  | Sensor Hub I/O List | 104 |
| dvalid_masterblaze_ext_enable | input |  | Sensor Hub I/O List | 104 |
| sleep_master | output |  | Sensor Hub I/O List | 104 |
| master_end_op_pulse | output |  | Sensor Hub I/O List | 104 |
| SH_SMARTFIFO_OCCUPATION | input |  | Sensor Hub I/O List | 104 |
| fifo_mode | input |  | Sensor Hub I/O List | 104 |
| fifo_offset | input | [11:0] | Sensor Hub I/O List | 104 |
| HSELS | input |  | Sensor Hub I/O List | 104 |
| HADDRS | input | [ADDRWIDTH- 1:0] 104 | Sensor Hub I/O List | 104 |
| HTRANSS | input | [1:0] | Sensor Hub I/O List | 105 |
| HSIZES | input | [2:0] | Sensor Hub I/O List | 105 |
| HWRITES | input |  | Sensor Hub I/O List | 105 |
| HREADYS | input |  | Sensor Hub I/O List | 105 |
| HWDATAS | input | [31:0] | Sensor Hub I/O List | 105 |
| HREADYOUTS | output |  | Sensor Hub I/O List | 105 |
| HRESPS | output |  | Sensor Hub I/O List | 105 |
| HRDATAS | output | [31:0] | Sensor Hub I/O List | 105 |
| ahb_master_on | output |  | Sensor Hub I/O List | 105 |
| HRDATAM | input | [31:0] | Sensor Hub I/O List | 105 |
| HREADYM | input |  | Sensor Hub I/O List | 105 |
| HRESPM | input | [1:0] | Sensor Hub I/O List | 105 |
| HGRANTM | input |  | Sensor Hub I/O List | 105 |
| HSELM | output |  | Sensor Hub I/O List | 105 |
| HADDRM | output | [31:0] | Sensor Hub I/O List | 105 |
| HTRANSM | output | [1:0] | Sensor Hub I/O List | 105 |
| HWRITEM | output |  | Sensor Hub I/O List | 105 |
| HSIZEM | output | [2:0] | Sensor Hub I/O List | 105 |
| HBURSTM | output | [2:0] | Sensor Hub I/O List | 105 |
| HPROTM | output | [3:0] | Sensor Hub I/O List | 105 |
| HWDATAM | output | [31:0] | Sensor Hub I/O List | 105 |
| HBUSREQM | output |  | Sensor Hub I/O List | 105 |
| HLOCKM | output |  | Sensor Hub I/O List | 105 |

## 3. Block requirements

#### IPOS-SENSOR-HUB-001

The output port sleep_master shall be set to 1 when an I2C operation is not on-going. [Vpriority High] [TO: IPOS_Senshub_MB_i2c]

Covers: DDS_STBIO1_9017

[End]

#### IPOS-SENSOR-HUB-002

[DDS_STBIO1_9018] Requirement The trigger of sensor hub shall be possible in both ways, HW via dvalid_masterblaze_ext signal or SW using DVALID_REGISTER.

Covers: DDS_STBIO1_9018

[End]

#### IPOS-SENSOR-HUB-003

[DDS_STBIO1_9024] Requirement Only I2C target addressed through SLAVE0_DEV_ADD register shall be the destination of an I2C Write Operation.

Covers: DDS_STBIO1_9024

[End]

#### IPOS-SENSOR-HUB-004

[DDS_STBIO1_9027] Requirement The bit 0 of SLAVE1_DEV_ADD, SLAVE2_DEV_ADD, SLAVE3_DEV_ADD registers shall be ignored: ONLY read operations towards I2C target 1, 2 and 3 shall be performed.

Covers: DDS_STBIO1_9027

[End]

#### IPOS-SENSOR-HUB-005

AUX_SENS_ON possible values shall be 0,1,2,3. All the values shall be tested since MasterBlaze supports up-to 4 I2C targets connected to I2C network. [TO: IPOS_Senshub_MB_i2c]

Covers: DDS_STBIO1_9029

[End]

#### IPOS-SENSOR-HUB-006

[DDS_STBIO1_9030] Requirement SLAVEx_NUMOP bit-fields of SLAVEx_CONFIG registers shall define how many bytes per slave are read as per effect of an I2C operation (x from 1 to 12).

Covers: DDS_STBIO1_9030

[End]

#### IPOS-SENSOR-HUB-007

All the values from 1 to 12 shall be valid for SLAVEx_NUMOP bit-fields of SLAVEx_CONFIG registers. [TO: IPOS_Senshub_MB_i2c]

Covers: DDS_STBIO1_9031

[End]

#### IPOS-SENSOR-HUB-008

[DDS_STBIO1_9032] Requirement SLAVE0_NUMOP bitfield shall be ignored if an I2C Write operation is issued to the I2C Slave 0.

Covers: DDS_STBIO1_9032

[End]

#### IPOS-SENSOR-HUB-009

All the SENSOR_HUB_x registers shall be reset every time a new I2C operation starts. [TO: IPOS_Senshub_MB_i2c]

Covers: DDS_STBIO1_9033

[End]

#### IPOS-SENSOR-HUB-010

The NACK_REGISTER register is reset every time a new I2C operation starts. [TO: IPOS_Senshub_MB_i2c]

Covers: DDS_STBIO1_9034

[End]

#### IPOS-SENSOR-HUB-011

When Quokka MasterBlaze µP runs (sleep_master signal = 0), any dvalid triggered (HW or SW) shall be ignored. [TO: IPOS_Senshub_MB_i2c]

Covers: DDS_STBIO1_9035

[End]

#### IPOS-SENSOR-HUB-012

Each data stored in FIFO shall be composed by 4 words (16bytes). [TO: IPOS_Senshub_MB_i2c]

Covers: DDS_STBIO1_9260

[End]

#### IPOS-SENSOR-HUB-013

3 words (12bytes) of the data stored in FIFO shall contain the data from each I2C slave. [TO: IPOS_Senshub_MB_i2c]

Covers: DDS_STBIO1_9261

[End]

#### IPOS-SENSOR-HUB-014

24-bits of the data stored in FIFO shall be reserved at ‘0’. [TO: IPOS_Senshub_MB_i2c]

Covers: DDS_STBIO1_9262

[End]

#### IPOS-SENSOR-HUB-015

The last 8-bits of the data stored in FIFO shall contain the TAG field. [TO: IPOS_Senshub_MB_i2c]

Covers: DDS_STBIO1_9263

[End]

#### IPOS-SENSOR-HUB-016

[DDS_STBIO1_9300] Requirement The AHB Write to registers has no effect if the IP masterblaze processor is busy (I2C operation on-going); the registers value shall not be updated.

Covers: DDS_STBIO1_9300

[End]

#### IPOS-SENSOR-HUB-017

If the sensor hub IP is not enabled, the SCL and SDA pad shall not toggle. [Vpriority High] [TO: IPOS_Senshub_MB_i2c]

Covers: DDS_STBIO1_9317

[End]

#### IPOS-SENSOR-HUB-018

[DDS_STBIO1_9500] Requirement In case also other sources that writes into FIFO are active, then sensor hub shall work always in multi-fifo mode fifo_mode input of sensor hub set to ‘1’ .

Covers: DDS_STBIO1_9500

[End]