# Digital Requirements Specification

## STBIO

Author: DRS Gen Spec Agent

Date: 2026-09-01

Source specification: C:/Users/iannuzza/LOCAL_PROJ/Github_copilot_proj/STBIO_AI/specs/DDS_STBIO1.pdf

## 0. Document Navigation

### 0.1 Table of contents

- STBIO

- 0. Document Navigation

- 1. Introduction

- 1.1 Purpose

- 1.2 Scope

- 1.3 Intended audience

- 1.4 References

- 2. Definitions and terminology

- 3. System context for digital behavior

- 4. Digital requirements

- 4.1 Digital block list

- 4.2 Control and state requirements

- 4.3 Register and configuration requirements

- 4.4 Interface and communication requirements

- 4.5 Data-path and buffering requirements

- 4.6 Digital performance requirements

- 4.7 Clock and reset synchronization

- 5. Validation and qualification requirements

- 6. Requirement identification and traceability

- 6.1 Numbering convention

- 6.2 Traceability contract

- 6.3 Requirement catalog

- 6.3.1 Digital DFT

- 6.3.2 Measurement Cycle Examples

- 6.4 Source Function Context

- 6.4.1 BIST MODE

- 6.4.2 BOOT Phase

- 6.4.3 Configuration Phase

- 6.4.4 Debug mode

- 6.4.5 ECG Only

- 6.4.6 ECG and BIA

- 6.4.7 Other source function context

- 6.4.8 PAD Mux

- 6.4.9 PPG Only

- 6.4.10 STBIO1 Register Map Peculiar Requirements

- 6.4.11 Scan mode

- 7. Interfaces and mixed-signal interactions

- 8. Assumptions and TBD

- 9. Project-specific digital block sections

- 9.1 Sensor-Hub

- 9.2 ADSP

- 9.3 Main Controller

- 9.4 Smart FIFO

- 9.5 IRQ logic

- 9.6 PMU

- 9.7 I2C_SPI_AHB

- 9.8 ISPU

- 10. Missing Inputs

### 0.2 Internal index for paragraphs and pages

#### Table 3. Section navigation index

| Section | Paragraph anchor | Internal link | Page (rendered PDF) |
| --- | --- | --- | --- |
| STBIO | 0 | Jump | Auto |
| 0. Document Navigation | 0 | Jump | Auto |
| 1. Introduction | 1 | Jump | Auto |
| 1.1 Purpose | 1.1 | Jump | Auto |
| 1.2 Scope | 1.2 | Jump | Auto |
| 1.3 Intended audience | 1.3 | Jump | Auto |
| 1.4 References | 1.4 | Jump | Auto |
| 2. Definitions and terminology | 2 | Jump | Auto |
| 3. System context for digital behavior | 3 | Jump | Auto |
| 4. Digital requirements | 4 | Jump | Auto |
| 4.1 Digital block list | 4.1 | Jump | Auto |
| 4.2 Control and state requirements | 4.2 | Jump | Auto |
| 4.3 Register and configuration requirements | 4.3 | Jump | Auto |
| 4.4 Interface and communication requirements | 4.4 | Jump | Auto |
| 4.5 Data-path and buffering requirements | 4.5 | Jump | Auto |
| 4.6 Digital performance requirements | 4.6 | Jump | Auto |
| 4.7 Clock and reset synchronization | 4.7 | Jump | Auto |
| 5. Validation and qualification requirements | 5 | Jump | Auto |
| 6. Requirement identification and traceability | 6 | Jump | Auto |
| 6.1 Numbering convention | 6.1 | Jump | Auto |
| 6.2 Traceability contract | 6.2 | Jump | Auto |
| 6.3 Requirement catalog | 6.3 | Jump | Auto |
| 6.3.1 Digital DFT | 6.3.1 | Jump | Auto |
| 6.3.2 Measurement Cycle Examples | 6.3.2 | Jump | Auto |
| 6.4 Source Function Context | 6.4 | Jump | Auto |
| 6.4.1 BIST MODE | 6.4.1 | Jump | Auto |
| 6.4.2 BOOT Phase | 6.4.2 | Jump | Auto |
| 6.4.3 Configuration Phase | 6.4.3 | Jump | Auto |
| 6.4.4 Debug mode | 6.4.4 | Jump | Auto |
| 6.4.5 ECG Only | 6.4.5 | Jump | Auto |
| 6.4.6 ECG and BIA | 6.4.6 | Jump | Auto |
| 6.4.7 Other source function context | 6.4.7 | Jump | Auto |
| 6.4.8 PAD Mux | 6.4.8 | Jump | Auto |
| 6.4.9 PPG Only | 6.4.9 | Jump | Auto |
| 6.4.10 STBIO1 Register Map Peculiar Requirements | 6.4.10 | Jump | Auto |
| 6.4.11 Scan mode | 6.4.11 | Jump | Auto |
| 7. Interfaces and mixed-signal interactions | 7 | Jump | Auto |
| 8. Assumptions and TBD | 8 | Jump | Auto |
| 9. Project-specific digital block sections | 9 | Jump | Auto |
| 9.1 Sensor-Hub | 9.1 | Jump | Auto |
| 9.2 ADSP | 9.2 | Jump | Auto |
| 9.3 Main Controller | 9.3 | Jump | Auto |
| 9.4 Smart FIFO | 9.4 | Jump | Auto |
| 9.5 IRQ logic | 9.5 | Jump | Auto |
| 9.6 PMU | 9.6 | Jump | Auto |
| 9.7 I2C_SPI_AHB | 9.7 | Jump | Auto |
| 9.8 ISPU | 9.8 | Jump | Auto |
| 10. Missing Inputs | 10 | Jump | Auto |

### 0.3 Document control

#### Table 1. Version history

| Version | Date | Description | Author |
| --- | --- | --- | --- |
| 1.0 | 2026-09-01 | Initial DRS baseline generated from Stage 1 and Stage 2 artifacts | DRS Gen Spec Agent |

#### Table 2. Reference documents

| Doc name | Version | Author |
| --- | --- | --- |
| artifacts/stage4_ars/analog_requirements_specification.md | Current workspace revision | ARS Gen Spec Agent |
| artifacts/stage1_requirements/requirements_summary.csv | Current workspace revision | Requirements Extraction Agent |
| artifacts/stage2_mirco_arc/micro_architecture_report.md | Current workspace revision | Micro-Architectural Analysis Agent |
| artifacts/stage2_mirco_arc/requirement_to_block_traceability.csv | Current workspace revision | Micro-Architectural Analysis Agent |
| templates/DRS_gen_AI_template_prompt.md | Current workspace revision | Project template maintainers |

#### Table 6. Category convention

| Category | Naming rule / prefix | Scope | Notes / example |
| --- | --- | --- | --- |
| Digital authored requirement | DRS-REQ-xxx | Project-specific DRS atomic entries | Covers one upstream requirement per row |
| Source digital/system/cross-domain requirement | source_req_id | Stage 1 source requirement catalog | Linked to authored SRS upstream ID |
| DRS upstream reference in Covers | SRS-REQ-xxx | Authored SRS requirement catalog | Required for every DRS authored requirement |

### 0.4 Table of tables

| Table | Title | Link |
| --- | --- | --- |
| Table 1 | Version history | Go to Table 1 |
| Table 2 | Reference documents | Go to Table 2 |
| Table 3 | Section navigation index | Go to Table 3 |
| Table 6 | Category convention | Go to Table 6 |

## 1. Introduction

### 1.1 Purpose

This document defines verifiable digital requirements derived from Stage 1 and micro-architecture artifacts.

### 1.2 Scope

This DRS covers digital subsystem behavior, control logic, interfaces, modes, and verification hooks.

### 1.3 Intended audience

System architects, digital design, firmware, validation, and verification stakeholders.

### 1.4 References

- artifacts/stage4_ars/analog_requirements_specification.md

- artifacts/stage1_requirements/requirements_summary.csv

- artifacts/stage2_mirco_arc/micro_architecture_report.md

- artifacts/stage2_mirco_arc/requirement_to_block_traceability.csv

- templates/DRS_gen_AI_template_prompt.md

## 2. Definitions and terminology

- Digital terminology follows source requirements and micro-architecture artifacts.

## 3. System context for digital behavior

- System context is summarized to scope digital requirements and mixed-signal interfaces.

## 4. Digital requirements

General digital requirements that are not mapped to any included digital block are listed below.

Residual digital-relevant requirements are captured in the requirement catalog with SRS upstream Covers links.

### 4.1 Digital block list

Digital implementation shall include blocks: Sensor-Hub, ADSP, Main Controller, Smart FIFO, Regmap, SPI interface, I2C interface, IRQ logic, PMU, BIST Controller, I2C_SPI_AHB, Unassigned, ISPU, ISPU debug.

### 4.2 Control and state requirements

Digital control behavior shall define enable, reset, mode transitions, and deterministic state handling.

### 4.3 Register and configuration requirements

Register map behavior shall define access policy, reset values, and safe update ordering.

### 4.4 Interface and communication requirements

Digital interfaces shall define protocol behavior, timing constraints, and error handling responses.

### 4.5 Data-path and buffering requirements

Data-path behavior shall define format, throughput, buffering, and overflow or underflow handling.

### 4.6 Digital performance requirements

Performance requirements shall cover latency, interrupt response, startup, and update timing constraints.

### 4.7 Clock and reset synchronization

The PMU is the authoritative clock and reset controller for the digital wrapper. This summary is derived from the PMU Clocks and Reset connection tables and PMU startup requirements.

#### Clock distribution summary

- The top-level 16 MHz and 64 kHz clocks feed u_pmu.clk_16m and u_pmu.clk_64k without clock gating (DDS_STBIO1_1020 and DDS_STBIO1_1021).

- The PMU supplies an ungated 32 kHz test clock to the Main Controller i_clk_32kHz and exports the 32 kHz test clock at the digital-wrapper boundary (DDS_STBIO1_1027 and DDS_STBIO1_1025).

- PMU 16 MHz clock paths serve ADSP stmc_clk, Main Controller sys_clk and i_clk_16MHz, AFE Xbar hclk, FIFO HCLK, ISPU clk, and Regmap/OTP hclk (DDS_STBIO1_1022 through DDS_STBIO1_1026, DDS_STBIO1_2024 through DDS_STBIO1_2026).

- The ADSP, Main Controller, AFE Xbar, FIFO, Regmap/OTP, and 16 MHz test-clock paths use the specified PMU enable/test/scan controls; the ISPU clock and the primary PMU input clocks are power-down controlled or ungated as specified in the clock recap table.

#### Reset synchronization summary

- PMU reset outputs synchronize reset release to the receiving clock domains: resetn_stmc to ADSP HRESETn, resetn_masterblaze to Main Controller HRESETn, resetn_regmap to Regmap/OTP hresetn and ISPU main_rst_n, resetn_fifo to Multi-Channel FIFO HRESETn, and the device-startup POR-main-reset path to AFE Xbar hresetn (DDS_STBIO1_1050 through DDS_STBIO1_1053, DDS_STBIO1_2052, and DDS_STBIO1_2053).

- When i_POR1V2_1V2 is low, the PMU keeps LDO1V8 disabled, powers down the clock function, deasserts 16 MHz ready, and disables the RC voltage reference. After POR rises, LDO1V8 and the divided 32 kHz clock become stable after at least 10 cycles of the 64 kHz input; the RC reference is enabled after at least 352 divided-clock cycles (DDS_STBIO1_0104 through DDS_STBIO1_0110).

- After i_START_CLK_16M asserts, o_CLK_16M_READY asserts after at least 20 cycles of the 16 MHz clock. On its first assertion, o_main_resetn is released after the same minimum 20-cycle delay, preventing downstream logic from leaving reset before the 16 MHz clock is ready (DDS_STBIO1_0112 and DDS_STBIO1_0113).

## 5. Validation and qualification requirements

- Verification shall combine inspection, analysis, and test with measurable acceptance criteria.

## 6. Requirement identification and traceability

### 6.1 Numbering convention

DRS authored requirements use DRS-REQ-### numbering. Source requirements retain their original IDs and are referenced through Covers linkage.

### 6.2 Traceability contract

DRS requirement catalog entries include statement only; full traceability details are maintained in artifacts/stage5_drs/drs_traceability_matrix.csv.

### 6.3 Requirement catalog

#### Dedicated source sections

##### 6.3.1 Digital DFT

[DRS-REQ-150] Requirement:

To turn on the ADC TEST LOW NOISE mode shall be:

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0xF, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable ADC in debug mode (ADDRESS=0x09, DATA=0x06)

- Select page 2 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable ADC TEST LOW NOISE and input of ADC from GPIO (ADDRESS=0x0A, DATA=0x14)

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable debug mode by writing to regmap (ADDRESS=0x3E, DATA=0x08)

Covers: SRS-REQ-150

[End]

[DRS-REQ-151] Requirement:

To turn on the ADC TEST LOW NOISE mode shall be:

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0xF, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable ADC in debug mode (ADDRESS=0x09, DATA=0x06)

- Select page 2 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable ADC TEST LOW NOISE and input of ADC from GPIO (ADDRESS=0x0A, DATA=0x14)

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable debug mode by writing to regmap (ADDRESS=0x3E, DATA=0x08)

Covers: SRS-REQ-151

[End]

[DRS-REQ-152] Requirement:

To turn on the ADC TEST LOW NOISE mode shall be:

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0xF, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable ADC in debug mode (ADDRESS=0x09, DATA=0x06)

- Select page 2 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable ADC TEST LOW NOISE and input of ADC from GPIO (ADDRESS=0x0A, DATA=0x0C)

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable debug mode by writing to regmap (ADDRESS=0x3E, DATA=0x08)

Covers: SRS-REQ-152

[End]

##### 6.3.2 Measurement Cycle Examples

[DRS-REQ-121] Requirement:

At the release of the POR signal from the analog domain, the PMU shall manage the turn-on of the LDO1V8 and starts clocks Start-Up Sequence (reported in PMU IPOS) [TO: IPOS_PMU]

Covers: SRS-REQ-121

[End]

[DRS-REQ-122] Requirement:

After At least 2 ms from the Release of the POR, the reset of the digital domain shall be de- asserted (an overlook on this part is described in the PMU IPOS) and the device shall start the Boot Phase [TO: IPOS_PMU]

Covers: SRS-REQ-122

[End]

#### Digital requirements

[DRS-REQ-002] Requirement:

When the device is turned on, i_POR1V2_1V2 shall be high. [TO: IPOS_PMU]

Covers: SRS-REQ-002

[End]

[DRS-REQ-004] Requirement:

When i_POR1V2_1V2 is low, o_PD_CLKF shall be high. [TO: IPOS_PMU]

Covers: SRS-REQ-004

[End]

[DRS-REQ-006] Requirement:

When i_POR1V2_1V2 is low, o_DIS_RC_VIREF shall be high. [TO: IPOS_PMU]

Covers: SRS-REQ-006

[End]

[DRS-REQ-060] Requirement:

After startup phase, once started with an operation, the IP shall ignore any further start incoming transactions. [Vpriority High]

Covers: SRS-REQ-060

[End]

#### Cross-domain requirements

- No residual unmapped requirements for this domain (mapped items are captured in project-specific sub-block requirement paragraphs).

#### Use case, user specific

##### FIFO_CTRL Peculiar Requirements (under Section 13.2.4 FIFO_CTRL integration)

[DRS-REQ-105] Requirement:

The user shall select the Multiple FIFO mode setting to 1 the first bit into the FIFO_CFG_MODE and 0 for the Unique FIFO mode. [TO: IPOS_FIFO_CTRL_89456975]

Covers: SRS-REQ-105

[End]

##### Main Controller Soft Reset (under Section 14 Soft Reset Procedures)

[DRS-REQ-120] Requirement:

To perform a SOFT Reset procedure, the user shall perform the following steps: 1. If Mode Operation has been set to 1, shall be reset to 0 2. Set the Soft Reset Main Controller Bit (SW_RESETN_REG[0]) to 0 for at least 1 ms 3. Set the Soft Reset Main Controller Bit (SW_RESETN_REG[0]) to 1 4. Wait for the DEVICE_STATUS_REGISTER to reach the IDLE Value

Covers: SRS-REQ-120

[End]

##### Requirements (under Section 13.2.1.1 ADSP Peculiar Requirements)

[DRS-REQ-058] Requirement:

OTP_IREF_WAKEUP shall be set to one by an external I2C/SPI to AHB write before initiating a new OTP_TEST or OTP_WRITE operation [Vpriority High]

Covers: SRS-REQ-058

[End]

[DRS-REQ-059] Requirement:

OTP_IREF_WAKEUP shall be setted to zero by an external I2C/SPI to AHB write at the end of OTP_TEST or OTP_WRITE operation [Vpriority High]

Covers: SRS-REQ-059

[End]

## 7. Interfaces and mixed-signal interactions

- Digital-to-analog interactions shall include configuration transfer, measurement readout, status monitoring, and calibration control handshakes.

## 8. Assumptions and TBD

- ASSUME-001: Firmware and verification environments provide deterministic digital stimulus and observability.

- TBD-001: Unassigned requirements in DRS traceability matrix require architectural owner review.

## 9. Project-specific digital block sections

### 9.1 Sensor-Hub

General functional description: Operate the Sensor Hub I2C master for external targets, including trigger/control, pad enable, data collection, and FIFO multi-mode coordination.

Inputs: Primary sensing input Outputs: Analog input signal

Source interface table 1: Table 26: Sensor Hub I/O List (source table from OCR extract page 105)

| Port name | Direction | Type / Function | TOP CONNECTION / Details |
| --- | --- | --- | --- |
| scan_mode | input |  |  |
| scan_enable | input |  |  |
| scan_rstn | input |  |  |
| HCLKM | input |  |  |
| HCLKS | input |  |  |
| sys_clk | input |  |  |
| HRESETn | input |  |  |
| DEBUG_MODE_i | input |  |  |
| scl_master_in | input |  |  |
| sda_master_in | input |  |  |
| scl_master_out | output |  |  |
| sda_master_out | output |  |  |
| dvalid_masterblaze_ext | input |  |  |
| dvalid_masterblaze_ext_enable | input |  |  |
| sleep_master | output |  |  |
| master_end_op_pulse | output |  |  |
| SH_SMARTFIFO_OCCUPATION | Input |  |  |
| fifo_mode | input |  |  |
| fifo_offset | input | [11:0] |  |
| HSELS | input |  |  |
| HADDRS | input | [ADDRWIDTH- |  |

| Port name | Direction | Type / Function | TOP CONNECTION / Details |
| --- | --- | --- | --- |
| HTRANSS | input | [1:0] |  |
| HSIZES | input | [2:0] |  |
| HWRITES | input |  |  |
| HREADYS | input |  |  |
| HWDATAS | input | [31:0] |  |
| HREADYOUTS | output |  |  |
| HRESPS | output |  |  |
| HRDATAS | output | [31:0] |  |
| ahb_master_on | output |  |  |
| HRDATAM | input | [31:0] |  |
| HREADYM | input |  |  |
| HRESPM | input | [1:0] |  |
| HGRANTM | input |  |  |
| HSELM | output |  |  |
| HADDRM | output | [31:0] |  |
| HTRANSM | output | [1:0] |  |
| HWRITEM | output |  |  |
| HSIZEM | output | [2:0] |  |
| HBURSTM | output | [2:0] |  |
| HPROTM | output | [3:0] |  |
| HWDATAM | output | [31:0] |  |
| HBUSREQM | output |  |  |
| HLOCKM | output |  |  |

[DRS-REQ-075] Requirement:

The Sensor-Hub block shall implement: If the sensor hub IP is not enabled, the SCL and SDA pad shall not toggle. [Vpriority High] [TO: IPOS_Senshub_MB_i2c].

Covers: SRS-REQ-075

[End]

[DRS-REQ-076] Requirement:

The Sensor-Hub block shall implement: Output scl_master_out of sensor hub shall be connected to enable of output buffer of sensor hub’s SCL PAD in case it is a bi-directional PAD). [Vpriority High] [TO: IPOS_Senshub_MB_i2c].

Covers: SRS-REQ-076

[End]

[DRS-REQ-077] Requirement:

The Sensor-Hub block shall implement: Output sda_master_out of sensor hub shall be connected to enable of output buffer of sensor hub’s SDA PAD. [Vpriority High] [TO: IPOS_Senshub_MB_i2c].

Covers: SRS-REQ-077

[End]

[DRS-REQ-078] Requirement:

The Sensor-Hub block shall implement: Input scl_master_in of sensor hub shall be connected to input buffer from sensor hub’s SCL PAD. [Vpriority High] [TO: IPOS_Senshub_MB_i2c].

Covers: SRS-REQ-078

[End]

[DRS-REQ-079] Requirement:

The Sensor-Hub block shall implement: Input sda_master_in of sensor hub shall be connected to input buffer from sensor hub’s SDA PAD. [Vpriority High] [TO: IPOS_Senshub_MB_i2c].

Covers: SRS-REQ-079

[End]

[DRS-REQ-080] Requirement:

The Sensor-Hub block shall implement: The output port sleep_master shall be set to 1 when an I2C operation is not on-going. [Vpriority High] [TO: IPOS_Senshub_MB_i2c].

Covers: SRS-REQ-080

[End]

[DRS-REQ-081] Requirement:

The Sensor-Hub block shall implement: Input sys_clk of sensor hub shall be connected to 8MHz clock to generate I2C communications compliant with FM+ I2C standard. [TO: IPOS_Senshub_MB_i2c].

Covers: SRS-REQ-081

[End]

[DRS-REQ-082] Requirement:

The Sensor-Hub block shall implement: The trigger of sensor hub shall be possible in both ways, HW via dvalid_masterblaze_ext signal or SW using DVALID_REGISTER.

Covers: SRS-REQ-082

[End]

[DRS-REQ-083] Requirement:

The Sensor-Hub block shall implement: Only I2C target addressed through SLAVE0_DEV_ADD register shall be the destination of an I2C Write Operation.

Covers: SRS-REQ-083

[End]

[DRS-REQ-084] Requirement:

The Sensor-Hub block shall implement: The bit 0 of SLAVE1_DEV_ADD, SLAVE2_DEV_ADD, SLAVE3_DEV_ADD registers shall be ignored: ONLY read operations towards I2C target 1, 2 and 3 shall be performed.

Covers: SRS-REQ-084

[End]

[DRS-REQ-085] Requirement:

The Sensor-Hub block shall implement: SLAVEx_NUMOP bit-fields of SLAVEx_CONFIG registers shall define how many bytes per slave are read as per effect of an I2C operation (x from 1 to 12).

Covers: SRS-REQ-085

[End]

[DRS-REQ-086] Requirement:

The Sensor-Hub block shall implement: SLAVE0_NUMOP bitfield shall be ignored if an I2C Write operation is issued to the I2C Slave 0.

Covers: SRS-REQ-086

[End]

[DRS-REQ-087] Requirement:

The Sensor-Hub block shall implement: AUX_SENS_ON possible values shall be 0,1,2,3. All the values shall be tested since MasterBlaze supports up-to 4 I2C targets connected to I2C network. [TO: IPOS_Senshub_MB_i2c].

Covers: SRS-REQ-087

[End]

[DRS-REQ-088] Requirement:

The Sensor-Hub block shall implement: All the values from 1 to 12 shall be valid for SLAVEx_NUMOP bit-fields of SLAVEx_CONFIG registers. [TO: IPOS_Senshub_MB_i2c].

Covers: SRS-REQ-088

[End]

[DRS-REQ-089] Requirement:

The Sensor-Hub block shall implement: All the SENSOR_HUB_x registers shall be reset every time a new I2C operation starts. [TO: IPOS_Senshub_MB_i2c].

Covers: SRS-REQ-089

[End]

[DRS-REQ-090] Requirement:

The Sensor-Hub block shall implement: When Quokka MasterBlaze µP runs (sleep_master signal = 0), any dvalid triggered (HW or SW) shall be ignored. [TO: IPOS_Senshub_MB_i2c].

Covers: SRS-REQ-090

[End]

[DRS-REQ-091] Requirement:

The Sensor-Hub block shall implement: The AHB Write to registers has no effect if the IP masterblaze processor is busy (I2C operation on-going); the registers value shall not be updated.

Covers: SRS-REQ-091

[End]

[DRS-REQ-092] Requirement:

The Sensor-Hub block shall implement: In case also other sources that writes into FIFO are active, then sensor hub shall work always in multi-fifo mode fifo_mode input of sensor hub set to ‘1’.

Covers: SRS-REQ-092

[End]

[DRS-REQ-093] Requirement:

The Sensor-Hub block shall implement: The system shall satisfy the following behavior: The NACK_REGISTER register is reset every time a new I2C operation starts. [TO: IPOS_Senshub_MB_i2c].

Covers: SRS-REQ-093

[End]

[DRS-REQ-094] Requirement:

The Sensor-Hub block shall implement: Each data stored in FIFO shall be composed by 4 words (16bytes). [TO: IPOS_Senshub_MB_i2c].

Covers: SRS-REQ-094

[End]

[DRS-REQ-095] Requirement:

The Sensor-Hub block shall implement: 3 words (12bytes) of the data stored in FIFO shall contain the data from each I2C slave. [TO: IPOS_Senshub_MB_i2c].

Covers: SRS-REQ-095

[End]

[DRS-REQ-096] Requirement:

The Sensor-Hub block shall implement: 24-bits of the data stored in FIFO shall be reserved at ‘0’. [TO: IPOS_Senshub_MB_i2c].

Covers: SRS-REQ-096

[End]

[DRS-REQ-097] Requirement:

The Sensor-Hub block shall implement: The last 8-bits of the data stored in FIFO shall contain the TAG field. [TO: IPOS_Senshub_MB_i2c].

Covers: SRS-REQ-097

[End]

### 9.2 ADSP

General functional description: Execute ADSP firmware for OTP boot/write/test and signal elaboration, using mapped program/data/register memory and soft-reset handling.

Inputs: Acquired samples and configuration data Outputs: Elaborated signal data and metadata

Source interface table 1: Table 19: ADSP I/O List (source table from OCR extract page 87)

| Parameter | Value / Type | Details |
| --- | --- | --- |
| ADDRWIDTH | 12 |  |

| Port name | Direction | Type / Function | TOP CONNECTION / Details |
| --- | --- | --- | --- |
| stmc_clk | input | wire u_pmu.tst_clk_16m |  |
| HCLK | input | wire u_pmu.clk_stmc |  |
| HRESETn | input | wire u_pmu.resetn_stmc |  |
| HSELS | input | wire u_xbar_afe. |  |
| HADDRS | input | wire [ADDRWIDTH-1:0] u_xbar_afe.haddr_M1 |  |
| HTRANSS | input | wire [ 1:0] u_xbar_afe.htrans_M1 |  |
| HSIZES | input | wire [ 2:0] u_xbar_afe. |  |
| HWRITES | input | wire u_xbar_afe. |  |
| HREADYS | input | wire u_xbar_afe. |  |
| HWDATAS | input | wire [ 31:0] u_xbar_afe. |  |
| HREADYOUTS | output | wire u_xbar_afe. |  |
| HRESPS | output | wire u_xbar_afe. |  |
| HRDATAS | output | wire [ 31:0] u_xbar_afe. |  |
| HRDATAM | input | wire [ 31:0] |  |
| HREADYM | input | wire |  |
| HRESPM | input | wire [ 1:0] |  |
| HGRANTM | input | wire |  |
| HSELM | output | wire |  |
| HADDRM | output | wire [ 31:0] |  |
| HTRANSM | output | wire [ 1:0] |  |
| HWRITEM | output | wire |  |
| HSIZEM | output | wire [ 2:0] |  |
| HBURSTM | output | wire [ 2:0] |  |
| HPROTM | output | wire [ 3:0] |  |
| HWDATAM | output | wire [ 31:0] |  |
| HBUSREQM | output | wire |  |
| HLOCKM | output | wire |  |
| stmc_data_CSN | output | wire |  |
| stmc_data_WEN | output | wire |  |
| stmc_data_A | output | wire [ 8:0] |  |
| stmc_data_D | output | wire [ 63:0] |  |
| stmc_data_M | output | wire [ 63:0] |  |
| stmc_data_Q | input | wire [ 63:0] |  |
| stmc_rom_CLK | output | wire |  |
| stmc_rom_CSN | output | wire |  |
| stmc_rom_A | output | wire [ 10:0] |  |
| stmc_rom_Q | input | wire [ 22:0] |  |
| quokka_run | output | wire |  |
| quokka_boot_end | output | wire main_controller_top.i_quokka_b |  |
| ahb_master_clk_en | output | wire U_pmu.clk_en_s1 |  |
| scan_mode | input | wire |  |
| scan_en | input | wire |  |
| scan_clk | input | wire |  |
| scan_rst_n | input | wire |  |

[DRS-REQ-047] Requirement:

The ADSP block shall implement: The BOOT routine shall starts when the OTP_BOOT bit rises and shall completes when quokka_boot_end rises. [Vpriority High].

Covers: SRS-REQ-047

[End]

[DRS-REQ-048] Requirement:

The ADSP block shall implement: During OTP boot operation, the ADSP shall copy the content of the OTP memory into the OTP registers, following the procedure to access OTP memory [Vpriority High].

Covers: SRS-REQ-048

[End]

[DRS-REQ-049] Requirement:

The ADSP block shall implement: During BOOT routine, ADSP shall first read the last location of the OTP memory, at address 0x7F. If the content is 0xFF, called “lifecycle” byte, then the READ operation proceeds from address 0 to address 0x2F; otherwise, ADSP shall not perform any further reads.

Covers: SRS-REQ-049

[End]

[DRS-REQ-050] Requirement:

The ADSP block shall implement: To perform a BOOT routine, OTP_BOOT bit shall be written when quokka_run is low. [Vpriority High].

Covers: SRS-REQ-050

[End]

[DRS-REQ-051] Requirement:

The ADSP block shall implement: The WRITE routine shall start when OTP Write Bit rises and completes when quokka_run falls. [Vpriority High].

Covers: SRS-REQ-051

[End]

[DRS-REQ-052] Requirement:

The ADSP block shall implement: During WRITE routine ADSP shall copy the content of the OTP regmap into the OTP memory, following the procedure to burn the fuses as expressed in the OTP datasheet [Vpriority High].

Covers: SRS-REQ-052

[End]

[DRS-REQ-053] Requirement:

The ADSP block shall implement: During WRITE routine, ADSP shall allow copying all OTP Registers into the OTP Memory only if the location 0x24 of OTP Memory (OTP[24] in the following diagram) is not set to 0xFF.

Covers: SRS-REQ-053

[End]

[DRS-REQ-054] Requirement:

The ADSP block shall implement: During the WRITE routine and after copying all OTP Registers into the OTP Memory, ADSP shall compare each value inside the OTP memory with the corresponding value in OTP Registers to check if the WRITE routine has been succesfull. In that case ADSP shall write 0xFF into the last location of the OTP memory.

Covers: SRS-REQ-054

[End]

[DRS-REQ-055] Requirement:

The ADSP block shall implement: OTP_WRITE, OTP_BOOT and OTP_TEST bits shall be cleared by ADSP at the end of their respective operation [Vpriority High].

Covers: SRS-REQ-055

[End]

[DRS-REQ-056] Requirement:

The ADSP block shall implement: A new operation shall start only when quokka_run is low, and one between OTP_TEST bit, OTP_WRITE bit or OTP_BOOT bit is written to 1 [Vpriority High].

Covers: SRS-REQ-056

[End]

[DRS-REQ-057] Requirement:

The ADSP block shall implement: The TEST routines shall allow reading an 8-bit word on OTP of a specific address written in OTP_A register, and write this value in OTP_D_TEST register.

Covers: SRS-REQ-057

[End]

[DRS-REQ-061] Requirement:

The ADSP block shall implement: The TEST routine shall start when OTP Test Bit rises and completes when quokka_run falls. [Vpriority High].

Covers: SRS-REQ-061

[End]

[DRS-REQ-062] Requirement:

The ADSP block shall implement: During the Soft Reset routine, ADSP shall write the ADSP RAM with the value 0. Only the first 3.5Kbyte of ADSP RAM shall be cleared, from address 0x000 to address 0xDFF [Vpriority High].

Covers: SRS-REQ-062

[End]

[DRS-REQ-064] Requirement:

The ADSP block shall implement: After each Boot routine performed by the ADSP, a value of 0x04 shall be written once to address 0x0CA00C to ensure correct operation of the AHB transactions. [Vpriority High].

Covers: SRS-REQ-064

[End]

[DRS-REQ-065] Requirement:

The ADSP block shall implement: The Soft Reset routine shall start when SOFT_RESET Bit (in CONTROL_A register) rises and completes when quokka_run falls. [Vpriority High].

Covers: SRS-REQ-065

[End]

[DRS-REQ-066] Requirement:

The ADSP block shall implement: If during the start of a new operation, more than one bit is written into CONTROL_A reg in the same write operation, ADSP shall execute only the routine at higher priority and discard the others. Priority is listed below, with first routine at higher priority: 1. SOFT RESET routine 2. OTP WRITE routine 3. OTP TEST routine 4. OTP BOOT routine [Vpriority High].

Covers: SRS-REQ-066

[End]

[DRS-REQ-154] Requirement:

The ADSP block shall implement: To move OTP signal, ADSP shall write a new value in a regbank by AHB protocol.

Covers: SRS-REQ-154

[End]

### 9.3 Main Controller

General functional description: Generate and receive Analog Domain control signals, perform DSP elaboration on ADC outputs, and write elaborated results to the defined addresses.

Inputs: Configuration register settings, sampled data Outputs: Control state and data routing

Source interface table 1: 13.1.3.2. Main Controller TOP I/O List (source table from OCR extract page 74)

| Parameter | Value / Type | Details |
| --- | --- | --- |
| ppg_frame_bit_top | 4 |  |
| n_max_frame | 12 |  |
| ppg_rx_channel_bit_top | 3 |  |
| T_tx_led_set_bit_top | 9 |  |
| T_tx_led_off_max_bit_top | 11 |  |
| m_odr_max_bit_top | 12 |  |
| mode_operation_n_bit_top | 2 |  |
| idac_dc_ppg_frame_bit_top | 6 |  |
| time_slot_sel_bit_top | 3 |  |
| T_tx_startup_bit_top | 8 |  |
| T_rx_set_bit_top | 9 |  |
| Nmax_bit_top | 4 |  |
| base_address_bit_top | 12 |  |
| delta_bit_top | 4 |  |
| gain_max_bit_top | 4 |  |
| T1_max_bit_top | 9 |  |
| T2_max_bit_top | 5 |  |
| N_clk_max_bit_top | 6 |  |
| n_average_ppg_max_bit_top | 3 |  |
| rx_ch_gain_max_bit_top | 4 |  |
| BTIA_ALC_register_bit_top | 9 |  |
| n_channel_bit_top | 4 |  |
| n_channel_adc_bit | 4 |  |
| quokka_routine_bit_top | 16 |  |
| vcm_bit_top | 14 |  |
| gnc_cmm_top | 15 |  |
| id_frame_max_bit_top | 4 |  |
| data_in_size_bit_top | 16 |  |
| ppg_start_up_bit_top | 9 |  |
| data_out_size_bit_top | 20 |  |
| n_average_ecg_bia_gsr_max_bit_top | 8 |  |
| rx_ch_frame_n_bit_top | 3 |  |
| curr_sel_bit_top | 8 |  |
| bit_outab_sel_top | 1 |  |
| n_average_ppg_max_value | 128 |  |
| n_average_ecg_bia_gsr_max_value | 16 |  |
| max_frame_repetition_top | 2 |  |
| n_tx_channel | 4 |  |
| ahb_base_address | [31:0] 32’h000C4000 |  |
| time_buffer_const_bit_top | 10 |  |
| T1_max_bit_from_decoder_top | 4 |  |
| T2_max_bit_from_decoder_top | 2 |  |
| ppg_frame_bit_ALC_BTIA_top | 12 |  |
| ecg_channel_bit | 2 |  |

| Port name | Direction | Type / Function | TOP CONNECTION / Details |
| --- | --- | --- | --- |
| i_clk_32kHz | input | clk 32kHz from pmu |  |
| i_clk_16MHz | input | clk 16MHz from pmu |  |
| i_clk_gsr_16M | input | clk for gsr signal |  |
| i_clk_ahb_master | input | clock for ahb master |  |
| i_rstn | input | reset in 16MHz domain |  |
| i_rstn_sync_32 | input | reset in 32kz domain |  |
| i_wave_velocity_mode | input | enable of the wave velocity mode |  |
| i_ispu_end_elab | input |  |  |
| i_n_ppg_frame | input | [ppg_frame_bit_top-1:0] number of frame selected |  |
| i_const_value_ioff | input | [data_in_size_bit_top-1:0] constant value from regmap used for ppg ioff |  |
| i_rx_ch_frame | input | [n_max_frame-1:0] ppg rx channel frame from regamp |  |
| i_T_tx_led_set | input | [n_max_frame-1:0] tx led set time from regmap |  |
| i_T_tx_led_off | input | [n_max_frame-1:0] tx led off time from regmap |  |
| i_curr_sel_tx_frame_x | input | [n_max_frame-1:0] current injection |  |
| i_tx1_outlab_sel_frame_x | input | [n_max_frame-1:0] led output selected from regmap for each frame |  |
| i_tx2_outlab_sel_frame_x | input | [n_max_frame-1:0] led output selected from regmap for each frame |  |
| i_tx3_outlab_sel_frame_x | input | [n_max_frame-1:0] led output selected from regmap for each frame |  |
| i_tx4_outlab_sel_frame_x | input | [n_max_frame-1:0] led output selected from regmap for each frame |  |
| i_idac_dc_ppg_frame | input | [n_max_frame-1:0] idac value for each frame selected from regmap |  |
| i_average_ppg | input | [n_max_frame-1:0] average ppg for each frame selected from |  |
| i_gain_alc_start | input | [gain_max_bit_top-1:0] btia gain for alc from regmap |  |
| i_gain_start | input | [n_max_frame-1:0] btia gain start for compensation from regmap |  |
| i_Tx_idac_Vref_debug | input | [Nmax_bit_top-1:0] Idac value for debug from regmap |  |

| Port name | Direction | Type / Function | TOP CONNECTION / Details |
| --- | --- | --- | --- |
| i_ppg_start_up_time | input | [ppg_start_up_decod_bit_top-1:0] start up time for ppg from regmap |  |
| i_en_txi | input | [n_max_frame-1:0] tx enabling for each frame from regmap |  |
| i_frame_repetition | input | [n_max_frame-1:0] i frame repetition for each frame from regmap |  |
| i_T_preset | input | [1:0] Preset time for digital ramp from regmap |  |
| i_m_ecg | input | [m_odr_max_bit_top-1:0] m parameter for ecg from regmap |  |
| i_m_bia | input | [m_odr_max_bit_top-1:0] m parameter for bia from regmap |  |
| i_m_gsr | input | [m_odr_max_bit_top-1:0] m parameter for gsr from regmap |  |
| i_m_ppg | input | [m_odr_max_bit_top-1:0] m parameter for ppg from regmap |  |
| i_debug_adc_error_cap8_0_bypass | input | error cap bypass for adc from regmap |  |
| i_base_address_ecg0 | input | [base_address_bit_top-1:0] base address for ecg0 in multiple fifo |  |
| i_base_address_bia | input | [base_address_bit_top-1:0] base address for bia/ecg1-2 in multiple fifo |  |
| i_base_address_gsr | input | [base_address_bit_top-1:0] base address for gsr in multiple fifo |  |
| i_base_address_ppg | input | [base_address_bit_top-1:0] base address for ppg in multiple fifo |  |
| i_frame_ppg_debug | input | [ppg_frame_bit_top-1:0] frame selection for ppg data debug |  |
| i_en_buf_gpio1 | input | enable buffer on gpio1 |  |
| i_en_buf_gpio2 | input | enable buffer on gpio2 |  |
| i_en_biobuf | input | enable biobuffer |  |
| i_do_calibration | input | enabling for adc calibration |  |
| i_mode_operation | input | [mode_operation_n_bit_top-1:0] mode operation selection |  |
| i_T_rx_set | input | [T1_max_bit_from_decoder_top- |  |
| i_en_clkf | input | en 16 MHz clock |  |
| i_quokka_boot_end | input | from quokka |  |
| i_quokka_run | input | from quokka |  |

| Port name | Direction | Type / Function | TOP CONNECTION / Details |
| --- | --- | --- | --- |
| i_clk_16MHz_ready | input | (from analog circuit) |  |
| i_imp_curinj_en | input | enabling current injection for bia |  |
| i_en_ldo_1v8 | input | enabling ldo 1v8 |  |
| i_debug_comp_rx_ck | input | enabling debug for comp rx ck |  |
| i_debug_gsr_on | input | gsr on debug mode |  |
| i_debug_gsr_curr_on | input | gsr curr on debug mode |  |
| i_alc_disabled | input | [n_max_frame-1:0] disabling alc bit for each frame |  |
| i_en_cm_buf | input | enabling common buffer |  |
| i_en_rld | input | enabling rld |  |
| i_en_avg1 | input | enabling avg1 |  |
| i_en_avg2 | input | enabling avg1 |  |
| i_hc_en | input | enabling health channel |  |
| i_debug | input | enabling debug mode |  |
| i_imp_en | input | enabling bia channel |  |
| i_debug_en_tia | input | enabling debug tia |  |
| i_debug_en_buffer | input | enabling debug for buffer |  |
| i_debug_en_idac | input | enabling debug for IDAC |  |
| i_en_clk_chop10k | input | enabling chopper clock |  |
| i_debug_tx_preset | input | enabling tx preset |  |
| i_en_ldo_1v8_ext | input | enabling external ldo 1v8 |  |
| i_clk_imp_10k_en | input | enabling clock chopper for bia |  |
| i_b_cap_idac | input | [1:0] enabling cap for idac |  |
| i_adc_mux_bio_channel_debug | input | [2:0] enabling debug adc mux bio channel |  |
| i_adc_mux_sel_debug | input | enabling adc mux |  |

| Port name | Direction | Type / Function | TOP CONNECTION / Details |
| --- | --- | --- | --- |
| i_ispu_core_sw_rstn | input | ispu core enabling |  |
| i_T_TIAON | input | [T1_max_bit_from_decoder_top- |  |
| i_scan_mode | input | scan mode enabling |  |
| i_reset_debug_adc_data | input | reset adc data debug |  |
| i_start_adc_debug | input | pilot adc start om debug |  |
| i_en_ADC_debug | input | enabling adc debug |  |
| i_debug_adc_readout_bypass | input | enabling adc readout bypass |  |
| i_su_hlt_dly | input | enabling start up delay for ecg |  |
| i_multiple_fifo_sel | input | enabling multiple fifo |  |
| i_debug_calibration | input | enabling debug calibraiton |  |
| i_en_gsr_dc_cds | input | [1:0] enabling gsr cds mode |  |
| i_ADC_EOC | input | end of conversion from adc |  |
| i_ecg0_sel | input | select ecg0 channel |  |
| i_bia_sel | input | select bia channel |  |
| i_gsr_sel | input | select gsr channel |  |
| i_ppg_sel | input | select ppg channel |  |
| i_ppg_ioff_calibration | input | enabling ppg ioff calibration |  |
| i_ecg1_ecg2_sel | input | enabling ecg1 and ecg2 channel |  |
| i_rst_async_ecg | input | reset async for ecg |  |
| i_rst_async_bia | input | reset async for bia |  |
| i_ppg_quokka_elab_sel | input | enabling adsp elab for ppg |  |
| i_data_ADC_out | input | [data_in_size_bit_top-1:0] (from analog circuit) |  |
| i_time_slot_selection | input | [time_slot_sel_bit_top-1:0] odr selection from regmap |  |

| Port name | Direction | Type / Function | TOP CONNECTION / Details |
| --- | --- | --- | --- |
| i_Nmax_to_reg | input | [Nmax_bit_top-1:0] Max value for digital ramp |  |
| i_nstart_case | input | [Nmax_bit_top-1:0] n_start value for frist sterp |  |
| i_debug_rx_ch | input | [rx_ch_frame_n_bit_top-1:0] enabling debug for rx channel |  |
| i_T_delta1_to_reg | input | [delta_bit_top-1:0] delta 1 time for digital ramp |  |
| i_gain_max_set | input | [gain_max_bit_top-1:0] gain max for alc |  |
| i_BTIA_ALC_debug | input | [BTIA_ALC_register_bit_top-1:0] BTIA ALC Debug enable |  |
| i_comp_rx_out | input | input from analog comparator |  |
| i_adc_error_cap8_debug | input | [6:0] adc cap error debug |  |
| i_adc_error_cap7_debug | input | [6:0] adc cap error debug |  |
| i_adc_error_cap6_debug | input | [6:0] adc cap error debug |  |
| i_adc_error_cap5_debug | input | [6:0] adc cap error debug |  |

Source interface table 2: Table 17: Main Controller I/O List (source table from OCR extract page 85)

| Port name | Direction | Type / Function | TOP CONNECTION / Details |
| --- | --- | --- | --- |
| o_adc_error_cap5 | output | [6:0] 0 adc calibration result |  |
| o_adc_error_cap4 | output | [6:0] 0 adc calibration result |  |
| o_adc_error_cap3 | output | [6:0] 0 adc calibration result |  |
| o_adc_error_cap2 | output | [6:0] 0 adc calibration result |  |
| o_adc_error_cap1 | output | [6:0] 0 adc calibration result |  |
| o_adc_error_cap0 | output | [6:0] 0 adc calibration result |  |
| o_pd_cm_buf | output | 1 power down common buffer |  |
| o_pd_rld | output | 1 power down rld |  |
| o_pd_avg1 | output | 1 power down avg1 |  |
| o_pd_avg2 | output | 1 power down avg2 |  |
| o_mc_ahb_clk_en | output | 0 enable for master ahb clock |  |
| o_main_controller_device_status_register | output | [2:0] 0 device status register |  |
| o_main_controller_elab_status_register | output | [3:0] 0 device status register |  |
| o_en_ldo_1v8 | output | 0 enable ldo |  |
| o_en_bufxbio | output | 0 enable biobuffer to analog domain |  |
| o_rx_ch_frame | output | [ppg_rx_channel_bit_top-1:0] 0 selected rx ch frame to analog domain |  |
| o_ldo_ext | output | 0 enable external ldo |  |
| o_ppg_status_register | output | [n_state_bit-1:0] 0 ppg state status register |  |
| o_b_cap_idac | output | [1:0] 0 b cap idac to analog domain |  |
| o_ispu_irq_ctrl | output | [3:0] 0 ispu irq |  |
| o_irq_start_measurement | output | 0 irq start measurement to GPIO |  |
| o_end_calibration | output | 1 end calibration to analog domain |  |
| o_alc_data | output | [data_out_size_bit_top+2:0] 0 alc data to regmap |  |
| o_ppg_data_raw | output | [data_out_size_bit_top+2:0] 0 ppg raw data to regmap |  |

| Port name | Direction | Type / Function | TOP CONNECTION / Details |
| --- | --- | --- | --- |
| o_pd_buf_gpio1 | output | 1 power down for buffer gpio1 |  |
| o_pd_buf_gpio2 | output | 1 power down for buffer gpio2 |  |
| o_adc_data_debug_alc_1 | output | [frame_ppg_debug_alc_top-1:0] 0 alc 1 registers data debug |  |
| o_adc_data_debug_alc_2 | output | [frame_ppg_debug_alc_top-1:0] 0 alc 2 registers data debug |  |
| o_adc_data_debug_ppg_raw | output | [frame_ppg_debug_ppg_raw_top- |  |
| o_en_lead_off | output | 0 mask lead off |  |
| o_mc_busy | output | 0 main controller is working |  |
| o_en_ck_m_p_q | output | 0 clock m and q bia |  |
| o_en_ck_chop_imp | output | 0 clock chopper bia |  |
| o_saturation_flag | output | [n_max_frame-1:0] 0 ppg saturation flag |  |
| o_en_ck_hc2_10k | output | 0 clock chipper ecg |  |
| o_en_clk_16MHz_sleep | output | 1 sleep signal for pmu |  |
| o_gsr_clk_en_16M | output | 0 enable clock for gsr channel |  |

### 9.4 Smart FIFO

General functional description: Provide FIFO storage and AHB/memory access in mutually exclusive Unique or Multiple modes, including sub-FIFO depth/addressing and tagged Sensor Hub data formatting.

Inputs: Digital samples, FIFO mode configuration Outputs: Buffered samples and FIFO status flags

Source interface table 1: Table 28: FIFO Controller I/O List (source table from OCR extract page 112)

| Parameter | Value / Type | Details |
| --- | --- | --- |
| AW | 13 |  |

| Port name | Direction | Type / Function | TOP CONNECTION / Details |
| --- | --- | --- | --- |
| TEST_MODE | input | ScanMode u_pad_mux. |  |
| HCLK | input | AHB Master Handler |  |
| HRESETn | input | u_pmu.resetn_fifo |  |
| HSEL | input | AHB Slave Port u_afa_xbar.hsel_M2 |  |
| HADDR | input | [AW-1:0] AHB Slave Port u_afa_xbar.haddr_M2 |  |
| HMASTER | input | [3:0] AHB Slave Port u_afa_xbar.hmaster_M2 |  |
| HTRANS | input | [1:0] AHB Slave Port u_afa_xbar.htrans_M2 |  |
| HSIZE | input | [2:0] AHB Slave Port u_afa_xbar.hsize_M2 |  |
| HWRITE | input | AHB Slave Port u_afa_xbar. |  |
| HWDATA | input | [31:0] AHB Slave Port u_afa_xbar. |  |
| HREADY | input | AHB Slave Port u_afa_xbar. |  |
| HREADYOUT | output | AHB Slave Port u_afa_xbar. |  |
| HRDATA | output | [31:0] AHB Slave Port u_afa_xbar. |  |
| HRESP | output | AHB Slave Port u_afa_xbar. |  |
| O_FIFO_RAM_A | output | [ADDR_WIDTH- |  |
| O_FIFO_RAM_D | output | [23:0] u_ca_fifo_bist_c.csns_1 |  |
| O_FIFO_RAM_CSN | output | u_ca_fifo_bist_c.dgs_1 |  |
| O_FIFO_RAM_M | output | [23:0] u_ca_fifo_bist_c.wens_1 |  |
| O_FIFO_RAM_WEN | output | u_ca_fifo_bist_c.mgs_1 |  |
| I_FIFO_RAM_Q | input | [23:0] u_ca_fifo_bist_c.dgrs_1 |  |

[DRS-REQ-098] Requirement:

The Smart FIFO block shall implement: The parameter MEM_LOC shall define the first Memory Location of the FIFO RAM: 0xC4000. [TO: IPOS_FIFO_CTRL_00000102].

Covers: SRS-REQ-098

[End]

[DRS-REQ-099] Requirement:

The Smart FIFO block shall implement: The FIFO register map first memory location shall be defined as MEM_LOC + 0x1000: 0xC5000. [TO: IPOS_FIFO_CTRL_00000202].

Covers: SRS-REQ-099

[End]

[DRS-REQ-100] Requirement:

The Smart FIFO block shall implement: The FIFO Controller shall be able to address 1.5 kbyte divided into 32-bit address. [TO: IPOS_FIFO_CTRL_89456557].

Covers: SRS-REQ-100

[End]

[DRS-REQ-101] Requirement:

The Smart FIFO block shall implement: The FIFO Controller shall work at 16 MHz clock frequency. [TO: IPOS_FIFO_CTRL_89459082].

Covers: SRS-REQ-101

[End]

[DRS-REQ-102] Requirement:

The Smart FIFO block shall implement: The FIFO Controller shall have an AHB Slave Interface connected to the AHB BUS and a Memory Master Interface connected to the Physical Memory. [TO: IPOS_FIFO_CTRL_89455261].

Covers: SRS-REQ-102

[End]

[DRS-REQ-103] Requirement:

The Smart FIFO block shall implement: The FIFO Controller shall have two working modes: Multiple FIFO and Unique FIFO. These two modes are mutually exclusive. [TO: IPOS_FIFO_CTRL_89455449].

Covers: SRS-REQ-103

[End]

[DRS-REQ-104] Requirement:

The Smart FIFO block shall implement: As default, the FIFO shall work in Unique FIFO Mode. [TO: IPOS_FIFO_CTRL_89455232].

Covers: SRS-REQ-104

[End]

[DRS-REQ-106] Requirement:

The Smart FIFO block shall implement: The FIFO Controller shall have a 6 i_depth input of 8 bit to be used in Multiple FIFO Mode to define the depth of each subFIFOs. [TO: IPOS_FIFO_CTRL_89457435].

Covers: SRS-REQ-106

[End]

[DRS-REQ-107] Requirement:

The Smart FIFO block shall implement: The system shall satisfy the following behavior: The FIFO 6 memory locations to access each sub-FIFO in Multiple mode are obtained adding multiples of 256 to the memory base address. Example: FIFO Memory Base Address = 0xC4000, then: FIFO 0 memory location 0xC4000 FIFO 1 memory location 0xC4100 FIFO 2 memory location 0xC4200 FIFO 3 memory location 0xC4300 FIFO 4 memory location 0xC4400 FIFO 5 memory location 0xC4500 Table 29: SUB FIFO Memory Address [TO: IPOS_FIFO_CTRL_89457790].

Covers: SRS-REQ-107

[End]

[DRS-REQ-108] Requirement:

The Smart FIFO block shall implement: If input signal HMASTER is set to 0 or 1, the FIFO Controller shall enter Memory mode. [TO: IPOS_FIFO_CTRL_00000008].

Covers: SRS-REQ-108

[End]

[DRS-REQ-109] Requirement:

The Smart FIFO block shall implement: When in Multiple Fifo, the 6 sub-FIFO offset shall coincide with the following table : FIFO OFFSET DATA TYPE ECG FIFO OFFSET ECG0_AC, ECG0_DC BIA FIFO OFFSET BIA_AC_P, BIA_DC_P, BIA_AC_Q, BIA_DC_Q, ECG1_AC, ECG2_DC GSR FIFO OFFSET GSR, ECG1_DC, ECG2_AC PPG FIFO OFFSET PPG Table 31 : FIFO Offset Data Table.

Covers: SRS-REQ-109

[End]

### 9.5 IRQ logic

General functional description: Aggregate and route FIFO, Sensor Hub, ISPU sample-ready, software, and timer interrupt status to configured GPIO/interrupt outputs.

Inputs: FIFO status, DRDY events, threshold/events Outputs: INT1, INT2

[DRS-REQ-110] Requirement:

The IRQ logic block shall implement: The irq_FIFO_U_M_IRQ_STsATUS interrupt shall be connected to FIFO status to bit 29.

Covers: SRS-REQ-110

[End]

[DRS-REQ-111] Requirement:

The IRQ logic block shall implement: The irq_SENSHUB_STS interrupt shall be connected to SENSOR HUB status to bit 28.

Covers: SRS-REQ-111

[End]

[DRS-REQ-112] Requirement:

The IRQ logic block shall implement: The irq_ISPU_MC_INTR_STS interrupt shall be connected to ECG, BIA or PPG samples ready for elaboration to bit 27.

Covers: SRS-REQ-112

[End]

[DRS-REQ-113] Requirement:

The IRQ logic block shall implement: The sw_interrupt interrupt shall be connected to sw interrupt used by ADSP or external user to bit 26:11 16.

Covers: SRS-REQ-113

[End]

[DRS-REQ-114] Requirement:

The IRQ logic block shall implement: The irq_timer interrupt shall be connected to Internal timer to bit 1.

Covers: SRS-REQ-114

[End]

### 9.6 PMU

General functional description: Sequence POR-driven LDO1V8 enable, 64/32 kHz and 16 MHz clocks, clock-ready signaling, and digital-reset release for boot and power modes.

Inputs: VDD/VDD_IO, power-on reset, clock references, clock-start requests, power-mode configuration Outputs: LDO and power-domain controls, clock enable/gating and ready status, reset-release signals, power-state status

Source interface table 1: 13.1.1.2. PMU TOP I/O List (source table from OCR extract page 37)

| Port name | Direction | Type / Function | TOP CONNECTION / Details |
| --- | --- | --- | --- |
| clk_16m | input | wire Clock 16 MHz from analog ring oscillators |  |
| clk_64k | input | wire Clock 64kHz from analog ring oscillators |  |
| POR1V2_1V2 | input | wire POR Signal |  |
| START_CLK_16M | input | wire Clock ready for 16MHz |  |
| PDClockF | input | wire Power Down for 16MHz Ring Oscillator |  |
| i_bio_divider | input | wire Enable 32kHz geneartion from 16MHz when |  |
| EN_LDO | input | wire enable LDO from main controller |  |
| PD_CLK_REG | input | wire power down 16MHz from Regmap |  |
| EN_LDO1V8_REG | input | wire Enable LDO from regmap |  |
| bist_mode | input | wire enable bist mode |  |
| debug_mode | input | wire enable debug |  |
| scan_mode | input | wire enable scan mode |  |
| scan_enable | input | wire start scan mode |  |
| scan_clk | input | wire scan clk |  |
| scan_rstn | input | wire scan rst |  |
| clk_bistctlr | input | wire bist controller and collar clock |  |
| rst_bistctlr_n | input | wire bist controller and collar resetn |  |
| clk_ext_en | input | wire enable external clock from pad |  |
| clk_ext_slow | input | wire external clock 32kHz |  |
| i_en_sh_clk | input | wire enable sensor hub clock |  |
| i_senshub_clk_sel | input | wire |  |
| i_test_clk_fast | input | wire enable test and trimming for 16MHz clock |  |
| i_half_charge_ldo | input | wire enable the low power mode for ldo when |  |
| mask_en_ldo_pdclkf | input | wire mask the control of pdf and ldo control |  |
| clk_en_s0 | input | wire clock gating logic inside stredl_afe_top |  |
| clk_en_s1 | input | wire if present should be HSEL STMC AHB |  |
| clk_en_s2_i2c | input | wire if present should be HSEL STMC AHB |  |
| clk_en_s2_on | input | wire clk gating ON from AHB I2C/SPI Interface |  |
| clk_en_s2_off | input | wire clk gating OFF from AHB I2C/SPI Interface |  |
| clk_en_s3 | input | wire clk gating from AHB Master Main Controller |  |
| clk_en_s4 | input | wire clock gating logic inside stredl_afe_top |  |
| clk_en_s5 | input | wire clock gating logic inside masterblaze_ahb |  |
| clk_en_m0 | input | wire clock gating logic inside stredl_afe_top |  |
| clk_en_m1 | input | wire HSEL M1 from xbar to STMC AHB Slave |  |
| clk_en_m2 | input | wire HSEL M2 from xbar to FIFO |  |
| clk_en_m22 | input | wire HSEL M2 from xbar to MASTERBLAZE |  |
| clk_en_m3 | input | wire HSEL M3 from xbar to Regmap |  |
| iso_i | input | wire from register |  |
| vdden_i | input | wire enable ISPU Power Island |  |
| i_start_operative_16MHz | input | wire enable the lead of check |  |
| main_ctrl_sw_resetn | input | wire soft reset for main controller |  |
| stmc_sw_resetn | input | wire adsp soft reset |  |
| fifo_sw_resetn | input | wire fifo soft reset |  |
| masterblaze_sw_resetn | input | wire sensor hub soft reset |  |
| regmap_sw_resetn | input | wire regmap soft reset |  |
| adsp_run | input | wire adsp is running |  |
| adsp_xbar_sw_en | input | wire adsp clk ahb |  |
| i_en_clk_gsr_16M | input | wire enable the clock for gsr digital circuit |  |
| i_en_ck_m_p_q | input | wire enable quadrature and phase clock |  |
| i_en_clk_mod_debug | input | wire enable the clock modulation debug |  |
| i_en_chop_ref | input | wire enable chopper clock |  |
| i_en_ck_chop_imp | input | wire enable chopper clock for bia |  |
| i_en_ck_hc2_10k | input | wire enable health channel clock |  |
| i_freq_bia_m_p_q | input | wire |  |
| i_en_clk_16MHz_sleep | input | wire enable clock in SLEEP mode when a bio- |  |
| EN_ADC_TEST_LOW_NOISE | input | wire enable adc test low noise |  |
| EN_ADC_TEST_FAST | input | wire enable adc test fast |  |
| if_sh_ck_in | input | wire sensor hub clock interface |  |
| o_EN_LDO1V8 | output | wire enable ldo to analog domain |  |
| o_PD_CLKF | output | wire power down 16MHz clock to analog domain |  |
| o_CLK_16M_READY | output | wire clock ready to the digital domain |  |
| o_DIS_RC_VIREF | output | wire dis rc viref to the analog domain |  |
| pd_osc_clk32k | output | wire power down for clk 64kHz |  |
| tst_clk_32k | output | wire clock slow 32k to main_controller |  |
| tst_clk_16m | output | wire clock fast 16M |  |
| tst_clk_bistctlr | output | wire test clock for bist ctrl |  |
| tst_rst_bistctlr_n | output | wire test reset for bist ctrl |  |
| clk_stredl | output | wire clock gating logic inside stredl_afe_top |  |
| clk_xbar | output | wire xbar clock |  |
| clk_intf | output | wire interface clock |  |
| clk_main_ctrl | output | wire clock for main controller |  |
| clk_stmc | output | wire adsp clock |  |
| clk_stmc_ahb | output | wire adsp clk elaboration |  |
| clk_fifo | output | wire fifo clock |  |
| clk_masterblaze_m | output | wire sensor hub master clock |  |
| clk_masterblaze_s | output | wire sensor hub slave clock |  |
| clk_regmap | output | wire regmap clock |  |
| o_clk_ext_en | output | wire enable external clock |  |
| resetn_stredl | output | wire reset mngmt inside stredl_afe_top |  |
| resetn_xbar | output | wire xbar reset |  |
| resetn_intf | output | wire reset interface |  |
| resetn_main_ctrl | output | wire reset main controller |  |
| resetn_32k_main_ctrl | output | wire reset 32kHz domain main controller |  |
| resetn_stmc | output | wire reset adsp |  |
| resetn_fifo | output | wire reset FIFO |  |
| resetn_masterblaze | output | wire reset sensor hub |  |
| resetn_regmap | output | wire reset regmap |  |
| o_trimming_clk_fast | output | wire trimming output clock fast |  |
| iso_o | output | wire left open . to be connected in synthesis |  |
| vdden_o | output | wire to be connected to psw_ctrl.VDDIEN |  |
| o_ck_bg | output | wire bg clock to analog domain |  |
| o_sh_clk | output | wire sensor hub clock |  |
| o_ck_chop_imp | output | wire chopper bia clock to analog domain |  |
| o_ck_m | output | wire modulation clock to analog domain |  |
| o_ck_hc2_10k | output | wire healt channel 10kHz clock to analog domain |  |
| o_ck_p | output | wire phase clock to analog domain |  |
| o_ck_q | output | wire quadrature clock to analog domain |  |
| o_clk_16M_mc | output | wire 16MHz clock for Main Controller |  |
| o_clk_dft_adc | output | wire 16MHz clock for ADC DFT |  |
| o_clk_gsr_16M | output | wire gsr clock to digital domain |  |
| o_ADC_clk_serial_dft | output | wire adc serial clock in dft |  |
| o_sh_clk_gate | output | wire status clk reg clk sh 16Mhz |  |

Source interface table 2: Table 10: PMU I/O List (source table from OCR extract page 42)

i_freq_bia_m_p_q input wire

[2:0]

frequency selector for phase and

quadrature

39

i_en_clk_16MHz_sleep input wire enable clock in SLEEP mode when a bio-

channel is selected

EN_ADC_TEST_LOW_NOISE input wire enable adc test low noise

EN_ADC_TEST_FAST input wire enable adc test fast

if_sh_ck_in input wire sensor hub clock interface

o_EN_LDO1V8 output wire enable ldo to analog domain

o_PD_CLKF output wire power down 16MHz clock to analog domain

o_CLK_16M_READY output wire clock ready to the digital domain

o_DIS_RC_VIREF output wire dis rc viref to the analog domain

pd_osc_clk32k output wire power down for clk 64kHz

tst_clk_32k output wire clock slow 32k to main_controller

tst_clk_16m output wire clock fast 16M

tst_clk_bistctlr output wire test clock for bist ctrl

tst_rst_bistctlr_n output wire test reset for bist ctrl

clk_stredl output wire clock gating logic inside stredl_afe_top

clk_xbar output wire xbar clock

clk_intf output wire interface clock

clk_main_ctrl output wire clock for main controller

clk_stmc output wire adsp clock

clk_stmc_ahb output wire adsp clk elaboration

clk_fifo output wire fifo clock

clk_masterblaze_m output wire sensor hub master clock

clk_masterblaze_s output wire sensor hub slave clock

clk_regmap output wire regmap clock

o_clk_ext_en output wire enable external clock

40

resetn_stredl output wire reset mngmt inside stredl_afe_top

resetn_xbar output wire xbar reset

resetn_intf output wire reset interface

resetn_main_ctrl output wire reset main controller

resetn_32k_main_ctrl output wire reset 32kHz domain main controller

resetn_stmc output wire reset adsp

resetn_fifo output wire reset FIFO

resetn_masterblaze output wire reset sensor hub

resetn_regmap output wire reset regmap

o_trimming_clk_fast output wire trimming output clock fast

iso_o output wire left open . to be connected in synthesis

vdden_o output wire to be connected to psw_ctrl.VDDIEN

o_ck_bg output wire bg clock to analog domain

o_sh_clk output wire sensor hub clock

o_ck_chop_imp output wire chopper bia clock to analog domain

o_ck_m output wire modulation clock to analog domain

o_ck_hc2_10k output wire healt channel 10kHz clock to analog domain

o_ck_p output wire phase clock to analog domain

o_ck_q output wire quadrature clock to analog domain

o_clk_16M_mc output wire 16MHz clock for Main Controller

o_clk_dft_adc output wire 16MHz clock for ADC DFT

o_clk_gsr_16M output wire gsr clock to digital domain

o_ADC_clk_serial_dft output wire adc serial clock in dft

o_sh_clk_gate output wire status clk reg clk sh 16Mhz

41

Table 10: PMU I/O List

Source interface table 3: 13.1.2.1. PAD MUX TOP I/O List (source table from OCR extract page 49)

| Port name | Direction | Type / Function | TOP CONNECTION / Details |
| --- | --- | --- | --- |
| i_write_protection_en | input |  |  |
| o_write_protection_en | output |  |  |
| i_tap_bist_mode | input |  |  |
| i_EN_0P05HP_ECG0 | input |  |  |
| i_EN_0P05HP_ECG12 | input |  |  |
| IMPQ_PSDRES_UP_02 | input | [2:0] OTP value for Analog block |  |
| IMPQ_PSDRES_DOWN_02 | input | [2:0] OTP value for Analog block |  |
| IMPP_PSDRES_UP_02 | input | [2:0] OTP value for Analog block |  |
| IMPP_PSDRES_DOWN_02 | input | [2:0] OTP value for Analog block |  |
| ECG_PSDRES_DOWN_02 | input | [2:0] OTP value for Analog block |  |
| ECG_PSDRES_UP_02 | input | [2:0] OTP value for Analog block |  |
| IMPQ_PSDRES_UP_05 | input | [2:0] OTP value for Analog block |  |
| IMPQ_PSDRES_DOWN_05 | input | [2:0] OTP value for Analog block |  |
| IMPP_PSDRES_UP_05 | input | [2:0] OTP value for Analog block |  |
| IMPP_PSDRES_DOWN_05 | input | [2:0] OTP value for Analog block |  |
| ECG_PSDRES_DOWN_05 | input | [2:0] OTP value for Analog block |  |
| ECG_PSDRES_UP_05 | input | [2:0] OTP value for Analog block |  |
| IMPQ_PSDRES_UP | output | [2:0] OTP value for Analog block |  |
| IMPQ_PSDRES_DOWN | output | [2:0] OTP value for Analog block |  |
| IMPP_PSDRES_UP | output | [2:0] OTP value for Analog block |  |
| IMPP_PSDRES_DOWN | output | [2:0] OTP value for Analog block |  |
| ECG_PSDRES_DOWN | output | [2:0] OTP value for Analog block |  |
| ECG_PSDRES_UP | output | [2:0] OTP value for Analog block |  |
| i_TS_IRQ_ST | input |  |  |
| i_DCCK_IRQ_ST | input |  |  |
| o_en_tx_1V2 | input |  |  |
| o_tx_preset_1V2 | input |  |  |
| o_tx1_outlab_sel_frame_x_1V2 | input |  |  |
| sel_irq_gpio1 | input | [3:0] sel irq gpi1 {000:NC, 001:i_fifo_irq, |  |
| sel_irq_gpio2 | input | [3:0] sel irq gpi2 {000:NC, 001:i_fifo_irq, |  |
| sel_irq_gpio3 | input | [3:0] sel irq gpi3 {000:NC, 001:i_fifo_irq, |  |
| sel_irq_gpio4 | input | [3:0] sel irq gpi4 {000:NC, 001:i_fifo_irq, |  |
| DCCK_P_en | input |  |  |
| DCCK_N_en | input |  |  |
| MODE_OPERATION | input |  |  |
| o_pd_p | output |  |  |
| o_pd_n | output |  |  |
| O_DCCK_P_i | input |  |  |
| O_DCCK_N_i | input |  |  |
| if_gpio1_in_reg | output |  |  |
| if_gpio1_en_reg | input |  |  |
| if_gpio1_out_reg | input |  |  |
| if_gpio2_in_reg | output |  |  |
| if_gpio2_en_reg | input |  |  |
| if_gpio2_out_reg | input |  |  |
| if_gpio3_in_reg | output |  |  |
| if_gpio3_en_reg | input |  |  |
| if_gpio3_out_reg | input |  |  |
| if_gpio4_in_reg | output |  |  |
| if_gpio4_en_reg | input |  |  |
| if_gpio4_out_reg | input |  |  |
| o_ext_reg_dt_afe | input | [7:0] i2c/spi read data u_regmap_otp |  |
| o_ext_reg_dt | output | [7:0] i2c/spi read data output |  |
| i_ext_reg_add | input | [7:0] i2c/spi address |  |
| i_ext_reg_dt | input | [7:0] i2c/spi write data |  |
| i_ext_reg_ck_wr | input |  |  |
| i_ext_reg_we | input |  |  |
| if_sclk_in | input |  |  |
| if_sclk_i3c_in | input |  |  |
| if_sclk_out | output |  |  |
| if_sclk_en | output |  |  |
| if_sclk_tud | output |  |  |
| if_sda_sdi_in | input |  |  |
| if_sda_sdi_i3c_in | input |  |  |
| if_sda_sdi_out | output |  |  |
| if_sda_sdi_en | output |  |  |
| if_sda_sdi_tud | output |  |  |
| if_cs_in | input |  |  |

Source interface table 4: Table 1313: PAD MUX I/O List (source table from OCR extract page 57)

o_ADC_start output

adc star output

i_N_clk input [6-

1:0]

n clk start time high

i_saturation_irq input

interrupt saturation from main controller

i_end_boot input

end boot signal from adsp

quokka_run input

status quokka elaboration 1:run 0:not

elaboration

sensor_hub_status input

status sensor_hob from regmap

SH_CKL_EN input

sensorhub clk enable from regmap

(master_end_op_clr or sleep_master_clr or

ahb_master_on_clr)

i_t_force_1v2 input

input force signal to IDDQ test

o_t_force_1v2 output

force signal to IDDQ test

i_bfail_fifo input

bist fail fifo ram

i_bfail_qk input

bist fail adsp ram

i_bfail_ispu1 input

bist fail ispu ram instruction mem

i_bfail_ispu2 input

bist fail ispu ram data mem 1

56

i_bfail_ispu3 input

bist fail ispu ram data mem 2

i_bfail_ispu4 input

bist fail ispu ram data mem 3

i_bend input

bist end signal

i_bbad input

signal fail bist test

o_bypass output

bypass memory signal

Table 1313: PAD MUX I/O List

57

13.1.2.2. Requirements

[DRS-REQ-003] Requirement:

The PMU block shall implement: When i_POR1V2_1V2 is low, o_EN_LDO1V8 shall be low. [TO: IPOS_PMU].

Covers: SRS-REQ-003

[End]

[DRS-REQ-005] Requirement:

The PMU block shall implement: When i_POR1V2_1V2 is low, o_CLK_16M_READY shall be low. [TO: IPOS_PMU].

Covers: SRS-REQ-005

[End]

[DRS-REQ-007] Requirement:

The PMU block shall implement: When i_POR1V2_1V2 rises, after at least 10 cycles of i_clk_64k (64kHz), o_EN_LDO1V8 shall be high and the clk_64k_divided is considered stable. [TO: IPOS_PMU].

Covers: SRS-REQ-007

[End]

[DRS-REQ-008] Requirement:

The PMU block shall implement: When clk_64k_divided is stable, after at least 352 cycles of clk_64k_divided (32kHz), o_DIS_RC_VIREF shall be low. [TO: IPOS_PMU].

Covers: SRS-REQ-008

[End]

[DRS-REQ-009] Requirement:

The PMU block shall implement: When i_mask_en_ldo_pdclkf_reg and o_EN_LDO1V8 are set to 1, o_PD_CLKF shall be low after at least 40 cycles of the clk_64k_divided. [TO: IPOS_PMU].

Covers: SRS-REQ-009

[End]

[DRS-REQ-010] Requirement:

The PMU block shall implement: When i_START_CLK_16M is high, after at least 20 cycles of i_clk_16M (16MHz), o_CLK_16M_READY shall be high. [TO: IPOS_PMU].

Covers: SRS-REQ-010

[End]

[DRS-REQ-011] Requirement:

The PMU block shall implement: When i_START_CLK_16M is high for the first time, after at least 20 cycles of i_clk_16M (16MHz), o_main_resetn shall be high. [TO: IPOS_PMU].

Covers: SRS-REQ-011

[End]

[DRS-REQ-012] Requirement:

The PMU block shall implement: The stbio1_top.clk_16m signal shall be connected to the stbio1_top.dig_wrapper_LV.u_pmu.clk_16m signal, Delay: No; Clock gating: No; F max: 16 MHz.

Covers: SRS-REQ-012

[End]

[DRS-REQ-013] Requirement:

The PMU block shall implement: The stbio1_top.clk_16m signal shall be connected to the stbio1_top.dig_wrapper_LV.u_pmu.clk signal, Delay: No; Clock gating: No; F max: 64 kHz.

Covers: SRS-REQ-013

[End]

[DRS-REQ-014] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmu.tst_clk_32k signal shall be connected to the stbio1_top.dig_wrapper_LV.u_main_controller_top.i_clk_32kHz signal, Delay: No; Clock gating: No; F max: 32 kHz.

Covers: SRS-REQ-014

[End]

[DRS-REQ-015] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmu.clk_16m_dft signal shall be connected to the stbio1_top.dig_wrapper_LV.u_ADSP.stmc_clk signal, Clock gating: stbio1_top.dig_wrapper_LV.u_pm u.adsp_run | stbio1_top.dig_wrapper_LV.u_pm u.all_test_mode | ~stbio1_top.dig_wrapper_LV.u_p ad_mux.scan_enable; F max: 16 MHz.

Covers: SRS-REQ-015

[End]

[DRS-REQ-016] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmu.w_sh_clk signal shall be connected to the stbio1_top.dig_wrapper_LV.u_masterblaze_ahb.sys_clk signal, Clock gating: stbio1_top.dig_wrapper_LV.u_pm u.i_en_sh_clk | stbio1_top.dig_wrapper_LV.u_pm u.all_test_mode | ~stbio1_top.dig_wrapper_LV.u_p ad_mux.scan_enable; F max: 16 MHz.

Covers: SRS-REQ-016

[End]

[DRS-REQ-017] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmu.clk_16m_dft signal shall be connected to the stbio1_top.dig_wrapper_LV.u_xbar_afe.hclk signal, Clock gating: stbio1_top.dig_wrapper_LV.u_pm u.clk_xbar_en | stbio1_top.dig_wrapper_LV.u_pm u.all_test_mode | ~stbio1_top.dig_wrapper_LV.u_p ad_mux.scan_enable; F max: 16 MHz.

Covers: SRS-REQ-017

[End]

[DRS-REQ-018] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmuclk_xbar signal shall be connected to the stbio1_top.dig_wrapper_LV.u.fifo.HCLK signal, Clock gating: stbio1_top.dig_wrapper_LV.u_pm u.clk_en_m2 | stbio1_top.dig_wrapper_LV.u_pm u.all_test_mode | stbio1_top.dig_wrapper_LV.u_pm u.clk_en_m2_reg | ~stbio1_top.dig_wrapper_LV.u_p ad_mux.scan_enable; F max: 16 MHz.

Covers: SRS-REQ-018

[End]

[DRS-REQ-019] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmu.clk_stredl signal shall be connected to the stbio1_top.dig_wrapper_LV.u_ispu.clk signal, Clock gating: No; F max: 16 MHz.

Covers: SRS-REQ-019

[End]

[DRS-REQ-020] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmu.clk_xbar signal shall be connected to the stbio1_top.dig_wrapper_LV.u_regmap_otp.hclk signal, Clock gating: stbio1_top.dig_wrapper_LV.u_pm u.clk_en_m3 | stbio1_top.dig_wrapper_LV.u_pm u.all_test_mode | stbio1_top.dig_wrapper_LV.u_pm u.clk_en_m3_reg | stbio1_top.dig_wrapper_LV.u_pm u.clk_en_s3 | ~stbio1_top.dig_wrapper_LV.u_p ad_mux.scan_enable; F max: 16 MHz.

Covers: SRS-REQ-020

[End]

[DRS-REQ-021] Requirement:

The PMU block shall implement: The u_stbio1_top.u_pmu.u_device_startup.tst_clk_32k signal shall be connected to the stbio1_top.dig_wrapper_LV.tst_clk_32k signal, Delay: No; Clock gating: No; F max: 32 kHz.

Covers: SRS-REQ-021

[End]

[DRS-REQ-022] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmu.tst_clk_16m signal shall be connected to the stbio1_top.dig_wrapper_LV.u_main_controller_top.i_clk_16MHz signal, Clock gating: stbio1_top.dig_wrapper_LV.u_pm u.i_en_clk_16MHz_sleep | stbio1_top.dig_wrapper_LV.u_pm u.scan_mode | ~stbio1_top.dig_wrapper_LV.u_p ad_mux.scan_enable; F max: 16 MHz.

Covers: SRS-REQ-022

[End]

[DRS-REQ-023] Requirement:

The PMU block shall implement: The u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_stmc signal shall be connected to the u_stbio1_top.dig_wrapper_LV.u_ADSP.HRESETn signal.

Covers: SRS-REQ-023

[End]

[DRS-REQ-024] Requirement:

The PMU block shall implement: The u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_masterblaze signal shall be connected to the u_stbio1_top.dig_wrapper_LV.u_masterblaze_ahb.HRESETn signal.

Covers: SRS-REQ-024

[End]

[DRS-REQ-025] Requirement:

The PMU block shall implement: The u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_regmap signal shall be connected to the u_stbio1_top.dig_wrapper_LV.u_regmap_otp.hresetn signal.

Covers: SRS-REQ-025

[End]

[DRS-REQ-026] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmu signal shall be connected to the u_device_startup.u_por_main_resetn.RST_N_FF signal.

Covers: SRS-REQ-026

[End]

[DRS-REQ-027] Requirement:

The PMU block shall implement: The u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_regmap signal shall be connected to the u_stbio1_top.dig_wrapper_LV.u_ispu.main_rst_n signal.

Covers: SRS-REQ-027

[End]

[DRS-REQ-028] Requirement:

The PMU block shall implement: The u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_fifo signal shall be connected to the u_stbio1_top.dig_wrapper_LV.u_multi_channel_fifo_top.HRESETn signal.

Covers: SRS-REQ-028

[End]

[DRS-REQ-029] Requirement:

The PMU block shall implement: The u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_main_ctrl signal shall be connected to the u_stbio1_top.dig_wrapper_LV.u_main_controller_top.i_rstn signal.

Covers: SRS-REQ-029

[End]

[DRS-REQ-030] Requirement:

The PMU block shall implement: The u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_32k_main_ctrl signal shall be connected to the u_stbio1_top.dig_wrapper_LV.u_main_controller_top.i_rstn_sync_32 signal.

Covers: SRS-REQ-030

[End]

### 9.7 I2C_SPI_AHB

General functional description: Accept I2C/SPI accesses and translate them as needed into AHB transactions for device configuration and readback, including ISPU access.

Inputs: Host bus transactions Outputs: Protocol-compliant responses

Source interface table 1: Table 24: I2C/SPI AHB I/O List (source table from OCR extract page 101)

i_if_sda_sdi_in input  SDA/SDI from pad

o_if_sda_sdi_en_n output  SDA/SDI pad enable

o_if_sda_sdi_out output  SDA/SDI to pad

o_ctrl_reg_7 output [7:0] 8 configuration bit set by

local register (addr. 0x5e)

i_ext_reg_dt input  [7:0] Data from external register

block (if present)

o_ext_reg_wr_add output [7:0] Write address to external

register block (if present)

o_ext_reg_rd_add output [7:0] Read address to external

register block (if present)

o_ext_reg_dt output [7:0] Data to external register

block (if present)

o_ext_reg_ck_wr output  Write enable for external

register block (if present)

o_ext_reg_we output

Write direct enable for

external register block (if

present)

rst_16m_n   Sw reset for AHB domain

sys_16m_clk input  AHB clock

o_hselm output  AHB_M

o_haddrm output [31:0] AHB_M

o_htransm output [1:0] AHB_M

o_hmasterm output [3:0] AHB_M

o_hwritem output  AHB_M

o_hsizem output [2:0] AHB_M

o_hmastlockm output  AHB_M

o_hwdatam output [31:0] AHB_M

o_hburstm output [2:0] AHB_M

o_hprotm output [3:0] AHB_M

i_hrdatam input  [31:0] AHB_M

i_hreadym input  AHB_M

i_hrespm input  AHB_M

o_gt_ck_on output

Pulse on interface clock to

inform system that AHB

clock must be switched on

100

o_gt_ck_off output

Pulse on AHB clock to

inform system that AHB

clock can be switched off

o_ahb_error output

Error on AHB side. Active

if bit 1 or  of register at

address 0x4f are active.

o_rd_byte_nr_err output  Active when

o_req_error output

Active when number of

byte is missing for read or

timeout between double

yte write and read is

expired

scan_mode input  Scan mode

scan_enable input  Scan enable

scan_clk input  Scan clk

scan_rst input      Scan reset

Table 24: I2C/SPI AHB I/O List

101

[DRS-REQ-067] Requirement:

The I2C_SPI_AHB block shall implement: I2C interface shall support all the specified modalities (Standard, FM, FM+) and, if configured, HS is also supported.

Covers: SRS-REQ-067

[End]

[DRS-REQ-068] Requirement:

The I2C_SPI_AHB block shall implement: Clock frequency ratio between system (AHB) and I2C/SPI interfaces domains shall be at least 2 times. [TO: IPOS_I2C_SPI_AHB_0006] [Vpriority High].

Covers: SRS-REQ-068

[End]

[DRS-REQ-069] Requirement:

The I2C_SPI_AHB block shall implement: An I2C/SPI write access to the address 0x60-0x67 shall be translated in an AHB write access to the configured AHB address (contained in registers SAM_ADDR_BYTE0/1/2/3, address 0x50/0x51/0x52/0x53). [TO: IPOS_I2C_SPI_AHB_0009] [Vpriority High].

Covers: SRS-REQ-069

[End]

[DRS-REQ-070] Requirement:

The I2C_SPI_AHB block shall implement: A double byte write access to the address 0x68-0x6f (with data that is the number of bytes that the master wants to read) shall start the data preload from AHB in order to have the internal fifo not empty when the read request will be done by the master. [TO: IPOS_I2C_SPI_AHB_0010] [Vpriority High].

Covers: SRS-REQ-070

[End]

[DRS-REQ-071] Requirement:

The I2C_SPI_AHB block shall implement: An I2C/SPI read access to the address 0x70-0x7f shall be translated in an AHB read access to the configured AHB address (contained in registers SAM_ADDR_BYTE0/1/2/3, address 0x50/0x51/0x52/0x53). [TO: IPOS_I2C_SPI_AHB_0011] [Vpriority High].

Covers: SRS-REQ-071

[End]

[DRS-REQ-072] Requirement:

The I2C_SPI_AHB block shall implement: The system shall satisfy the following behavior: The IP has a serial interface (I2C or SPI) and a parallel bus interface AHB. [TO: IPOS_I2C_SPI_AHB_0007].

Covers: SRS-REQ-072

[End]

[DRS-REQ-073] Requirement:

The I2C_SPI_AHB block shall implement: On AHB interface, data size shall be limited to BYTE and WORD. [TO: IPOS_I2C_SPI_AHB_0361].

Covers: SRS-REQ-073

[End]

[DRS-REQ-074] Requirement:

The I2C_SPI_AHB block shall implement: On AHB interface, BURST shall not be supported as well as LOCKED transfers. [TO: IPOS_I2C_SPI_AHB_0362].

Covers: SRS-REQ-074

[End]

### 9.8 ISPU

General functional description: Represent the ISPU digital architecture block and its source/destination responsibilities described by Stage 2 interaction evidence.

Inputs: Defined by interaction matrix ingress and upstream requirement evidence Outputs: Defined by interaction matrix egress and upstream requirement evidence

Source interface table 1: Table 32: ISPU I/O List (source table from OCR extract page 129)

select the

clock and

memory

ports in

ATPG mode

0:

Functional

Clock /

Functional

Ports 1:

Test Clock/

Test Ports

u_ca_fifo_bist_c_oen_1

ca_stredl_bist_c_tst_mem_

selck_3 input BIST To

select the

u_ca_fifo_bist_c_oen_1

128

clock and

memory

ports in

ATPG mode

0:

Functional

Clock /

Functional

Ports 1:

Test Clock/

Test Ports

ca_stredl_bist_c_tst_mem_

selck_4 input

BIST To

select the

clock and

memory

ports in

ATPG mode

0:

Functional

Clock /

Functional

Ports 1:

Test Clock/

Test Ports

u_ca_fifo_bist_c_oen_1

ca_stredl_bist_c_par_mem

sel input

BIST

Parallel

Memory

Selection

Bus

{ 1'b0, u_ca_fifo_bist_c_oen_1,

u_ca_fifo_bist_c_oen_1,

u_ca_fifo_bist_c_oen_1 }

Table 32: ISPU I/O List

129

13.3.1. Requirements

[DDS_STBIO1_0500] Requirement: ] Requirement:

Interrupt lines list:

Interrupt bit Description

irq_FIFO_U_M_IRQ_STsATUS 29 FIFO status [DDS_STBIO1_0501]

irq_SENSHUB_STS 28 SENSOR HUB status [DDS_STBIO1_0502]

irq_ISPU_MC_INTR_STS 27 ECG, BIA or PPG samples ready for

elaboration[DDS_STBIO1_0503]

sw_interrupt 26:11 16 sw interrupt used by ADSP or external

user [DDS_STBIO1_0504]

irq_timer 1 Internal timer [DDS_STBIO1_0505]

[END]

[DDS_STBIO1_0506] Description:

The ISPU is a dedicated IP for implementing complex processing algorithms. By default, it is

turned off. This is located inside a power island

[End]

[DRS-REQ-115] Requirement:

The ISPU block shall implement: The system shall satisfy the following behavior: Requirement: To put the ISPU into sleep mode, it is necessary to perform a read at address 0x80054.

Covers: SRS-REQ-115

[End]

[DRS-REQ-116] Requirement:

The ISPU block shall implement: The system shall satisfy the following behavior: Requirement: To wake up the ISPU, it is necessary to trigger an interrupt.

Covers: SRS-REQ-116

[End]

[DRS-REQ-117] Requirement:

The ISPU block shall implement: The system shall satisfy the following behavior: Requirement: When the ISPU is in sleep mode, it is possible to perform reads and writes from the data and code memory by triggering an AHB transition.

Covers: SRS-REQ-117

[End]

[DRS-REQ-118] Requirement:

The ISPU block shall implement: To turn off the ISPU shall be:.

- Disable the power island by writing to regmap POWER CONFIGURATION REGISTER via AHB interface (ADDRESS=0xCA000, DATA=0x00)

Covers: SRS-REQ-118

[End]

[DRS-REQ-119] Requirement:

The ISPU block shall implement: After the initial boot phase, to turn on the ISPU shall be:.

- Select page 3 of the regmap (ADDRESS=0x04, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable user mode by writing to regmap (ADDRESS=0x5E, DATA=0x01)

- Enable the power island by writing to regmap POWER CONFIGURATION REGISTER via AHB interface (ADDRESS=0xCA000, DATA=0x01)

Covers: SRS-REQ-119

[End]

## 6.4 Source Function Context

### 6.4.1 BIST MODE

[DRS-REQ-153] Requirement:

After the initial boot phase, to turn on the Debug mode shall be:

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable the power island

- Enable bist mode by writing to regmap (ADDRESS=0x3E, DATA=0x04)

Covers: SRS-REQ-153

[End]

Source paragraph: Section 18.5 BIST MODE (under Section 18 Digital DFT), paragraph 030 (page 161)

### 6.4.2 BOOT Phase

[DRS-REQ-123] Requirement:

To check the end of the BOOT phase, the user shall read the main_controller_status_register (DEVICE_STATUS_REG register [2:0]) and follow these steps:

- Read the DEVICE_STATUS_REG [2:0]

- Check if the Value is 1 (BOOT phase still in process)

- If the value is 2, then the BOOT phase is ended and the device is in IDLE. [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-123

[End]

Source paragraph: Section 15.1 BOOT Phase (under Section 15 Measurement Cycle Examples), paragraph 013 (page 145)

### 6.4.3 Configuration Phase

[DRS-REQ-124] Requirement:

During the Configuration Phase (IDLE STATE of the Device_FSM), the user shall be able to:

- Configure Bio-Channel or Optical Channel, the ADC Calibration and then Start a measurement cycle.

- Put the device in Debug-Mode. [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-124

[End]

Source paragraph: Section 15.2 Configuration Phase (under Section 15 Measurement Cycle Examples), paragraph 007 (page 146)

### 6.4.4 Debug mode

[DRS-REQ-149] Requirement:

After the initial boot phase, to turn on the Debug mode shall be:

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable debug mode by writing to regmap (ADDRESS=0x3E, DATA=0x08)

Covers: SRS-REQ-149

[End]

Source paragraph: Section 18.2 Debug mode (under Section 18 Digital DFT), paragraph 024 (page 159)

### 6.4.5 ECG Only

[DRS-REQ-125] Requirement:

- The user shall configure time slot duration in DEVICE_CONFIG_OPERATION register [4:2] [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-125

[End]

Source paragraph: Section 1 TIME SLOT LENGTH: (under Section 15.3.1 ECG Only), paragraph 008 (page 148)

[DRS-REQ-126] Requirement:

- The user shall select active channel, in this case only ECG0_SEL and DS_SEL (Data Storage Mode) in DEVICE_CONFIG_CHANNEL register writing 0x21. [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-126

[End]

Source paragraph: Section 2 SELECT CHANNEL: (under Section 15.3.1 ECG Only), paragraph 012 (page 148)

[DRS-REQ-127] Requirement:

- The user shall select a division index for ECG, configuring M_ECG_l and M_ECG_h in SET_ECG_FREQ_l register and SET_ECG_FREQ_h register. [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-127

[End]

Source paragraph: Section 3 SELECT DIVISION INDEX FOR CONFIGURED CHANNELS: (under Section 15.3.1 ECG Only), paragraph 017 (page 148)

[DRS-REQ-128] Requirement:

- The user shall set the number of clocks for sampling ECG data from ADC in ADC_CONFIG_ECG register. [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-128

[End]

Source paragraph: Section 4 CONFIGURE ADC SAMPLING PERIOD: (under Section 15.3.1 ECG Only), paragraph 022 (page 148)

[DRS-REQ-129] Requirement:

- The User shall set at 1 HC_EN in ECG_CTRL register. [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-129

[End]

Source paragraph: Section 5 GENERAL CONFIG: (under Section 15.3.1 ECG Only), paragraph 027 (page 148)

[DRS-REQ-130] Requirement:

- The user shall start measurement writing register DEVICE_CONFIG_OPERATION [0] at 1. [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-130

[End]

Source paragraph: Section 6 SELECT OPERATIVE MODE: (under Section 15.3.1 ECG Only), paragraph 031 (page 148)

[DRS-REQ-131] Requirement:

- To check data in unique FIFO (default config) the user shall check the first MSB which will have to be equal to the table in DATA TAG Table. [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-131

[End]

Source paragraph: Section 7 CHECK DATA: (under Section 15.3.1 ECG Only), paragraph 036 (page 148)

### 6.4.6 ECG and BIA

[DRS-REQ-132] Requirement:

- The user shall configure the time slot duration in DEVICE_CONFIG_OPERATION register [4:2].

Covers: SRS-REQ-132

[End]

Source paragraph: Section 1 TIME SLOT LENGTH: (under Section 15.3.2 ECG and BIA), paragraph 012 (page 150)

[DRS-REQ-133] Requirement:

- The user shall select active channel, in this case ECG0_SEL, BIA_SEL and DS_SEL (Data Storage Mode) in DEVICE_CONFIG_CHANNEL writing 0x23. [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-133

[End]

Source paragraph: Section 2 SELECT CHANNEL: (under Section 15.3.2 ECG and BIA), paragraph 016 (page 150)

[DRS-REQ-134] Requirement:

- The user shall select a division index for ECG, configuring M_ECG_l and M_ECG_h in SET_ECG_FREQ_l register and SET_ECG_FREQ_h register. [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-134

[End]

Source paragraph: Section 3 SELECT DIVISION INDEX FOR CONFIGURED CHANNELS: (under Section 15.3.2 ECG and BIA), paragraph 021 (page 150)

[DRS-REQ-135] Requirement:

- The user shall select a division index for ECG, configuring M_BIA_l and M_BIA_h in SET_BIA_FREQ_l register and SET_BIA_FREQ_h register. [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-135

[End]

Source paragraph: Section 3 SELECT DIVISION INDEX FOR CONFIGURED CHANNELS: (under Section 15.3.2 ECG and BIA), paragraph 025 (page 150)

[DRS-REQ-136] Requirement:

- The user shall set number of clocks for sampling ECG and BIA data from ADC in ADC_CONFIG_ECG register. [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-136

[End]

Source paragraph: Section 4 CONFIGURE ADC SAMPLING PERIOD: (under Section 15.3.2 ECG and BIA), paragraph 029 (page 150)

[DRS-REQ-137] Requirement:

- The user shall set at 1 HC_EN in ECG_CTRL register. [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-137

[End]

Source paragraph: Section 5 GENERAL CONFIG: (under Section 15.3.2 ECG and BIA), paragraph 033 (page 150)

[DRS-REQ-138] Requirement:

- The user shall set at 1 IMP_EN in IMP_CTRL register. [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-138

[End]

Source paragraph: Section 5 GENERAL CONFIG: (under Section 15.3.2 ECG and BIA), paragraph 036 (page 150)

[DRS-REQ-139] Requirement:

- The user shall start measurement writing register DEVICE_CONFIG_OPERATION [0] at 1. [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-139

[End]

Source paragraph: Section 6 SELECT OPERATIVE MODE: (under Section 15.3.2 ECG and BIA), paragraph 040 (page 150)

[DRS-REQ-140] Requirement:

- To check data in unique FIFO (default config) the user shall check the first MSB which will have to be equal to the one described in the DATA TAG Table. [TO: IPOS_MAIN_CTRL]

Covers: SRS-REQ-140

[End]

Source paragraph: Section 7 CHECK DATA: (under Section 15.3.2 ECG and BIA), paragraph 044 (page 150)

### 6.4.7 Other source function context

[DRS-REQ-063] Requirement:

If bit OTP_PRG10[31] in OTP Registers, called write_protection_en, is set to 1, and STBIO1 is not in debug_mode, register map shall refuse any incoming write request if the transaction address is within the OTP Registers space (from 0xCD000 to 0xCD02F).

Covers: SRS-REQ-063

[End]

Source paragraph: Unheaded source context; OCR numbered-list label is not a source paragraph

### 6.4.8 PAD Mux

[DRS-REQ-031] Requirement:

If MODE_CONFIG register is 0x00 the operative mode shall be functional. [Vpriority High]

Covers: SRS-REQ-031

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 010 (page 59)

[DRS-REQ-032] Requirement:

If MODE_CONFIG register is 0x01 the operative mode shall be SCAN.

Covers: SRS-REQ-032

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 014 (page 59)

[DRS-REQ-033] Requirement:

If MODE_CONFIG register is 0x04 the operative mode shall be BIST.

Covers: SRS-REQ-033

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 018 (page 59)

[DRS-REQ-034] Requirement:

If MODE_CONFIG register is 0x08 the operative mode shall be DEBUG.

Covers: SRS-REQ-034

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 022 (page 59)

[DRS-REQ-035] Requirement:

If MODE_CONFIG register is 0x10 the operative mode shall be ISPU_DEBUG. [Vpriority High]

Covers: SRS-REQ-035

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 026 (page 59)

[DRS-REQ-036] Requirement:

If MODE_CONFIG register is 0x20 the operative mode shall be Bist MEMSAFE mode. [Vpriority High]

Covers: SRS-REQ-036

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 030 (page 59)

[DRS-REQ-037] Requirement:

If MODE_CONFIG register is 0x40 the operative mode shall be Bist Bypass mode. [Vpriority High]

Covers: SRS-REQ-037

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 034 (page 59)

[DRS-REQ-038] Requirement:

In function mode, the GPIO1,2,3,4_out shall be General-Purpose if sel irq gpi1,2,3,4 is 0x00 and ext_cap_en is “0” [Vpriority High]

Covers: SRS-REQ-038

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 038 (page 59)

[DRS-REQ-039] Requirement:

In function mode, the GPIO1,2,3,4_out shall be irq_fifo if sel irq gpio1,2,3,4 is 0x01 and ext_cap_en is “0” [Vpriority High]

Covers: SRS-REQ-039

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 043 (page 59)

[DRS-REQ-040] Requirement:

In function mode, the GPIO1,2,3,4_out shall be i_LED_Pilot if sel irq gpi1,2,3,4 is 0x02 and ext_cap_en is “0” [Vpriority High]

Covers: SRS-REQ-040

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 051 (page 59)

[DRS-REQ-041] Requirement:

In function mode, the GPIO1,2,3,4_out shall be i_DCCK_irq if sel irq gpi1,2,3,4 is 0x03 and ext_cap_en is “0” [Vpriority High]

Covers: SRS-REQ-041

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 004 (page 60)

[DRS-REQ-042] Requirement:

In function mode, the GPIO1,2,3,4 output shall be i_error_time_slot if sel irq gpi1,2,3,4 is 0x04 and ext_cap_en is “0” [Vpriority High]

Covers: SRS-REQ-042

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 009 (page 60)

[DRS-REQ-043] Requirement:

In function mode, the GPIO1,2,3,4 output shall be i_time_slot_irq if sel irq gpi1,2,3,4 is 0x05 and ext_cap_en is “0” [Vpriority High]

Covers: SRS-REQ-043

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 014 (page 60)

[DRS-REQ-044] Requirement:

If O_DCCK_P_i changes its value from ‘0’ to ‘1’ and DCCK_IRQ_MASK is set to ‘1’, then irq_DCCK_IRQ_ST, output of u_regmap_otp, shall be set to ‘1’

Covers: SRS-REQ-044

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 018 (page 71)

[DRS-REQ-045] Requirement:

If O_DCCK_N_i changes its value from ‘0’ to ‘1’ and DCCK_IRQ_MASK is set to ‘1’, then irq_DCCK_IRQ_ST, output of u_regmap_otp, shall be set to ‘1’

Covers: SRS-REQ-045

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 023 (page 71)

[DRS-REQ-046] Requirement:

If O_DCCK_P_i changes its value from ‘1’ to ‘0’ and DCCK_IRQ_MASK is set to ‘1’, then irq_DCCK_IRQ_ST, output of u_regmap_otp, shall be set to ‘1’

Covers: SRS-REQ-046

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 028 (page 71)

### 6.4.9 PPG Only

[DRS-REQ-141] Requirement:

- The User shall configure time slot duration in DEVICE_CONFIG_OPERATION register [4:2].

Covers: SRS-REQ-141

[End]

Source paragraph: Section 1 TIME SLOT LENGTH: (under Section 15.3.3 PPG Only), paragraph 010 (page 152)

[DRS-REQ-142] Requirement:

- The user shall configure a division index PPG, writing M_PPG_l and M_PPG_h in SET_PPG_FREQ_l register and SET_PPG_FREQ_h register.

Covers: SRS-REQ-142

[End]

Source paragraph: Section 3 SELECT DIVISION INDEX FOR CONFIGURED CHANNELS: (under Section 15.3.3 PPG Only), paragraph 018 (page 152)

[DRS-REQ-143] Requirement:

- The user shall configure the number of clocks for sampling PPG data from ADC in ADC_CONFIG_PPG register.

Covers: SRS-REQ-143

[End]

Source paragraph: Section 4 CONFIGURE ADC SAMPLING PERIOD: (under Section 15.3.3 PPG Only), paragraph 022 (page 152)

[DRS-REQ-144] Requirement:

- The user shall select active channel, in this case only PPG_SEL and DS_SEL (data Storage) in DEVICE_CONFIG_CHANNEL register shall be set to 0x30.

Covers: SRS-REQ-144

[End]

Source paragraph: Section 2 SELECT CHANNEL: (under Section 15.3.3 PPG Only), paragraph 014 (page 152)

[DRS-REQ-145] Requirement:

The user shall configure each frame following these steps:

5.1 ALC PARAMETERS:

- T1 in PPG_ALC_CONFIG_PARAM_1 register.

- T2, GAIN_START_SET, GAIN_MAX_SET in PPG_ALC_CONFIG_PARAM_2 register.

5.2 DIGITAL RAMP PARAMETERS:

- N_MAX, T_DELTA in DIGITAL_RAMP_CONFIG_PARAM_1 register.

- DELTA, T_DELTA1 in DIGITAL_RAMP_CONFIG_PARAM_2 register.

- DELTA1 in DIGITAL_RAMP_CONFIG_PARAM_3 register.

5.3 GENERAL PPG PARAMETERS:

- N_PPG_FRAME, WAVE_VELOCITY_MODE, SEL_PPG_ELAB in GENERAL_PPG_PARAMETERS_1 register.

- T_TX_STARTUP in GENERAL_PPG_PARAMETERS_2 register.

- T_RX_SET, PPG_START_UP_TIME_l in GENERAL_PPG_PARAMETERS_3 register.

- PPG_START_UP_TIME_h in GENERAL_PPG_PARAMETERS_4 register.

5.4 PARAMETERS FOR EACH FRAME CONFIGURED BY USER:

- RX_CH_FRAME_{n}, FRAME_{n}SAMPLES, FRAME{n}_REPETITION in FRAME_n_PARAMETERS_1 register.

- BTIA_GAIN_FRAME_{n}, TX_LEDSET_FRAME_l_{n} in FRAME_n_PARAMETERS_2 register.

- TX_LEDSET_FRAME_h_{n}, TX_LEDOFF_FRAME_l_{n} in FRAME_n_PARAMETERS_3 register.

- TX_LEDOFF_FRAME_h_{n} in FRAME_n_PARAMETERS_4 register.

- IDAC_DC_PPG_FRAME_{n} in FRAME_n_PARAMETERS_5 register.

- EN_TX_i in FRAME_n_PARAMETERS_6 register.

- CURR_SEL_TX_FRAME{n} in FRAME_n_PARAMETERS_7 register.

- TX1_OUTAB_SEL_FRAME_{n}, TX2_OUTAB_SEL_FRAME_{n}, TX3_OUTAB_SEL_FRAME_{n}, TX4_OUTAB_SEL_FRAME_{n} in FRAME_n_PARAMETERS_10 register.

Covers: SRS-REQ-145

[End]

Source paragraph: Section 5 PPG FRAMES CONFIG: (under Section 15.3.3 PPG Only), paragraph 026 (page 152)

[DRS-REQ-146] Requirement:

- The user shall write DEVICE_CONFIG_OPERATION register [0] at 1.

Covers: SRS-REQ-146

[End]

Source paragraph: Section 6 SELECT OPERATIVE MODE: (under Section 15.3.3 PPG Only), paragraph 008 (page 153)

[DRS-REQ-147] Requirement:

- To check data in unique FIFO (default config) the user shall check the first MSB. PPG DATA has 20 bits of data and 4 of TAG. The 4 bits, from 0 to 11, are used to identify the frame (as described in the table in DATA TAG table).

Covers: SRS-REQ-147

[End]

Source paragraph: Section 7 CHECK DATA: (under Section 15.3.3 PPG Only), paragraph 011 (page 153)

### 6.4.10 STBIO1 Register Map Peculiar Requirements

[DRS-REQ-001] Requirement:

Write enable for each OTP REGISTER shall be 0, in functional mode, if bit 31 of OTP_PRG10 is set 1.

Covers: SRS-REQ-001

[End]

Source paragraph: Section 13.4.2 STBIO1 Register Map Peculiar Requirements (under Section 13.4 STBIO1 Register Map), paragraph 023 (page 135)

### 6.4.11 Scan mode

[DRS-REQ-148] Requirement:

After the initial boot phase, only with SPI interface, to turn on the Scan mode shall be:

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable debug mode by writing to regmap (ADDRESS=0x3E, DATA=0x01) Figure 26 scan mode activation timing

Covers: SRS-REQ-148

[End]

Source paragraph: Section 18.1 Scan mode (under Section 18 Digital DFT), paragraph 011 (page 159)

## 10. Missing Inputs

- None