# System Requirements Specification

## STBIO

Author: SRS Gen Spec Agent

Date: 2026-09-01

Source specification: C:/Users/iannuzza/LOCAL_PROJ/Github_copilot_proj/STBIO_AI/specs/DDS_STBIO1.pdf

## 0. Document Navigation

### 0.1 Table of contents

Provide internal markdown links to all top-level sections.

- STBIO

- 0. Document Navigation

- 1. Introduction

- 1.1 Purpose

- 1.2 Scope

- 1.3 Intended audience

- 1.4 References

- 2. Definitions and terminology

- 2.1 System terminology

- 2.2 Analog terminology

- 2.3 Digital terminology

- 2.4 Measurement and acceptance terms

- 2.5 Category Convention

- 2.5.1 Comment

- 2.5.2 Definition

- 2.5.3 Assumption

- 2.5.4 Requirement

- 3. System-level overview

- 3.1 General system description

- 3.2 Operating modes

- 3.3 Power states

- 3.4 System interfaces

- 3.5 System constraints

- 3.6 System performance targets

- 4. Analog sub-system

- 4.1 Analog block list

- 4.2 Analog I/O characteristics

- 4.3 Sampling and conversion

- 4.4 Analog operating modes

- 4.5 Analog calibration

- 4.6 Analog performance

- 5. Digital sub-system

- 5.1 Digital block list

- 5.2 Control and register requirements

- 5.3 Mode control requirements

- 5.4 Communication requirements

- 5.5 Digital timing requirements

- 5.6 Digital data-path requirements

- 6. Cross-domain mixed-signal requirements

- 6.1 Analog-digital interactions

- 6.2 Transition behavior across domains

- 6.3 Clock and synchronization across domains

- 6.4 Power sequencing across domains

- 7. Validation and qualification requirements

- 7.1 Verification scope

- 7.2 Pass/fail criteria

- 7.3 GR&R and guard band

- 7.4 Validation configurations

- 8. Requirement identification and traceability

- 8.1 Numbering convention

- 8.2 Traceability contract

- 8.3 Residual requirement catalog by domain

- 8.3.1 ADSP Peculiar Requirements - Boot routine/operation

- 8.4 Source Function Context

- 8.4.1 BIST MODE

- 8.4.2 BOOT Phase

- 8.4.3 Configuration Phase

- 8.4.4 Debug mode

- 8.4.5 ECG Only

- 8.4.6 ECG and BIA

- 8.4.7 Other source function context

- 8.4.8 PAD Mux

- 8.4.9 PPG Only

- 8.4.10 STBIO1 Register Map Peculiar Requirements

- 8.4.11 Scan mode

- 9. Project-specific block sections

- 9.1 Sensor-Hub

- 9.2 ADC

- 9.3 ADSP

- 9.4 Main Controller

- 9.5 Smart FIFO

- 9.6 IRQ logic

- 9.7 PMU

- 9.8 I2C_SPI_AHB

- 9.9 ISPU

- 10. Missing Inputs

- Assumptions and TBD

### 0.2 Internal index for paragraphs and pages

Include section/paragraph navigation tables and block navigation tables with links.

#### Table 3. Section navigation index

| Section | Paragraph anchor | Internal link | Page (rendered PDF) |
| --- | --- | --- | --- |
| 0. Document Navigation | 0 | Jump | Auto |
| 1. Introduction | 1 | Jump | Auto |
| 1.1 Purpose | 1.1 | Jump | Auto |
| 1.2 Scope | 1.2 | Jump | Auto |
| 1.3 Intended audience | 1.3 | Jump | Auto |
| 1.4 References | 1.4 | Jump | Auto |
| 2. Definitions and terminology | 2 | Jump | Auto |
| 2.1 System terminology | 2.1 | Jump | Auto |
| 2.2 Analog terminology | 2.2 | Jump | Auto |
| 2.3 Digital terminology | 2.3 | Jump | Auto |
| 2.4 Measurement and acceptance terms | 2.4 | Jump | Auto |
| 2.5 Category Convention | 2.5 | Jump | Auto |
| 2.5.1 Comment | 2.5.1 | Jump | Auto |
| 2.5.2 Definition | 2.5.2 | Jump | Auto |
| 2.5.3 Assumption | 2.5.3 | Jump | Auto |
| 2.5.4 Requirement | 2.5.4 | Jump | Auto |
| 3. System-level overview | 3 | Jump | Auto |
| 3.1 General system description | 3.1 | Jump | Auto |
| 3.2 Operating modes | 3.2 | Jump | Auto |
| 3.3 Power states | 3.3 | Jump | Auto |
| 3.4 System interfaces | 3.4 | Jump | Auto |
| 3.5 System constraints | 3.5 | Jump | Auto |
| 3.6 System performance targets | 3.6 | Jump | Auto |
| 4. Analog sub-system | 4 | Jump | Auto |
| 4.1 Analog block list | 4.1 | Jump | Auto |
| 4.2 Analog I/O characteristics | 4.2 | Jump | Auto |
| 4.3 Sampling and conversion | 4.3 | Jump | Auto |
| 4.4 Analog operating modes | 4.4 | Jump | Auto |
| 4.5 Analog calibration | 4.5 | Jump | Auto |
| 4.6 Analog performance | 4.6 | Jump | Auto |
| 5. Digital sub-system | 5 | Jump | Auto |
| 5.1 Digital block list | 5.1 | Jump | Auto |
| 5.2 Control and register requirements | 5.2 | Jump | Auto |
| 5.3 Mode control requirements | 5.3 | Jump | Auto |
| 5.4 Communication requirements | 5.4 | Jump | Auto |
| 5.5 Digital timing requirements | 5.5 | Jump | Auto |
| 5.6 Digital data-path requirements | 5.6 | Jump | Auto |
| 6. Cross-domain mixed-signal requirements | 6 | Jump | Auto |
| 6.1 Analog-digital interactions | 6.1 | Jump | Auto |
| 6.2 Transition behavior across domains | 6.2 | Jump | Auto |
| 6.3 Clock and synchronization across domains | 6.3 | Jump | Auto |
| 6.4 Power sequencing across domains | 6.4 | Jump | Auto |
| 7. Validation and qualification requirements | 7 | Jump | Auto |
| 7.1 Verification scope | 7.1 | Jump | Auto |
| 7.2 Pass/fail criteria | 7.2 | Jump | Auto |
| 7.3 GR&R and guard band | 7.3 | Jump | Auto |
| 7.4 Validation configurations | 7.4 | Jump | Auto |
| 8. Requirement identification and traceability | 8 | Jump | Auto |
| 8.1 Numbering convention | 8.1 | Jump | Auto |
| 8.2 Traceability contract | 8.2 | Jump | Auto |
| 8.3 Residual requirement catalog by domain | 8.3 | Jump | Auto |
| 8.3.1 ADSP Peculiar Requirements - Boot routine/operation | 8.3.1 | Jump | Auto |
| 8.4 Source Function Context | 8.4 | Jump | Auto |
| 8.4.1 BIST MODE | 8.4.1 | Jump | Auto |
| 8.4.2 BOOT Phase | 8.4.2 | Jump | Auto |
| 8.4.3 Configuration Phase | 8.4.3 | Jump | Auto |
| 8.4.4 Debug mode | 8.4.4 | Jump | Auto |
| 8.4.5 ECG Only | 8.4.5 | Jump | Auto |
| 8.4.6 ECG and BIA | 8.4.6 | Jump | Auto |
| 8.4.7 Other source function context | 8.4.7 | Jump | Auto |
| 8.4.8 PAD Mux | 8.4.8 | Jump | Auto |
| 8.4.9 PPG Only | 8.4.9 | Jump | Auto |
| 8.4.10 STBIO1 Register Map Peculiar Requirements | 8.4.10 | Jump | Auto |
| 8.4.11 Scan mode | 8.4.11 | Jump | Auto |
| 9. Project-specific block sections | 9 | Jump | Auto |
| 9.1 Sensor-Hub | 9.1 | Jump | Auto |
| 9.2 ADC | 9.2 | Jump | Auto |
| 9.3 ADSP | 9.3 | Jump | Auto |
| 9.4 Main Controller | 9.4 | Jump | Auto |
| 9.5 Smart FIFO | 9.5 | Jump | Auto |
| 9.6 IRQ logic | 9.6 | Jump | Auto |
| 9.7 PMU | 9.7 | Jump | Auto |
| 9.8 I2C_SPI_AHB | 9.8 | Jump | Auto |
| 9.9 ISPU | 9.9 | Jump | Auto |
| 10. Missing Inputs | 10 | Jump | Auto |

#### Table 4. Block navigation index

| Block | Paragraph anchor | Internal link | Requirement IDs |
| --- | --- | --- | --- |
| Sensor-Hub | 9.1 | Jump | SRS-REQ-075 to SRS-REQ-097 |
| ADC | 9.2 | Jump | SRS-REQ-150 to SRS-REQ-152 |
| ADSP | 9.3 | Jump | SRS-REQ-047 to SRS-REQ-154 |
| Main Controller | 9.4 | Jump | SRS-REQ-120 |
| Smart FIFO | 9.5 | Jump | SRS-REQ-098 to SRS-REQ-109 |
| IRQ logic | 9.6 | Jump | SRS-REQ-110 to SRS-REQ-114 |
| PMU | 9.7 | Jump | SRS-REQ-002 to SRS-REQ-122 |
| I2C_SPI_AHB | 9.8 | Jump | SRS-REQ-067 to SRS-REQ-074 |
| ISPU | 9.9 | Jump | SRS-REQ-115 to SRS-REQ-119 |

#### Table 5. Requirement paragraph index by block

| Requirement group | Paragraph anchor | Internal link | Page (rendered PDF) |
| --- | --- | --- | --- |
| SRS-REQ-075 to SRS-REQ-097 | 9.1 | Sensor-Hub | Auto |
| SRS-REQ-150 to SRS-REQ-152 | 9.2 | ADC | Auto |
| SRS-REQ-047 to SRS-REQ-154 | 9.3 | ADSP | Auto |
| SRS-REQ-120 | 9.4 | Main Controller | Auto |
| SRS-REQ-098 to SRS-REQ-109 | 9.5 | Smart FIFO | Auto |
| SRS-REQ-110 to SRS-REQ-114 | 9.6 | IRQ logic | Auto |
| SRS-REQ-002 to SRS-REQ-122 | 9.7 | PMU | Auto |
| SRS-REQ-067 to SRS-REQ-074 | 9.8 | I2C_SPI_AHB | Auto |
| SRS-REQ-115 to SRS-REQ-119 | 9.9 | ISPU | Auto |

### 0.3 Document control

Include a version history table and a reference documents table.

#### Table 1. Version history

| Version | Date | Description | Author |
| --- | --- | --- | --- |
| 1.0 | 2026-09-01 | Initial SRS baseline generated from Stage 1 and Stage 2 artifacts | SRS Gen Spec Agent |

#### Table 2. Reference documents

| Doc name | Version | Author |
| --- | --- | --- |
| artifacts/stage1_requirements/requirements_summary.csv | Current workspace revision | Requirements Extraction Agent |
| artifacts/stage2_mirco_arc/micro_architecture_report.md | Current workspace revision | Micro-Architectural Analysis Agent |
| artifacts/stage2_mirco_arc/requirement_to_block_traceability.csv | Current workspace revision | Micro-Architectural Analysis Agent |
| artifacts/stage2_mirco_arc/block_inventory.csv | Current workspace revision | Micro-Architectural Analysis Agent |
| artifacts/stage2_mirco_arc/interface_catalog.csv | Current workspace revision | Micro-Architectural Analysis Agent |
| artifacts/stage2_mirco_arc/interaction_matrix.csv | Current workspace revision | Micro-Architectural Analysis Agent |
| templates/SRS_gen_AI_template_prompt.md | Current workspace revision | Project template maintainers |

#### Table 6. Category convention

| Category | Naming rule / prefix | Scope | Notes / example |
| --- | --- | --- | --- |
| System authored requirement | SRS-REQ-xxx | Project-specific SRS atomic entries | Covers one upstream requirement per row |
| Source system requirement | SYS-RQ-xxx or source_req_id | Upstream Stage 1 requirement catalog | Referenced through Covers linkage |
| Source analog requirement | ANA-RQ-xxx or source_req_id | Upstream Stage 1 requirement catalog | Referenced through Covers linkage |
| Source digital requirement | DIG-RQ-xxx or source_req_id | Upstream Stage 1 requirement catalog | Referenced through Covers linkage |
| Cross-domain requirement | XDN-RQ-xxx or source_req_id | Upstream Stage 1 requirement catalog | Referenced through Covers linkage |

### 0.4 Table of tables

List all tables with links.

| Table | Title | Link |
| --- | --- | --- |
| Table 1 | Section navigation index | Go to Table 1 |
| Table 2 | Block navigation index | Go to Table 2 |
| Table 3 | Requirement paragraph index by block | Go to Table 3 |
| Table 4 | Version history | Go to Table 4 |
| Table 5 | Reference documents | Go to Table 5 |
| Table 6 | Category convention | Go to Table 6 |

## 1. Introduction

### 1.1 Purpose

Define the purpose of the SRS and device scope.

### 1.2 Scope

Cover:

- system-level functionality

- analog subsystem behavior

- digital subsystem behavior

- interfaces and configuration model

- power, timing, performance constraints

- validation expectations

### 1.3 Intended audience

List relevant stakeholders (architecture, design, verification, test, product, customer).

### 1.4 References

List project artifacts and external standards.

## 2. Definitions and terminology

### 2.1 System terminology

Define project-specific system terms and abbreviations.

### 2.2 Analog terminology

Define analog terms (input/output range, gain, offset, noise, bandwidth, full-scale, ODR, sensitivity).

### 2.3 Digital terminology

Define digital terms (register, bit field, mode, state, reset, interrupt, boot sequence, interface).

### 2.4 Measurement and acceptance terms

Define terms such as RMS noise, SNR, latency, precision, tolerance, GR&R, guard band, pass/fail.

### 2.5 Category Convention

#### 2.5.1 Comment

This category denotes that the content of the object text is a general comment. For example, this may be an explanation why the requirement demands certain items when there would also be other possibilities. A comment should not be necessary to understand the related requirements

#### 2.5.2 Definition

This category is used for the definition of terms, wordings, technical expressions etc. It is a documentation of design decisions and general instructions that belong to the document itself, e.g. owner of the document, structure of the document. It is needed to understand a related requirement. It is not linked to any test case.

#### 2.5.3 Assumption

This category is used for requirements this document provide upstream to another document, indicating what this IP/block/system needs to work properly.

#### 2.5.4 Requirement

This category denotes a requirement that has to be implemented and verified. Accordingly, it is necessary to establish traces from the different requirement levels to the test cases for this category.

## 3. System-level overview

### 3.1 General system description

The system architecture is derived from current Stage 1 and Stage 2A artifacts.

The main system functionalities are:

- Signal acquisition (sensor-path sensing; ADC conversion)

- Digital sample control (sample-rate control; data update policy; digital status handling)

- FIFO and interrupt signaling (FIFO buffering modes; watermark/overrun detection; INT1/INT2 signaling)

- SPI host register access path (SPI timing-compliant transfer; register read/write)

- I2C host register access path (I2C slave acknowledge protocol; register read/write; auto-increment)

- Power, clock, and startup behavior (power-domain supervision; low-power transitions; readiness control)

### 3.2 Operating modes

For each mode define:

- active blocks

- disabled blocks

- power target

- latency and throughput behavior

- register accessibility

- allowed transitions

Operating mode behavior shall be controlled by configuration and verified in functional tests.

### 3.3 Power states

Define power-up, normal, low-power, standby, shutdown, and debug/factory behavior.

### 3.4 System interfaces

Define supply, reset, clock, host communication, interrupt, and test interfaces.

The architecture exposes structured interfaces including:

- SENSOR_ANALOG_INPUT (input, analog, owner=Sensor-Hub)

- ADC_DATA (output, digital, owner=ADC)

- FIFO_DATA (output, digital, owner=Smart FIFO)

- SERIAL_CLK (input, digital, owner=SPI interface)

- SERIAL_SELECT (input, digital, owner=SPI interface)

- SERIAL_DATA_IN (input, digital, owner=SPI interface)

- SERIAL_DATA_OUT (output, digital, owner=SPI interface)

- ALT_SERIAL_CLK (input, digital, owner=I2C interface)

- ALT_SERIAL_DATA (bidirectional, digital, owner=I2C interface)

- INT1 (output, digital, owner=IRQ logic)

### 3.5 System constraints

Define environmental, electrical, timing, packaging, reliability, and safety constraints.

### 3.6 System performance targets

Define measurable system KPIs with units and acceptance limits.

## 4. Analog sub-system

### 4.1 Analog block list

- ADC

### 4.2 Analog I/O characteristics

Define ranges, linearity, noise, bandwidth, saturation, settling, and constraints per block.

### 4.3 Sampling and conversion

Define ODR, conversion latency, synchronization, filtering, and resolution expectations.

### 4.4 Analog operating modes

Define normal, low-power, high-performance, calibration, test, and shutdown behavior.

### 4.5 Analog calibration

Define offset/gain calibration, compensation, and self-test behavior if applicable.

### 4.6 Analog performance

Define noise, distortion, sensitivity, drift, crosstalk, and PSRR/CMRR requirements as applicable.

## 5. Digital sub-system

### 5.1 Digital block list

- Sensor-Hub

- ADSP

- Main Controller

- Smart FIFO

- Regmap

- SPI interface

- I2C interface

- IRQ logic

- PMU

- BIST Controller

- I2C_SPI_AHB

- Unassigned

- ISPU

- ISPU debug

### 5.2 Control and register requirements

Define register model, reset defaults, access policy, reserved bit behavior, and boot defaults.

### 5.3 Mode control requirements

Define mode select/switch/retention behavior, transitions, and debug access constraints.

### 5.4 Communication requirements

Define protocol-level behavior for applicable interfaces (for example UART, I2C, SPI).

### 5.5 Digital timing requirements

Define clocking, setup/hold, propagation, response, interrupt latency, and synchronization.

### 5.6 Digital data-path requirements

Define data format, alignment, buffering/FIFO, overflow/underflow, and framing rules.

## 6. Cross-domain mixed-signal requirements

### 6.1 Analog-digital interactions

Define enable/disable controls, measurement triggers, update timing, data-ready signaling, and calibration control.

Cross-domain orchestration shall be implemented through interaction paths:

- ADC -> Main Controller via digital samples (trigger: sample cycle)

- Main Controller -> Smart FIFO via sample stream and FIFO mode control (trigger: FIFO enabled)

- Smart FIFO -> IRQ logic via watermark/overrun/empty flags (trigger: FIFO status change)

- Regmap -> Main Controller via configuration writes (trigger: register update)

- SPI interface -> Regmap via SPI register transactions (trigger: host SPI access)

- I2C interface -> Regmap via I2C register transactions (trigger: host I2C access)

- Regmap -> SPI interface via register read response (trigger: SPI read)

- Regmap -> I2C interface via register read response (trigger: I2C read)

### Connection Matrix Requirements

Each populated source-to-destination connection-matrix cell shall be implemented as follows:

[SRS-REQ-155] Requirement:

The Main Controller block shall be connected to the Regmap block.

Covers: DDS_STBIO1_0201

[End]

[SRS-REQ-156] Requirement:

The Main Controller block shall be connected to the Smart FIFO block.

Covers: DDS_STBIO1_0202

[End]

[SRS-REQ-157] Requirement:

The Main Controller block shall be connected to the ADSP block.

Covers: DDS_STBIO1_0219

[End]

[SRS-REQ-158] Requirement:

The ADSP block shall be connected to the Regmap block.

Covers: DDS_STBIO1_0203

[End]

[SRS-REQ-159] Requirement:

The ADSP block shall be connected to the Sensor-Hub block.

Covers: DDS_STBIO1_0204

[End]

[SRS-REQ-160] Requirement:

The ADSP block shall be connected to the Smart FIFO block.

Covers: DDS_STBIO1_0206

[End]

[SRS-REQ-161] Requirement:

The I2C_SPI_AHB block shall be connected to the Regmap block.

Covers: DDS_STBIO1_0207

[End]

[SRS-REQ-162] Requirement:

The I2C_SPI_AHB block shall be connected to the Sensor-Hub block.

Covers: DDS_STBIO1_0208

[End]

[SRS-REQ-163] Requirement:

The I2C_SPI_AHB block shall be connected to the ISPU block.

Covers: DDS_STBIO1_0209

[End]

[SRS-REQ-164] Requirement:

The I2C_SPI_AHB block shall be connected to the Smart FIFO block.

Covers: DDS_STBIO1_0210

[End]

[SRS-REQ-165] Requirement:

The I2C_SPI_AHB block shall be connected to the ADSP block.

Covers: DDS_STBIO1_0221

[End]

[SRS-REQ-166] Requirement:

The Sensor-Hub block shall be connected to the Smart FIFO block.

Covers: DDS_STBIO1_0211

[End]

[SRS-REQ-167] Requirement:

The ISPU block shall be connected to the Regmap block.

Covers: DDS_STBIO1_0212

[End]

[SRS-REQ-168] Requirement:

The ISPU block shall be connected to the Sensor-Hub block.

Covers: DDS_STBIO1_0213

[End]

[SRS-REQ-169] Requirement:

The ISPU block shall be connected to the Smart FIFO block.

Covers: DDS_STBIO1_0215

[End]

[SRS-REQ-170] Requirement:

The ISPU block shall be connected to the ADSP block.

Covers: DDS_STBIO1_0222

[End]

[SRS-REQ-171] Requirement:

The ISPU debug block shall be connected to the Regmap block.

Covers: DDS_STBIO1_0216

[End]

[SRS-REQ-172] Requirement:

The ISPU debug block shall be connected to the Sensor-Hub block.

Covers: DDS_STBIO1_0217

[End]

[SRS-REQ-173] Requirement:

The ISPU debug block shall be connected to the ISPU block.

Covers: DDS_STBIO1_0218

[End]

[SRS-REQ-174] Requirement:

The ISPU debug block shall be connected to the Smart FIFO block.

Covers: DDS_STBIO1_0220

[End]

[SRS-REQ-175] Requirement:

The ISPU debug block shall be connected to the ADSP block.

Covers: DDS_STBIO1_0223

[End]

### 6.2 Transition behavior across domains

Define data retention/flush behavior and accessibility during mode transitions.

### 6.3 Clock and synchronization across domains

Define source dependencies, sample alignment, timestamping, and reset synchronization.

The PMU is the authoritative clock and reset controller for the digital wrapper. The following summary is derived from the PMU Clocks and Reset connection tables and PMU startup requirements.

#### Clock distribution summary

- The top-level 16 MHz and 64 kHz clocks feed u_pmu.clk_16m and u_pmu.clk_64k without clock gating (DDS_STBIO1_1020 and DDS_STBIO1_1021).

- The PMU supplies an ungated 32 kHz test clock to the Main Controller i_clk_32kHz and exports the 32 kHz test clock at the digital-wrapper boundary (DDS_STBIO1_1027 and DDS_STBIO1_1025).

- PMU 16 MHz clock paths serve ADSP stmc_clk, Main Controller sys_clk and i_clk_16MHz, AFE Xbar hclk, FIFO HCLK, ISPU clk, and Regmap/OTP hclk (DDS_STBIO1_1022 through DDS_STBIO1_1026, DDS_STBIO1_2024 through DDS_STBIO1_2026).

- The ADSP, Main Controller, AFE Xbar, FIFO, Regmap/OTP, and 16 MHz test-clock paths use the specified PMU enable/test/scan controls; the ISPU clock and the primary PMU input clocks are power-down controlled or ungated as specified in the clock recap table.

#### Reset synchronization summary

- PMU reset outputs synchronize reset release to the receiving clock domains: resetn_stmc to ADSP HRESETn, resetn_masterblaze to Main Controller HRESETn, resetn_regmap to Regmap/OTP hresetn and ISPU main_rst_n, resetn_fifo to Multi-Channel FIFO HRESETn, and the device-startup POR-main-reset path to AFE Xbar hresetn (DDS_STBIO1_1050 through DDS_STBIO1_1053, DDS_STBIO1_2052, and DDS_STBIO1_2053).

- When i_POR1V2_1V2 is low, the PMU keeps LDO1V8 disabled, powers down the clock function, deasserts 16 MHz ready, and disables the RC voltage reference. After POR rises, LDO1V8 and the divided 32 kHz clock become stable after at least 10 cycles of the 64 kHz input; the RC reference is enabled after at least 352 divided-clock cycles (DDS_STBIO1_0104 through DDS_STBIO1_0110).

- After i_START_CLK_16M asserts, o_CLK_16M_READY asserts after at least 20 cycles of the 16 MHz clock. On its first assertion, o_main_resetn is released after the same minimum 20-cycle delay, preventing downstream logic from leaving reset before the 16 MHz clock is ready (DDS_STBIO1_0112 and DDS_STBIO1_0113).

### 6.4 Power sequencing across domains

Define startup ordering, brownout handling, reset dependencies, and shutdown ordering.

Power sequencing is controlled by the PMU and is anchored to the i_POR1V2_1V2 power-on-reset input. The following sequence summarizes the source requirements.

#### PMU startup and power-down sequence

- The device-on condition requires i_POR1V2_1V2 high. While it is low, the PMU shall keep o_EN_LDO1V8 low, o_PD_CLKF high, o_CLK_16M_READY low, and o_DIS_RC_VIREF high, establishing the power-down safe state (DDS_STBIO1_0104 through DDS_STBIO1_0108).

- After i_POR1V2_1V2 rises, the PMU waits at least 10 cycles of the 64 kHz input before asserting o_EN_LDO1V8 and declaring the divided 32 kHz clock stable (DDS_STBIO1_0109).

- Once the divided 32 kHz clock is stable, the PMU waits at least 352 divided-clock cycles before deasserting o_DIS_RC_VIREF; this enables the RC voltage-reference path only after the low-frequency clock has stabilized (DDS_STBIO1_0110).

- When i_mask_en_ldo_pdclkf_reg and o_EN_LDO1V8 are both asserted, the PMU deasserts o_PD_CLKF after at least 40 divided-clock cycles. A subsequent i_START_CLK_16M assertion requires at least 20 cycles of the 16 MHz clock before o_CLK_16M_READY and, on first startup, o_main_resetn are asserted (DDS_STBIO1_0111 through DDS_STBIO1_0113).

- At release of the analog-domain POR signal, the PMU manages LDO1V8 turn-on and the clock startup sequence reported by PMU IPOS (DDS_STBIO1_0114).

#### ISPU power-island sequence

- After initial boot, ISPU bring-up selects Regmap page 3, enables LDO1V8 through VIREF_LDO1V8_CLK16M at address 0x0F with data 0x02, enables the clock through the same register with data 0x12, enables user mode at address 0x5E with data 0x01, and enables the ISPU power island by writing 0x01 to the POWER CONFIGURATION REGISTER at AHB address 0xCA000 (DDS_STBIO1_0507).

- ISPU shutdown disables the power island by writing 0x00 to the POWER CONFIGURATION REGISTER at AHB address 0xCA000 (DDS_STBIO1_0508).

## 7. Validation and qualification requirements

### 7.1 Verification scope

Define simulation, emulation, silicon validation, characterization, final test, qualification scope.

### 7.2 Pass/fail criteria

Define thresholds, tolerance bands, confidence/population criteria, and acceptance rules.

### 7.3 GR&R and guard band

Define GR&R expectations, uncertainty handling, and guard-banding approach.

### 7.4 Validation configurations

Define approved modes/settings/conditions for validation execution.

## 8. Requirement identification and traceability

### 8.1 Numbering convention

Use consistent IDs, for example:

- SYS-RQ-001

- ANA-RQ-001

- DIG-RQ-001

- XDN-RQ-001

- SRS-REQ-001 (for newly authored SRS requirements that include Covers links)

### 8.2 Traceability contract

For each requirement include:

- ID

- statement

- rationale

- source

- verification method

- acceptance criteria

- owning block(s)

### 8.3 Residual requirement catalog by domain

List only residual requirements not already represented in project-specific atomic sub-block requirement entries.

#### Dedicated source sections

##### 8.3.1 ADSP Peculiar Requirements - Boot routine/operation

[SRS-REQ-047] Requirement:

The BOOT routine shall starts when the OTP_BOOT bit rises and shall completes when quokka_boot_end rises. [Vpriority High]

Covers: DDS_STBIO1_0014

[End]

[SRS-REQ-048] Requirement:

During OTP boot operation, the ADSP shall copy the content of the OTP memory into the OTP registers, following the procedure to access OTP memory [Vpriority High]

Covers: DDS_STBIO1_0015

[End]

[SRS-REQ-049] Requirement:

During BOOT routine, ADSP shall first read the last location of the OTP memory, at address 0x7F. If the content is 0xFF, called “lifecycle” byte, then the READ operation proceeds from address 0 to address 0x2F; otherwise, ADSP shall not perform any further reads.

Covers: DDS_STBIO1_0016

[End]

[SRS-REQ-050] Requirement:

To perform a BOOT routine, OTP_BOOT bit shall be written when quokka_run is low. [Vpriority High]

Covers: DDS_STBIO1_0017

[End]

[SRS-REQ-064] Requirement:

After each Boot routine performed by the ADSP, a value of 0x04 shall be written once to address 0x0CA00C to ensure correct operation of the AHB transactions. [Vpriority High]

Covers: DDS_STBIO1_0032

[End]

[SRS-REQ-066] Requirement:

If during the start of a new operation, more than one bit is written into CONTROL_A reg in the same write operation, ADSP shall execute only the routine at higher priority and discard the others. Priority is listed below, with first routine at higher priority: 1. SOFT RESET routine 2. OTP WRITE routine 3. OTP TEST routine 4. OTP BOOT routine [Vpriority High]

Covers: DDS_STBIO1_0035

[End]

#### System requirements

- No residual unmapped requirements for this domain (mapped items are captured in project-specific sub-block requirement paragraphs).

#### Analog requirements

- No residual unmapped requirements for this domain (mapped items are captured in project-specific sub-block requirement paragraphs).

#### Digital requirements

- No residual unmapped requirements for this domain (mapped items are captured in project-specific sub-block requirement paragraphs).

#### Cross-domain requirements

- No residual unmapped requirements for this domain (mapped items are captured in project-specific sub-block requirement paragraphs).

#### Use case, user specific

- None

## 9. Project-specific block sections

### 9.1 Sensor-Hub

General functional description: Operate the Sensor Hub I2C master for external targets, including trigger/control, pad enable, data collection, and FIFO multi-mode coordination.

Inputs: Primary sensing input Outputs: Analog input signal

Interface catalog evidence:

| Interface | Direction | Type | Owner | Purpose |
| --- | --- | --- | --- | --- |
| SENSOR_ANALOG_INPUT | input | analog | Sensor-Hub | Raw sensor-domain analog input |

[SRS-REQ-075] Requirement:

The Sensor-Hub block shall implement: If the sensor hub IP is not enabled, the SCL and SDA pad shall not toggle. [Vpriority High] [TO: IPOS_Senshub_MB_i2c].

Covers: DDS_STBIO1_9317

[End]

[SRS-REQ-076] Requirement:

The Sensor-Hub block shall implement: Output scl_master_out of sensor hub shall be connected to enable of output buffer of sensor hub’s SCL PAD in case it is a bi-directional PAD). [Vpriority High] [TO: IPOS_Senshub_MB_i2c].

Covers: DDS_STBIO1_9013

[End]

[SRS-REQ-077] Requirement:

The Sensor-Hub block shall implement: Output sda_master_out of sensor hub shall be connected to enable of output buffer of sensor hub’s SDA PAD. [Vpriority High] [TO: IPOS_Senshub_MB_i2c].

Covers: DDS_STBIO1_9014

[End]

[SRS-REQ-078] Requirement:

The Sensor-Hub block shall implement: Input scl_master_in of sensor hub shall be connected to input buffer from sensor hub’s SCL PAD. [Vpriority High] [TO: IPOS_Senshub_MB_i2c].

Covers: DDS_STBIO1_9011

[End]

[SRS-REQ-079] Requirement:

The Sensor-Hub block shall implement: Input sda_master_in of sensor hub shall be connected to input buffer from sensor hub’s SDA PAD. [Vpriority High] [TO: IPOS_Senshub_MB_i2c].

Covers: DDS_STBIO1_9012

[End]

[SRS-REQ-080] Requirement:

The Sensor-Hub block shall implement: The output port sleep_master shall be set to 1 when an I2C operation is not on-going. [Vpriority High] [TO: IPOS_Senshub_MB_i2c].

Covers: DDS_STBIO1_9017

[End]

[SRS-REQ-081] Requirement:

The Sensor-Hub block shall implement: Input sys_clk of sensor hub shall be connected to 8MHz clock to generate I2C communications compliant with FM+ I2C standard. [TO: IPOS_Senshub_MB_i2c].

Covers: DDS_STBIO1_9004

[End]

[SRS-REQ-082] Requirement:

The Sensor-Hub block shall implement: Requirement The trigger of sensor hub shall be possible in both ways, HW via dvalid_masterblaze_ext signal or SW using DVALID_REGISTER.

Covers: DDS_STBIO1_9018

[End]

[SRS-REQ-083] Requirement:

The Sensor-Hub block shall implement: Requirement Only I2C target addressed through SLAVE0_DEV_ADD register shall be the destination of an I2C Write Operation.

Covers: DDS_STBIO1_9024

[End]

[SRS-REQ-084] Requirement:

The Sensor-Hub block shall implement: Requirement The bit 0 of SLAVE1_DEV_ADD, SLAVE2_DEV_ADD, SLAVE3_DEV_ADD registers shall be ignored: ONLY read operations towards I2C target 1, 2 and 3 shall be performed.

Covers: DDS_STBIO1_9027

[End]

[SRS-REQ-085] Requirement:

The Sensor-Hub block shall implement: Requirement SLAVEx_NUMOP bit-fields of SLAVEx_CONFIG registers shall define how many bytes per slave are read as per effect of an I2C operation (x from 1 to 12).

Covers: DDS_STBIO1_9030

[End]

[SRS-REQ-086] Requirement:

The Sensor-Hub block shall implement: Requirement SLAVE0_NUMOP bitfield shall be ignored if an I2C Write operation is issued to the I2C Slave 0.

Covers: DDS_STBIO1_9032

[End]

[SRS-REQ-087] Requirement:

The Sensor-Hub block shall implement: AUX_SENS_ON possible values shall be 0,1,2,3. All the values shall be tested since MasterBlaze supports up-to 4 I2C targets connected to I2C network. [TO: IPOS_Senshub_MB_i2c].

Covers: DDS_STBIO1_9029

[End]

[SRS-REQ-088] Requirement:

The Sensor-Hub block shall implement: All the values from 1 to 12 shall be valid for SLAVEx_NUMOP bit-fields of SLAVEx_CONFIG registers. [TO: IPOS_Senshub_MB_i2c].

Covers: DDS_STBIO1_9031

[End]

[SRS-REQ-089] Requirement:

The Sensor-Hub block shall implement: All the SENSOR_HUB_x registers shall be reset every time a new I2C operation starts. [TO: IPOS_Senshub_MB_i2c].

Covers: DDS_STBIO1_9033

[End]

[SRS-REQ-090] Requirement:

The Sensor-Hub block shall implement: When Quokka MasterBlaze µP runs (sleep_master signal = 0), any dvalid triggered (HW or SW) shall be ignored. [TO: IPOS_Senshub_MB_i2c].

Covers: DDS_STBIO1_9035

[End]

[SRS-REQ-091] Requirement:

The Sensor-Hub block shall implement: Requirement The AHB Write to registers has no effect if the IP masterblaze processor is busy (I2C operation on-going); the registers value shall not be updated.

Covers: DDS_STBIO1_9300

[End]

[SRS-REQ-092] Requirement:

The Sensor-Hub block shall implement: Requirement In case also other sources that writes into FIFO are active, then sensor hub shall work always in multi-fifo mode fifo_mode input of sensor hub set to ‘1’.

Covers: DDS_STBIO1_9500

[End]

[SRS-REQ-093] Requirement:

The Sensor-Hub block shall implement: The NACK_REGISTER register is reset every time a new I2C operation starts. [TO: IPOS_Senshub_MB_i2c].

Covers: DDS_STBIO1_9034

[End]

[SRS-REQ-094] Requirement:

The Sensor-Hub block shall implement: Each data stored in FIFO shall be composed by 4 words (16bytes). [TO: IPOS_Senshub_MB_i2c].

Covers: DDS_STBIO1_9260

[End]

[SRS-REQ-095] Requirement:

The Sensor-Hub block shall implement: 3 words (12bytes) of the data stored in FIFO shall contain the data from each I2C slave. [TO: IPOS_Senshub_MB_i2c].

Covers: DDS_STBIO1_9261

[End]

[SRS-REQ-096] Requirement:

The Sensor-Hub block shall implement: 24-bits of the data stored in FIFO shall be reserved at ‘0’. [TO: IPOS_Senshub_MB_i2c].

Covers: DDS_STBIO1_9262

[End]

[SRS-REQ-097] Requirement:

The Sensor-Hub block shall implement: The last 8-bits of the data stored in FIFO shall contain the TAG field. [TO: IPOS_Senshub_MB_i2c].

Covers: DDS_STBIO1_9263

[End]

### 9.2 ADC

General functional description: Convert configured bio/optical channel inputs to samples, with register-controlled sampling periods and ADC test/debug modes.

Inputs: Conditioned analog signal Outputs: Digital samples

Interface catalog evidence:

| Interface | Direction | Type | Owner | Purpose |
| --- | --- | --- | --- | --- |
| ADC_DATA | output | digital | ADC | Digitized samples |

[SRS-REQ-150] Requirement:

The ADC block shall implement: To turn on the ADC TEST LOW NOISE mode shall be:.

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0xF, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable ADC in debug mode (ADDRESS=0x09, DATA=0x06)

- Select page 2 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable ADC TEST LOW NOISE and input of ADC from GPIO (ADDRESS=0x0A, DATA=0x14)

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable debug mode by writing to regmap (ADDRESS=0x3E, DATA=0x08)

Covers: DDS_STBIO1_0701

[End]

[SRS-REQ-151] Requirement:

The ADC block shall implement: To turn on the ADC TEST LOW NOISE mode shall be:.

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0xF, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable ADC in debug mode (ADDRESS=0x09, DATA=0x06)

- Select page 2 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable ADC TEST LOW NOISE and input of ADC from GPIO (ADDRESS=0x0A, DATA=0x14)

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable debug mode by writing to regmap (ADDRESS=0x3E, DATA=0x08)

Covers: DDS_STBIO1_0702

[End]

[SRS-REQ-152] Requirement:

The ADC block shall implement: To turn on the ADC TEST LOW NOISE mode shall be:.

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0xF, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable ADC in debug mode (ADDRESS=0x09, DATA=0x06)

- Select page 2 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable ADC TEST LOW NOISE and input of ADC from GPIO (ADDRESS=0x0A, DATA=0x0C)

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable debug mode by writing to regmap (ADDRESS=0x3E, DATA=0x08)

Covers: DDS_STBIO1_1507

[End]

### 9.3 ADSP

General functional description: Execute ADSP firmware for OTP boot/write/test and signal elaboration, using mapped program/data/register memory and soft-reset handling.

Inputs: Acquired samples and configuration data Outputs: Elaborated signal data and metadata

[SRS-REQ-051] Requirement:

The ADSP block shall implement: The WRITE routine shall start when OTP Write Bit rises and completes when quokka_run falls. [Vpriority High].

Covers: DDS_STBIO1_0018

[End]

[SRS-REQ-052] Requirement:

The ADSP block shall implement: During WRITE routine ADSP shall copy the content of the OTP regmap into the OTP memory, following the procedure to burn the fuses as expressed in the OTP datasheet [Vpriority High].

Covers: DDS_STBIO1_0019

[End]

[SRS-REQ-053] Requirement:

The ADSP block shall implement: During WRITE routine, ADSP shall allow copying all OTP Registers into the OTP Memory only if the location 0x24 of OTP Memory (OTP[24] in the following diagram) is not set to 0xFF.

Covers: DDS_STBIO1_0020

[End]

[SRS-REQ-054] Requirement:

The ADSP block shall implement: During the WRITE routine and after copying all OTP Registers into the OTP Memory, ADSP shall compare each value inside the OTP memory with the corresponding value in OTP Registers to check if the WRITE routine has been succesfull. In that case ADSP shall write 0xFF into the last location of the OTP memory.

Covers: DDS_STBIO1_0021

[End]

[SRS-REQ-055] Requirement:

The ADSP block shall implement: OTP_WRITE, OTP_BOOT and OTP_TEST bits shall be cleared by ADSP at the end of their respective operation [Vpriority High].

Covers: DDS_STBIO1_0022

[End]

[SRS-REQ-056] Requirement:

The ADSP block shall implement: A new operation shall start only when quokka_run is low, and one between OTP_TEST bit, OTP_WRITE bit or OTP_BOOT bit is written to 1 [Vpriority High].

Covers: DDS_STBIO1_0023

[End]

[SRS-REQ-057] Requirement:

The ADSP block shall implement: The TEST routines shall allow reading an 8-bit word on OTP of a specific address written in OTP_A register, and write this value in OTP_D_TEST register.

Covers: DDS_STBIO1_0028

[End]

[SRS-REQ-058] Requirement:

The ADSP block shall implement: OTP_IREF_WAKEUP shall be set to one by an external I2C/SPI to AHB write before initiating a new OTP_TEST or OTP_WRITE operation [Vpriority High].

Covers: DDS_STBIO1_0024

[End]

[SRS-REQ-059] Requirement:

The ADSP block shall implement: OTP_IREF_WAKEUP shall be setted to zero by an external I2C/SPI to AHB write at the end of OTP_TEST or OTP_WRITE operation [Vpriority High].

Covers: DDS_STBIO1_0025

[End]

[SRS-REQ-060] Requirement:

The ADSP block shall implement: After startup phase, once started with an operation, the IP shall ignore any further start incoming transactions. [Vpriority High].

Covers: DDS_STBIO1_0026

[End]

[SRS-REQ-061] Requirement:

The ADSP block shall implement: The TEST routine shall start when OTP Test Bit rises and completes when quokka_run falls. [Vpriority High].

Covers: DDS_STBIO1_0027

[End]

[SRS-REQ-062] Requirement:

The ADSP block shall implement: During the Soft Reset routine, ADSP shall write the ADSP RAM with the value 0. Only the first 3.5Kbyte of ADSP RAM shall be cleared, from address 0x000 to address 0xDFF [Vpriority High].

Covers: DDS_STBIO1_0034

[End]

[SRS-REQ-065] Requirement:

The ADSP block shall implement: The Soft Reset routine shall start when SOFT_RESET Bit (in CONTROL_A register) rises and completes when quokka_run falls. [Vpriority High].

Covers: DDS_STBIO1_0033

[End]

[SRS-REQ-154] Requirement:

The ADSP block shall implement: To move OTP signal, ADSP shall write a new value in a regbank by AHB protocol.

Covers: DDS_STBIO1_0013

[End]

### 9.4 Main Controller

General functional description: Generate and receive Analog Domain control signals, perform DSP elaboration on ADC outputs, and write elaborated results to the defined addresses.

Inputs: Configuration register settings, sampled data Outputs: Control state and data routing

[SRS-REQ-120] Requirement:

The Main Controller block shall implement: To perform a SOFT Reset procedure, the user shall perform the following steps: 1. If Mode Operation has been set to 1, shall be reset to 0 2. Set the Soft Reset Main Controller Bit (SW_RESETN_REG[0]) to 0 for at least 1 ms 3. Set the Soft Reset Main Controller Bit (SW_RESETN_REG[0]) to 1 4. Wait for the DEVICE_STATUS_REGISTER to reach the IDLE Value.

Covers: DDS_STBIO1_1114

[End]

### 9.5 Smart FIFO

General functional description: Provide FIFO storage and AHB/memory access in mutually exclusive Unique or Multiple modes, including sub-FIFO depth/addressing and tagged Sensor Hub data formatting.

Inputs: Digital samples, FIFO mode configuration Outputs: Buffered samples and FIFO status flags

Interface catalog evidence:

| Interface | Direction | Type | Owner | Purpose |
| --- | --- | --- | --- | --- |
| FIFO_DATA | output | digital | Smart FIFO | Buffered output samples |

[SRS-REQ-098] Requirement:

The Smart FIFO block shall implement: The parameter MEM_LOC shall define the first Memory Location of the FIFO RAM: 0xC4000. [TO: IPOS_FIFO_CTRL_00000102].

Covers: DDS_STBIO1_00000102

[End]

[SRS-REQ-099] Requirement:

The Smart FIFO block shall implement: The FIFO register map first memory location shall be defined as MEM_LOC + 0x1000: 0xC5000. [TO: IPOS_FIFO_CTRL_00000202].

Covers: DDS_STBIO1_00000202

[End]

[SRS-REQ-100] Requirement:

The Smart FIFO block shall implement: The FIFO Controller shall be able to address 1.5 kbyte divided into 32-bit address. [TO: IPOS_FIFO_CTRL_89456557].

Covers: DDS_STBIO1_89456557

[End]

[SRS-REQ-101] Requirement:

The Smart FIFO block shall implement: The FIFO Controller shall work at 16 MHz clock frequency. [TO: IPOS_FIFO_CTRL_89459082].

Covers: DDS_STBIO1_89459082

[End]

[SRS-REQ-102] Requirement:

The Smart FIFO block shall implement: The FIFO Controller shall have an AHB Slave Interface connected to the AHB BUS and a Memory Master Interface connected to the Physical Memory. [TO: IPOS_FIFO_CTRL_89455261].

Covers: DDS_STBIO1_89455261

[End]

[SRS-REQ-103] Requirement:

The Smart FIFO block shall implement: The FIFO Controller shall have two working modes: Multiple FIFO and Unique FIFO. These two modes are mutually exclusive. [TO: IPOS_FIFO_CTRL_89455449].

Covers: DDS_STBIO1_89455449

[End]

[SRS-REQ-104] Requirement:

The Smart FIFO block shall implement: As default, the FIFO shall work in Unique FIFO Mode. [TO: IPOS_FIFO_CTRL_89455232].

Covers: DDS_STBIO1_89455232

[End]

[SRS-REQ-105] Requirement:

The Smart FIFO block shall implement: The user shall select the Multiple FIFO mode setting to 1 the first bit into the FIFO_CFG_MODE and 0 for the Unique FIFO mode. [TO: IPOS_FIFO_CTRL_89456975].

Covers: DDS_STBIO1_89456975

[End]

[SRS-REQ-106] Requirement:

The Smart FIFO block shall implement: The FIFO Controller shall have a 6 i_depth input of 8 bit to be used in Multiple FIFO Mode to define the depth of each subFIFOs. [TO: IPOS_FIFO_CTRL_89457435].

Covers: DDS_STBIO1_89457435

[End]

[SRS-REQ-107] Requirement:

The Smart FIFO block shall implement: The FIFO 6 memory locations to access each sub-FIFO in Multiple mode are obtained adding multiples of 256 to the memory base address. Example: FIFO Memory Base Address = 0xC4000, then: FIFO 0 memory location 0xC4000 FIFO 1 memory location 0xC4100 FIFO 2 memory location 0xC4200 FIFO 3 memory location 0xC4300 FIFO 4 memory location 0xC4400 FIFO 5 memory location 0xC4500 Table 29: SUB FIFO Memory Address [TO: IPOS_FIFO_CTRL_89457790].

Covers: DDS_STBIO1_89457790

[End]

[SRS-REQ-108] Requirement:

The Smart FIFO block shall implement: If input signal HMASTER is set to 0 or 1, the FIFO Controller shall enter Memory mode. [TO: IPOS_FIFO_CTRL_00000008].

Covers: DDS_STBIO1_00000008

[End]

[SRS-REQ-109] Requirement:

The Smart FIFO block shall implement: When in Multiple Fifo, the 6 sub-FIFO offset shall coincide with the following table : FIFO OFFSET DATA TYPE ECG FIFO OFFSET ECG0_AC, ECG0_DC BIA FIFO OFFSET BIA_AC_P, BIA_DC_P, BIA_AC_Q, BIA_DC_Q, ECG1_AC, ECG2_DC GSR FIFO OFFSET GSR, ECG1_DC, ECG2_AC PPG FIFO OFFSET PPG Table 31 : FIFO Offset Data Table.

Covers: DDS_STBIO1_00001199

[End]

### 9.6 IRQ logic

General functional description: Aggregate and route FIFO, Sensor Hub, ISPU sample-ready, software, and timer interrupt status to configured GPIO/interrupt outputs.

Inputs: FIFO status, DRDY events, threshold/events Outputs: INT1, INT2

Interface catalog evidence:

| Interface | Direction | Type | Owner | Purpose |
| --- | --- | --- | --- | --- |
| INT1 | output | digital | IRQ logic | Primary interrupt output |
| INT2 | output | digital | IRQ logic | Secondary interrupt output |

[SRS-REQ-110] Requirement:

The IRQ logic block shall implement: The irq_FIFO_U_M_IRQ_STsATUS interrupt shall be connected to FIFO status to bit 29.

Covers: DDS_STBIO1_0501

[End]

[SRS-REQ-111] Requirement:

The IRQ logic block shall implement: The irq_SENSHUB_STS interrupt shall be connected to SENSOR HUB status to bit 28.

Covers: DDS_STBIO1_0502

[End]

[SRS-REQ-112] Requirement:

The IRQ logic block shall implement: The irq_ISPU_MC_INTR_STS interrupt shall be connected to ECG, BIA or PPG samples ready for elaboration to bit 27.

Covers: DDS_STBIO1_0503

[End]

[SRS-REQ-113] Requirement:

The IRQ logic block shall implement: The sw_interrupt interrupt shall be connected to sw interrupt used by ADSP or external user to bit 26:11 16.

Covers: DDS_STBIO1_0504

[End]

[SRS-REQ-114] Requirement:

The IRQ logic block shall implement: The irq_timer interrupt shall be connected to Internal timer to bit 1.

Covers: DDS_STBIO1_0505

[End]

### 9.7 PMU

General functional description: Sequence POR-driven LDO1V8 enable, 64/32 kHz and 16 MHz clocks, clock-ready signaling, and digital-reset release for boot and power modes.

Inputs: VDD/VDD_IO, power-on reset, clock references, clock-start requests, power-mode configuration Outputs: LDO and power-domain controls, clock enable/gating and ready status, reset-release signals, power-state status

Interface catalog evidence:

| Interface | Direction | Type | Owner | Purpose |
| --- | --- | --- | --- | --- |
| VDD | input | power | PMU | Core supply |
| VDD_IO | input | power | PMU | Digital IO supply |

[SRS-REQ-002] Requirement:

The PMU block shall implement: When the device is turned on, i_POR1V2_1V2 shall be high. [TO: IPOS_PMU].

Covers: DDS_STBIO1_0104

[End]

[SRS-REQ-003] Requirement:

The PMU block shall implement: When i_POR1V2_1V2 is low, o_EN_LDO1V8 shall be low. [TO: IPOS_PMU].

Covers: DDS_STBIO1_0105

[End]

[SRS-REQ-004] Requirement:

The PMU block shall implement: When i_POR1V2_1V2 is low, o_PD_CLKF shall be high. [TO: IPOS_PMU].

Covers: DDS_STBIO1_0106

[End]

[SRS-REQ-005] Requirement:

The PMU block shall implement: When i_POR1V2_1V2 is low, o_CLK_16M_READY shall be low. [TO: IPOS_PMU].

Covers: DDS_STBIO1_0107

[End]

[SRS-REQ-006] Requirement:

The PMU block shall implement: When i_POR1V2_1V2 is low, o_DIS_RC_VIREF shall be high. [TO: IPOS_PMU].

Covers: DDS_STBIO1_0108

[End]

[SRS-REQ-007] Requirement:

The PMU block shall implement: When i_POR1V2_1V2 rises, after at least 10 cycles of i_clk_64k (64kHz), o_EN_LDO1V8 shall be high and the clk_64k_divided is considered stable. [TO: IPOS_PMU].

Covers: DDS_STBIO1_0109

[End]

[SRS-REQ-008] Requirement:

The PMU block shall implement: When clk_64k_divided is stable, after at least 352 cycles of clk_64k_divided (32kHz), o_DIS_RC_VIREF shall be low. [TO: IPOS_PMU].

Covers: DDS_STBIO1_0110

[End]

[SRS-REQ-009] Requirement:

The PMU block shall implement: When i_mask_en_ldo_pdclkf_reg and o_EN_LDO1V8 are set to 1, o_PD_CLKF shall be low after at least 40 cycles of the clk_64k_divided. [TO: IPOS_PMU].

Covers: DDS_STBIO1_0111

[End]

[SRS-REQ-010] Requirement:

The PMU block shall implement: When i_START_CLK_16M is high, after at least 20 cycles of i_clk_16M (16MHz), o_CLK_16M_READY shall be high. [TO: IPOS_PMU].

Covers: DDS_STBIO1_0112

[End]

[SRS-REQ-011] Requirement:

The PMU block shall implement: When i_START_CLK_16M is high for the first time, after at least 20 cycles of i_clk_16M (16MHz), o_main_resetn shall be high. [TO: IPOS_PMU].

Covers: DDS_STBIO1_0113

[End]

[SRS-REQ-012] Requirement:

The PMU block shall implement: The stbio1_top.clk_16m signal shall be connected to the stbio1_top.dig_wrapper_LV.u_pmu.clk_16m signal, Delay: No; Clock gating: No; F max: 16 MHz.

Covers: DDS_STBIO1_1020

[End]

[SRS-REQ-013] Requirement:

The PMU block shall implement: The stbio1_top.clk_16m signal shall be connected to the stbio1_top.dig_wrapper_LV.u_pmu.clk signal, Delay: No; Clock gating: No; F max: 64 kHz.

Covers: DDS_STBIO1_1021

[End]

[SRS-REQ-014] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmu.tst_clk_32k signal shall be connected to the stbio1_top.dig_wrapper_LV.u_main_controller_top.i_clk_32kHz signal, Delay: No; Clock gating: No; F max: 32 kHz.

Covers: DDS_STBIO1_1027

[End]

[SRS-REQ-015] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmu.clk_16m_dft signal shall be connected to the stbio1_top.dig_wrapper_LV.u_ADSP.stmc_clk signal, Clock gating: stbio1_top.dig_wrapper_LV.u_pm u.adsp_run | stbio1_top.dig_wrapper_LV.u_pm u.all_test_mode | ~stbio1_top.dig_wrapper_LV.u_p ad_mux.scan_enable; F max: 16 MHz.

Covers: DDS_STBIO1_1022

[End]

[SRS-REQ-016] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmu.w_sh_clk signal shall be connected to the stbio1_top.dig_wrapper_LV.u_masterblaze_ahb.sys_clk signal, Clock gating: stbio1_top.dig_wrapper_LV.u_pm u.i_en_sh_clk | stbio1_top.dig_wrapper_LV.u_pm u.all_test_mode | ~stbio1_top.dig_wrapper_LV.u_p ad_mux.scan_enable; F max: 16 MHz.

Covers: DDS_STBIO1_1023

[End]

[SRS-REQ-017] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmu.clk_16m_dft signal shall be connected to the stbio1_top.dig_wrapper_LV.u_xbar_afe.hclk signal, Clock gating: stbio1_top.dig_wrapper_LV.u_pm u.clk_xbar_en | stbio1_top.dig_wrapper_LV.u_pm u.all_test_mode | ~stbio1_top.dig_wrapper_LV.u_p ad_mux.scan_enable; F max: 16 MHz.

Covers: DDS_STBIO1_1024

[End]

[SRS-REQ-018] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmuclk_xbar signal shall be connected to the stbio1_top.dig_wrapper_LV.u.fifo.HCLK signal, Clock gating: stbio1_top.dig_wrapper_LV.u_pm u.clk_en_m2 | stbio1_top.dig_wrapper_LV.u_pm u.all_test_mode | stbio1_top.dig_wrapper_LV.u_pm u.clk_en_m2_reg | ~stbio1_top.dig_wrapper_LV.u_p ad_mux.scan_enable; F max: 16 MHz.

Covers: DDS_STBIO1_2024

[End]

[SRS-REQ-019] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmu.clk_stredl signal shall be connected to the stbio1_top.dig_wrapper_LV.u_ispu.clk signal, Clock gating: No; F max: 16 MHz.

Covers: DDS_STBIO1_2025

[End]

[SRS-REQ-020] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmu.clk_xbar signal shall be connected to the stbio1_top.dig_wrapper_LV.u_regmap_otp.hclk signal, Clock gating: stbio1_top.dig_wrapper_LV.u_pm u.clk_en_m3 | stbio1_top.dig_wrapper_LV.u_pm u.all_test_mode | stbio1_top.dig_wrapper_LV.u_pm u.clk_en_m3_reg | stbio1_top.dig_wrapper_LV.u_pm u.clk_en_s3 | ~stbio1_top.dig_wrapper_LV.u_p ad_mux.scan_enable; F max: 16 MHz.

Covers: DDS_STBIO1_2026

[End]

[SRS-REQ-021] Requirement:

The PMU block shall implement: The u_stbio1_top.u_pmu.u_device_startup.tst_clk_32k signal shall be connected to the stbio1_top.dig_wrapper_LV.tst_clk_32k signal, Delay: No; Clock gating: No; F max: 32 kHz.

Covers: DDS_STBIO1_1025

[End]

[SRS-REQ-022] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmu.tst_clk_16m signal shall be connected to the stbio1_top.dig_wrapper_LV.u_main_controller_top.i_clk_16MHz signal, Clock gating: stbio1_top.dig_wrapper_LV.u_pm u.i_en_clk_16MHz_sleep | stbio1_top.dig_wrapper_LV.u_pm u.scan_mode | ~stbio1_top.dig_wrapper_LV.u_p ad_mux.scan_enable; F max: 16 MHz.

Covers: DDS_STBIO1_1026

[End]

[SRS-REQ-023] Requirement:

The PMU block shall implement: The u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_stmc signal shall be connected to the u_stbio1_top.dig_wrapper_LV.u_ADSP.HRESETn signal.

Covers: DDS_STBIO1_1050

[End]

[SRS-REQ-024] Requirement:

The PMU block shall implement: The u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_masterblaze signal shall be connected to the u_stbio1_top.dig_wrapper_LV.u_masterblaze_ahb.HRESETn signal.

Covers: DDS_STBIO1_1051

[End]

[SRS-REQ-025] Requirement:

The PMU block shall implement: The u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_regmap signal shall be connected to the u_stbio1_top.dig_wrapper_LV.u_regmap_otp.hresetn signal.

Covers: DDS_STBIO1_1052

[End]

[SRS-REQ-026] Requirement:

The PMU block shall implement: The stbio1_top.dig_wrapper_LV.u_pmu signal shall be connected to the u_device_startup.u_por_main_resetn.RST_N_FF signal.

Covers: DDS_STBIO1_2052

[End]

[SRS-REQ-027] Requirement:

The PMU block shall implement: The u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_regmap signal shall be connected to the u_stbio1_top.dig_wrapper_LV.u_ispu.main_rst_n signal.

Covers: DDS_STBIO1_2053

[End]

[SRS-REQ-028] Requirement:

The PMU block shall implement: The u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_fifo signal shall be connected to the u_stbio1_top.dig_wrapper_LV.u_multi_channel_fifo_top.HRESETn signal.

Covers: DDS_STBIO1_1053

[End]

[SRS-REQ-029] Requirement:

The PMU block shall implement: The u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_main_ctrl signal shall be connected to the u_stbio1_top.dig_wrapper_LV.u_main_controller_top.i_rstn signal.

Covers: DDS_STBIO1_1054

[End]

[SRS-REQ-030] Requirement:

The PMU block shall implement: The u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_32k_main_ctrl signal shall be connected to the u_stbio1_top.dig_wrapper_LV.u_main_controller_top.i_rstn_sync_32 signal.

Covers: DDS_STBIO1_2054

[End]

[SRS-REQ-121] Requirement:

The PMU block shall implement: At the release of the POR signal from the analog domain, the PMU shall manage the turn-on of the LDO1V8 and starts clocks Start-Up Sequence (reported in PMU IPOS) [TO: IPOS_PMU].

Covers: DDS_STBIO1_0114

[End]

[SRS-REQ-122] Requirement:

The PMU block shall implement: After At least 2 ms from the Release of the POR, the reset of the digital domain shall be de- asserted (an overlook on this part is described in the PMU IPOS) and the device shall start the Boot Phase [TO: IPOS_PMU].

Covers: DDS_STBIO1_0115

[End]

### 9.8 I2C_SPI_AHB

General functional description: Accept I2C/SPI accesses and translate them as needed into AHB transactions for device configuration and readback, including ISPU access.

Inputs: Host bus transactions Outputs: Protocol-compliant responses

Interface catalog evidence:

| Interface | Direction | Type | Owner | Purpose |
| --- | --- | --- | --- | --- |
| SERIAL_CLK | input | digital | SPI interface | Serial clock input |
| SERIAL_SELECT | input | digital | SPI interface | Serial frame/select input |
| SERIAL_DATA_IN | input | digital | SPI interface | Serial data input |
| SERIAL_DATA_OUT | output | digital | SPI interface | Serial data output |
| ALT_SERIAL_CLK | input | digital | I2C interface | Alternate serial clock input |
| ALT_SERIAL_DATA | bidirectional | digital | I2C interface | Alternate serial bidirectional data line |

[SRS-REQ-067] Requirement:

The I2C_SPI_AHB block shall implement: I2C interface shall support all the specified modalities (Standard, FM, FM+) and, if configured, HS is also supported.

Covers: DDS_STBIO1_8000

[End]

[SRS-REQ-068] Requirement:

The I2C_SPI_AHB block shall implement: Clock frequency ratio between system (AHB) and I2C/SPI interfaces domains shall be at least 2 times. [TO: IPOS_I2C_SPI_AHB_0006] [Vpriority High].

Covers: DDS_STBIO1_8001

[End]

[SRS-REQ-069] Requirement:

The I2C_SPI_AHB block shall implement: An I2C/SPI write access to the address 0x60-0x67 shall be translated in an AHB write access to the configured AHB address (contained in registers SAM_ADDR_BYTE0/1/2/3, address 0x50/0x51/0x52/0x53). [TO: IPOS_I2C_SPI_AHB_0009] [Vpriority High].

Covers: DDS_STBIO1_8003

[End]

[SRS-REQ-070] Requirement:

The I2C_SPI_AHB block shall implement: A double byte write access to the address 0x68-0x6f (with data that is the number of bytes that the master wants to read) shall start the data preload from AHB in order to have the internal fifo not empty when the read request will be done by the master. [TO: IPOS_I2C_SPI_AHB_0010] [Vpriority High].

Covers: DDS_STBIO1_8004

[End]

[SRS-REQ-071] Requirement:

The I2C_SPI_AHB block shall implement: An I2C/SPI read access to the address 0x70-0x7f shall be translated in an AHB read access to the configured AHB address (contained in registers SAM_ADDR_BYTE0/1/2/3, address 0x50/0x51/0x52/0x53). [TO: IPOS_I2C_SPI_AHB_0011] [Vpriority High].

Covers: DDS_STBIO1_8005

[End]

[SRS-REQ-072] Requirement:

The I2C_SPI_AHB block shall implement: The IP has a serial interface (I2C or SPI) and a parallel bus interface AHB. [TO: IPOS_I2C_SPI_AHB_0007].

Covers: DDS_STBIO1_8002

[End]

[SRS-REQ-073] Requirement:

The I2C_SPI_AHB block shall implement: On AHB interface, data size shall be limited to BYTE and WORD. [TO: IPOS_I2C_SPI_AHB_0361].

Covers: DDS_STBIO1_8006

[End]

[SRS-REQ-074] Requirement:

The I2C_SPI_AHB block shall implement: On AHB interface, BURST shall not be supported as well as LOCKED transfers. [TO: IPOS_I2C_SPI_AHB_0362].

Covers: DDS_STBIO1_8007

[End]

### 9.9 ISPU

General functional description: Represent the ISPU digital architecture block and its source/destination responsibilities described by Stage 2 interaction evidence.

Inputs: Defined by interaction matrix ingress and upstream requirement evidence Outputs: Defined by interaction matrix egress and upstream requirement evidence

[SRS-REQ-115] Requirement:

The ISPU block shall implement: To put the ISPU into sleep mode, it is necessary to perform a read at address 0x80054.

Covers: DDS_STBIO1_0509

[End]

[SRS-REQ-116] Requirement:

The ISPU block shall implement: To wake up the ISPU, it is necessary to trigger an interrupt.

Covers: DDS_STBIO1_05010

[End]

[SRS-REQ-117] Requirement:

The ISPU block shall implement: When the ISPU is in sleep mode, it is possible to perform reads and writes from the data and code memory by triggering an AHB transition.

Covers: DDS_STBIO1_05011

[End]

[SRS-REQ-118] Requirement:

The ISPU block shall implement: To turn off the ISPU shall be:.

- Disable the power island by writing to regmap POWER CONFIGURATION REGISTER via AHB interface (ADDRESS=0xCA000, DATA=0x00)

Covers: DDS_STBIO1_0508

[End]

[SRS-REQ-119] Requirement:

The ISPU block shall implement: After the initial boot phase, to turn on the ISPU shall be:.

- Select page 3 of the regmap (ADDRESS=0x04, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable user mode by writing to regmap (ADDRESS=0x5E, DATA=0x01)

- Enable the power island by writing to regmap POWER CONFIGURATION REGISTER via AHB interface (ADDRESS=0xCA000, DATA=0x01)

Covers: DDS_STBIO1_0507

[End]

## 8.4 Source Function Context

### 8.4.1 BIST MODE

[SRS-REQ-153] Requirement:

After the initial boot phase, to turn on the Debug mode shall be:

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable the power island []

- Enable bist mode by writing to regmap (ADDRESS=0x3E, DATA=0x04)

Covers: DDS_STBIO1_2507

[End]

Source paragraph: Section 18.5 BIST MODE (under Section 18 Digital DFT), paragraph 030 (page 161)

### 8.4.2 BOOT Phase

[SRS-REQ-123] Requirement:

To check the end of the BOOT phase, the user shall read the main_controller_status_register (DEVICE_STATUS_REG register [2:0]) and follow these steps:

- Read the DEVICE_STATUS_REG [2:0]

- Check if the Value is 1 (BOOT phase still in process)

- If the value is 2, then the BOOT phase is ended and the device is in IDLE. [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0116

[End]

Source paragraph: Section 15.1 BOOT Phase (under Section 15 Measurement Cycle Examples), paragraph 013 (page 145)

### 8.4.3 Configuration Phase

[SRS-REQ-124] Requirement:

During the Configuration Phase (IDLE STATE of the Device_FSM), the user shall be able to:

- Configure Bio-Channel or Optical Channel, the ADC Calibration and then Start a measurement cycle.

- Put the device in Debug-Mode. [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0117

[End]

Source paragraph: Section 15.2 Configuration Phase (under Section 15 Measurement Cycle Examples), paragraph 007 (page 146)

### 8.4.4 Debug mode

[SRS-REQ-149] Requirement:

After the initial boot phase, to turn on the Debug mode shall be:

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable debug mode by writing to regmap (ADDRESS=0x3E, DATA=0x08)

Covers: DDS_STBIO1_0700

[End]

Source paragraph: Section 18.2 Debug mode (under Section 18 Digital DFT), paragraph 024 (page 159)

### 8.4.5 ECG Only

[SRS-REQ-125] Requirement:

- The user shall configure time slot duration in DEVICE_CONFIG_OPERATION register [4:2] [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0118

[End]

Source paragraph: Section 1 TIME SLOT LENGTH: (under Section 15.3.1 ECG Only), paragraph 008 (page 148)

[SRS-REQ-126] Requirement:

- The user shall select active channel, in this case only ECG0_SEL and DS_SEL (Data Storage Mode) in DEVICE_CONFIG_CHANNEL register writing 0x21. [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0119

[End]

Source paragraph: Section 2 SELECT CHANNEL: (under Section 15.3.1 ECG Only), paragraph 012 (page 148)

[SRS-REQ-127] Requirement:

- The user shall select a division index for ECG, configuring M_ECG_l and M_ECG_h in SET_ECG_FREQ_l register and SET_ECG_FREQ_h register. [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0120

[End]

Source paragraph: Section 3 SELECT DIVISION INDEX FOR CONFIGURED CHANNELS: (under Section 15.3.1 ECG Only), paragraph 017 (page 148)

[SRS-REQ-128] Requirement:

- The user shall set the number of clocks for sampling ECG data from ADC in ADC_CONFIG_ECG register. [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0121

[End]

Source paragraph: Section 4 CONFIGURE ADC SAMPLING PERIOD: (under Section 15.3.1 ECG Only), paragraph 022 (page 148)

[SRS-REQ-129] Requirement:

- The User shall set at 1 HC_EN in ECG_CTRL register. [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0122

[End]

Source paragraph: Section 5 GENERAL CONFIG: (under Section 15.3.1 ECG Only), paragraph 027 (page 148)

[SRS-REQ-130] Requirement:

- The user shall start measurement writing register DEVICE_CONFIG_OPERATION [0] at 1. [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0123

[End]

Source paragraph: Section 6 SELECT OPERATIVE MODE: (under Section 15.3.1 ECG Only), paragraph 031 (page 148)

[SRS-REQ-131] Requirement:

- To check data in unique FIFO (default config) the user shall check the first MSB which will have to be equal to the table in DATA TAG Table. [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0124

[End]

Source paragraph: Section 7 CHECK DATA: (under Section 15.3.1 ECG Only), paragraph 036 (page 148)

### 8.4.6 ECG and BIA

[SRS-REQ-132] Requirement:

- The user shall configure the time slot duration in DEVICE_CONFIG_OPERATION register [4:2].

Covers: DDS_STBIO1_0125

[End]

Source paragraph: Section 1 TIME SLOT LENGTH: (under Section 15.3.2 ECG and BIA), paragraph 012 (page 150)

[SRS-REQ-133] Requirement:

- The user shall select active channel, in this case ECG0_SEL, BIA_SEL and DS_SEL (Data Storage Mode) in DEVICE_CONFIG_CHANNEL writing 0x23. [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0126

[End]

Source paragraph: Section 2 SELECT CHANNEL: (under Section 15.3.2 ECG and BIA), paragraph 016 (page 150)

[SRS-REQ-134] Requirement:

- The user shall select a division index for ECG, configuring M_ECG_l and M_ECG_h in SET_ECG_FREQ_l register and SET_ECG_FREQ_h register. [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0127

[End]

Source paragraph: Section 3 SELECT DIVISION INDEX FOR CONFIGURED CHANNELS: (under Section 15.3.2 ECG and BIA), paragraph 021 (page 150)

[SRS-REQ-135] Requirement:

- The user shall select a division index for ECG, configuring M_BIA_l and M_BIA_h in SET_BIA_FREQ_l register and SET_BIA_FREQ_h register. [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0128

[End]

Source paragraph: Section 3 SELECT DIVISION INDEX FOR CONFIGURED CHANNELS: (under Section 15.3.2 ECG and BIA), paragraph 025 (page 150)

[SRS-REQ-136] Requirement:

- The user shall set number of clocks for sampling ECG and BIA data from ADC in ADC_CONFIG_ECG register. [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0129

[End]

Source paragraph: Section 4 CONFIGURE ADC SAMPLING PERIOD: (under Section 15.3.2 ECG and BIA), paragraph 029 (page 150)

[SRS-REQ-137] Requirement:

- The user shall set at 1 HC_EN in ECG_CTRL register. [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0130

[End]

Source paragraph: Section 5 GENERAL CONFIG: (under Section 15.3.2 ECG and BIA), paragraph 033 (page 150)

[SRS-REQ-138] Requirement:

- The user shall set at 1 IMP_EN in IMP_CTRL register. [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0140

[End]

Source paragraph: Section 5 GENERAL CONFIG: (under Section 15.3.2 ECG and BIA), paragraph 036 (page 150)

[SRS-REQ-139] Requirement:

- The user shall start measurement writing register DEVICE_CONFIG_OPERATION [0] at 1. [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0141

[End]

Source paragraph: Section 6 SELECT OPERATIVE MODE: (under Section 15.3.2 ECG and BIA), paragraph 040 (page 150)

[SRS-REQ-140] Requirement:

- To check data in unique FIFO (default config) the user shall check the first MSB which will have to be equal to the one described in the DATA TAG Table. [TO: IPOS_MAIN_CTRL]

Covers: DDS_STBIO1_0142

[End]

Source paragraph: Section 7 CHECK DATA: (under Section 15.3.2 ECG and BIA), paragraph 044 (page 150)

### 8.4.7 Other source function context

[SRS-REQ-063] Requirement:

If bit OTP_PRG10[31] in OTP Registers, called write_protection_en, is set to 1, and STBIO1 is not in debug_mode, register map shall refuse any incoming write request if the transaction address is within the OTP Registers space (from 0xCD000 to 0xCD02F).

Covers: DDS_STBIO1_0036

[End]

Source paragraph: Section 4 OTP BOOT routine, paragraph 030 (page 97)

### 8.4.8 PAD Mux

[SRS-REQ-031] Requirement:

If MODE_CONFIG register is 0x00 the operative mode shall be functional. [Vpriority High]

Covers: DDS_STBIO1_302

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 010 (page 59)

[SRS-REQ-032] Requirement:

If MODE_CONFIG register is 0x01 the operative mode shall be SCAN.

Covers: DDS_STBIO1_303

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 014 (page 59)

[SRS-REQ-033] Requirement:

If MODE_CONFIG register is 0x04 the operative mode shall be BIST.

Covers: DDS_STBIO1_304

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 018 (page 59)

[SRS-REQ-034] Requirement:

If MODE_CONFIG register is 0x08 the operative mode shall be DEBUG.

Covers: DDS_STBIO1_305

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 022 (page 59)

[SRS-REQ-035] Requirement:

If MODE_CONFIG register is 0x10 the operative mode shall be ISPU_DEBUG. [Vpriority High]

Covers: DDS_STBIO1_306

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 026 (page 59)

[SRS-REQ-036] Requirement:

If MODE_CONFIG register is 0x20 the operative mode shall be Bist MEMSAFE mode. [Vpriority High]

Covers: DDS_STBIO1_307

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 030 (page 59)

[SRS-REQ-037] Requirement:

If MODE_CONFIG register is 0x40 the operative mode shall be Bist Bypass mode. [Vpriority High]

Covers: DDS_STBIO1_308

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 034 (page 59)

[SRS-REQ-038] Requirement:

In function mode, the GPIO1,2,3,4_out shall be General-Purpose if sel irq gpi1,2,3,4 is 0x00 and ext_cap_en is “0” [Vpriority High]

Covers: DDS_STBIO1_314

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 038 (page 59)

[SRS-REQ-039] Requirement:

In function mode, the GPIO1,2,3,4_out shall be irq_fifo if sel irq gpio1,2,3,4 is 0x01 and ext_cap_en is “0” [Vpriority High]

Covers: DDS_STBIO1_309

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 043 (page 59)

[SRS-REQ-040] Requirement:

In function mode, the GPIO1,2,3,4_out shall be i_LED_Pilot if sel irq gpi1,2,3,4 is 0x02 and ext_cap_en is “0” [Vpriority High]

Covers: DDS_STBIO1_310

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 051 (page 59)

[SRS-REQ-041] Requirement:

In function mode, the GPIO1,2,3,4_out shall be i_DCCK_irq if sel irq gpi1,2,3,4 is 0x03 and ext_cap_en is “0” [Vpriority High]

Covers: DDS_STBIO1_311

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 004 (page 60)

[SRS-REQ-042] Requirement:

In function mode, the GPIO1,2,3,4 output shall be i_error_time_slot if sel irq gpi1,2,3,4 is 0x04 and ext_cap_en is “0” [Vpriority High]

Covers: DDS_STBIO1_312

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 009 (page 60)

[SRS-REQ-043] Requirement:

In function mode, the GPIO1,2,3,4 output shall be i_time_slot_irq if sel irq gpi1,2,3,4 is 0x05 and ext_cap_en is “0” [Vpriority High]

Covers: DDS_STBIO1_313

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 014 (page 60)

[SRS-REQ-044] Requirement:

If O_DCCK_P_i changes its value from ‘0’ to ‘1’ and DCCK_IRQ_MASK is set to ‘1’, then irq_DCCK_IRQ_ST, output of u_regmap_otp, shall be set to ‘1’

Covers: DDS_STBIO1_392

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 018 (page 71)

[SRS-REQ-045] Requirement:

If O_DCCK_N_i changes its value from ‘0’ to ‘1’ and DCCK_IRQ_MASK is set to ‘1’, then irq_DCCK_IRQ_ST, output of u_regmap_otp, shall be set to ‘1’

Covers: DDS_STBIO1_393

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 023 (page 71)

[SRS-REQ-046] Requirement:

If O_DCCK_P_i changes its value from ‘1’ to ‘0’ and DCCK_IRQ_MASK is set to ‘1’, then irq_DCCK_IRQ_ST, output of u_regmap_otp, shall be set to ‘1’

Covers: DDS_STBIO1_394

[End]

Source paragraph: Section 13.1.2.2 Requirements (under Section 13.1.2 PAD Mux), paragraph 028 (page 71)

### 8.4.9 PPG Only

[SRS-REQ-141] Requirement:

- The User shall configure time slot duration in DEVICE_CONFIG_OPERATION register [4:2].

Covers: DDS_STBIO1_0143

[End]

Source paragraph: Section 1 TIME SLOT LENGTH: (under Section 15.3.3 PPG Only), paragraph 010 (page 152)

[SRS-REQ-142] Requirement:

- The user shall configure a division index PPG, writing M_PPG_l and M_PPG_h in SET_PPG_FREQ_l register and SET_PPG_FREQ_h register.

Covers: DDS_STBIO1_0145

[End]

Source paragraph: Section 3 SELECT DIVISION INDEX FOR CONFIGURED CHANNELS: (under Section 15.3.3 PPG Only), paragraph 018 (page 152)

[SRS-REQ-143] Requirement:

- The user shall configure the number of clocks for sampling PPG data from ADC in ADC_CONFIG_PPG register.

Covers: DDS_STBIO1_0146

[End]

Source paragraph: Section 4 CONFIGURE ADC SAMPLING PERIOD: (under Section 15.3.3 PPG Only), paragraph 022 (page 152)

[SRS-REQ-144] Requirement:

- The user shall select active channel, in this case only PPG_SEL and DS_SEL (data Storage) in DEVICE_CONFIG_CHANNEL register shall be set to 0x30.

Covers: DDS_STBIO1_0144

[End]

Source paragraph: Section 2 SELECT CHANNEL: (under Section 15.3.3 PPG Only), paragraph 014 (page 152)

[SRS-REQ-145] Requirement:

The user shall configure each frame following these steps:

5.1 ALC PARAMETERS: - T1 in PPG_ALC_CONFIG_PARAM_1 register. - T2, GAIN_START_SET, GAIN_MAX_SET in PPG_ALC_CONFIG_PARAM_2 register.

5.2 DIGITAL RAMP PARAMETERS: - N_MAX, T_DELTA in DIGITAL_RAMP_CONFIG_PARAM_1 register. - DELTA, T_DELTA1 in DIGITAL_RAMP_CONFIG_PARAM_2 register. - DELTA1 in DIGITAL_RAMP_CONFIG_PARAM_3 register.

5.3 GENERAL PPG PARAMETERS: - N_PPG_FRAME, WAVE_VELOCITY_MODE, SEL_PPG_ELAB in GENERAL_PPG_PARAMETERS_1 register. - T_TX_STARTUP in GENERAL_PPG_PARAMETERS_2 register. - T_RX_SET, PPG_START_UP_TIME_l in GENERAL_PPG_PARAMETERS_3 register. - PPG_START_UP_TIME_h in GENERAL_PPG_PARAMETERS_4 register.

5.4 PARAMETERS FOR EACH FRAME CONFIGURED BY USER: - RX_CH_FRAME_{n}, FRAME_{n}SAMPLES, FRAME{n}REPETITION in FRAME_n_PARAMETERS_1 register. - BTIA_GAIN_FRAME{n}, TX_LEDSET_FRAME_l_{n} in FRAME_n_PARAMETERS_2 register. - TX_LEDSET_FRAME_h_{n}, TX_LEDOFF_FRAME_l_{n} in FRAME_n_PARAMETERS_3 register. - TX_LEDOFF_FRAME_h_{n} in FRAME_n_PARAMETERS_4 register. - IDAC_DC_PPG_FRAME_{n} in FRAME_n_PARAMETERS_5 register. - EN_TX_i in FRAME_n_PARAMETERS_6 register. - CURR_SEL_TX_FRAME{n} in FRAME_n_PARAMETERS_7 register. - TX1_OUTAB_SEL_FRAME_{n}, TX2_OUTAB_SEL_FRAME_{n}, TX3_OUTAB_SEL_FRAME_{n}, TX4_OUTAB_SEL_FRAME_{n} in FRAME_n_PARAMETERS_10 register.

Covers: DDS_STBIO1_0147

[End]

Source paragraph: Section 5 PPG FRAMES CONFIG: (under Section 15.3.3 PPG Only), paragraph 026 (page 152)

[SRS-REQ-146] Requirement:

- The user shall write DEVICE_CONFIG_OPERATION register [0] at 1.

Covers: DDS_STBIO1_0148

[End]

Source paragraph: Section 6 SELECT OPERATIVE MODE: (under Section 15.3.3 PPG Only), paragraph 008 (page 153)

[SRS-REQ-147] Requirement:

- To check data in unique FIFO (default config) the user shall check the first MSB. PPG DATA has 20 bits of data and 4 of TAG. The 4 bits, from 0 to 11, are used to identify the frame (as described in the table in DATA TAG table).

Covers: DDS_STBIO1_0149

[End]

Source paragraph: Section 7 CHECK DATA: (under Section 15.3.3 PPG Only), paragraph 011 (page 153)

### 8.4.10 STBIO1 Register Map Peculiar Requirements

[SRS-REQ-001] Requirement:

Write enable for each OTP REGISTER shall be 0, in functional mode, if bit 31 of OTP_PRG10 is set 1.

Covers: DDS_STBIO1_4000

[End]

Source paragraph: Section 13.4.2 STBIO1 Register Map Peculiar Requirements (under Section 13.4 STBIO1 Register Map), paragraph 023 (page 135)

### 8.4.11 Scan mode

[SRS-REQ-148] Requirement:

After the initial boot phase, only with SPI interface, to turn on the Scan mode shall be:

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable debug mode by writing to regmap (ADDRESS=0x3E, DATA=0x01) Figure 26 scan mode activation timing

Covers: DDS_STBIO1_0699

[End]

Source paragraph: Section 18.1 Scan mode (under Section 18 Digital DFT), paragraph 011 (page 159)

## Assumptions and TBD

- ASSUME-001: Verification environments include controllable stimulus for all listed operating modes.

- TBD-001: Unassigned requirements in traceability matrix need architectural owner review.

## 10. Missing Inputs

- None