DDS-Digital Design Specification

STBIO1

Marco Leo, Ugo Garozzo, Aldo Torino, Alessandro Lucio Iannuzzi

| REV. | DATE | AUTHOR |  |
| --- | --- | --- | --- |
| Draft 1.10 | 19 March 2026 | Ugo Garozzo, Marco Leo, Aldo Torino, Alessandro Lucio Iannuzzi |  |
|  | APPROVED By |  | DATE |
|  | Francesco Villa, Pasquale Biancolillo |  | 19 March 2026 |

2026

# Table of Contents

Table of Contents	1

Table of Tables	4

Table of Figures	4

1.	Abstract	6

2.	Version History	7

3.	Reference documents and Ip Repository	8

4.	Conventions	9

4.1.	Abbreviations	9

5.	STBIO1 toplevel overview	11

6.	Symbol	15

7.	I/O List	15

8.	STBIO1: Functional Modes	17

8.1.	Data Storage Mode	17

8.2.	Normal Mode	17

9.	Digital Architecture Overview	17

9.1.	TOP Pinout List	18

10.	Power Domains	30

10.1.	Introduction	30

10.2.	Definitions	31

10.3.	Power Domain Architecture	31

10.4.	Power Domain Descriptions	32

10.4.1.	PD_TOP1V2	32

10.4.2.	PD_STREDL	32

10.4.3.	PD_TOP3V3	32

10.4.4.	PD_TOPIO	32

11.	Interconnect	32

11.1.	XBAR Connection Matrix	33

11.2.	XBAR Memory Mapping	34

12.	STBIO1 Operating Modes	35

13.	Modules	37

13.1.1.	PMU	37

13.1.2.	PAD Mux	49

13.1.3.	Main Controller TOP	72

13.2.	IPs	86

13.2.1.	ADSP integration	86

13.2.2.	I2C_SPI_AHB integration	98

13.2.3.	SensorHub integration	104

13.2.4.	FIFO_CTRL integration	112

13.3.	ISPU integration	116

13.3.1.	Requirements	130

13.4.	STBIO1 Register Map	133

13.4.1.	STBIO1 Register Map	133

13.4.2.	STBIO1 Register Map Peculiar Requirements	135

14.	Soft Reset Procedures	144

14.1.	Main Controller Soft Reset	144

15.	Measurement Cycle Examples	144

15.1.	BOOT Phase	145

15.2.	Configuration Phase	146

15.3.	Measurement Cycle	147

15.3.1.	ECG Only 	148

15.3.2.	ECG and BIA	150

15.3.3.	PPG Only	152

16.	SCAN	156

16.1.	Top Level Approach	156

17.	BIST	157

17.1.	Reference documents	157

17.2.	Memory BIST Architecture	157

17.2.1.	MBIST Collar	157

17.2.2.	MBIST Controller	157

17.2.3.	Top Level Approach	158

18.	Digital DFT	159

18.1.	Scan mode	159

18.2.	Debug mode	159

18.3.	ADC TEST LOW NOISE	160

18.4.	ADC TEST FAST	161

18.5.	BIST MODE	161

19.	References	163

# Table of Tables

Table 1: Revision History	7

Table 2: Revision History	8

Table 3: Git Repository Table	8

Table 4: Abbreviations Table	10

Table 5: I/O List	17

Table 6: Digital I/O List	30

Table 7: XBAR Connection Matrix	33

Table 8: Memory Map	34

Table 9: Operating modes Table	35

Table 10: PMU I/O List	42

Table 11: Clock Recap Table	46

Table 12: STBIO1 Functional reset Summary table	48

Table 13: PAD MUX I/O List	57

Table 14: PAD Configuration Table	58

Table 15: Main Controller Parameter	75

Table 16: Main Controller I/O List	85

Table 17: ADSP Parameter Table	86

Table 18: ADSP I/O List	87

Table 19: OTP I/O List	88

Table 20 Chosen OTP Timings	95

Table 21 Operation phases timings	95

Table 22: I2C/SPI AHB Parameters	99

Table 23: I2C/SPI AHB I/O List	101

Table 24: Sensor Hub Parameter	104

Table 25: Sensor Hub I/O List	105

Table 26: Fifo Parameter	112

Table 27: FIFO Controller I/O List	112

Table 28: SUB FIFO Memory Address	114

Table 29: Data Tag	115

Table 30: ISPU I/O List	129

Table 31: Address Register Map	133

Table 32: Location AFE_BLOCK Registers on the I2C/SPI pages.	134

Table 33:  AFE_BLOCK register list.	140

Table 34: I2C/SPI internal registers.	143

# Table of Figures

Figure 1: Time Slot Definition	12

Figure 2: STBIO1 Device Scheme	13

Figure 3: STBIO1 Reference Application	14

Figure 4: STBIO1 - Ball out. TOP View (Ball side down)	15

Figure 5: STBIO1 Digital Engine	18

Figure 6: STBIO1 Digital Scheme	33

Figure 7: Device Start-Up Phase	42

Figure 8: Mask Cell for LDO and Fast Ring Oscillator	43

Figure 9: Clocks Scheme	45

Figure 10: STBIO1 Reset Scheme	47

Figure 11: Main Controller Block Diagram	73

Figure 12: Boot Routine	91

Figure 13: Write Routine Start	92

Figure 14: WRITE routine and TEST regmap	93

Figure 15 : OTP Reading timing diagram	96

Figure 16: OTP Writing timing diagram	98

Figure 17: ISPU turn on phases	130

Figure 18: Timing turn on ISPU	131

Figure 19: Device Activity Flowchart	144

Figure 20: Boot Phase Flow	146

Figure 21: Configuration Phase Diagram	147

Figure 22: ECG ONLY FLOWCHART	150

Figure 23: ECG and BIA Measurement Flowchart	151

Figure 24: PPG Measurement Flowchart	154

Figure 25 Top Level Controller Configuration	158

Figure 26 scan mode activation timing	159

Figure 26 debug mode activation timing	160

Figure 27 bist mode activation timing	162

# Abstract

This document is the specification of the STBIO1 Project.

This document describes the functions, performance characteristics and required verification tests for the STBIO1 Project.

All requirements are considered Reviewed and Accepted once the document is officially approved.

# Version History

| Current Version | Previous Version | Date | Author | Change description |
| --- | --- | --- | --- | --- |
| 0.1 | N.A |  | Marco Leo | Starting point from VSM01 project |
| STBIO1_v0.3 | 0.2 | 6/17/2025 | Marco Leo | Added page numbers, update timing slot definition |
| STBIO1_v0.4 | 0.3 | 8/25/2025 | Marco Leo | Fix PMU Peculiar requirements. Added Soft Reset Procedure |
| STBIO1_v2 | 1.6.2 | 9/1/2025 | Ugo Garozzo | Adde new OTP routines description, Adde Soft Reset Procedures |
| STBIO1_v3 | 2 | 9/3/2025 | Ugo Garozzo | Add new output on TOP o_BDC_PPG, added Power Domain, Chanced ADSP IP ADSP_v0.3.21.OTP640 |
| STBIO1_v4 | 3 | 9/22/2025 | Ugo Garozzo, Marco Leo, Aldo Torino, Alessandro Iannuzzi | 1. Added reset value TOP; 2. Added Power DomainDescription; 3. Added resets of regmap; 4. Added new A2D input; 5. POR1V2_VDDIO; 6. new ADSP IP 7. new i2c/spi IP 8. Updated Top level Digital Scheme |
| STBIO1_v5 | 4 | 10/10/2025 | Aldo Torino, Ugo Garozzo | Added Approvers Data, Fix Tables Size, Fix Clocks and Reset Schemes, removed scan reports section |
| STBIO1_v6 | 5 | 11/5/2025 | Ugo Garozzo | Added DCCK, updated DDS_STBIO1_4000 new i2c/spi IP tag documentation, add pad_mux requirements |
| STBIO1_v7_DOC | 6 | 11/24/2025 | Aldo Torino | Added Requirement 00001199 for sub-fifo Offset |
| STBIO1_v8_DOC | 7 | 11/25/2025 | Ugo Garozzo | Updated |
| STBIO1_v9_DOC | 8 | 1/22/2026 | Aldo Torino | Fix set user mode description |
| STBIO1_v10_DOC | 9 | 3/19/2026 | Aldo Torino | Add Functional Mode Chapter, Reference Table and add Reference Chapter |

Table 1: Revision History

# Reference documents and Ip Repository

This is a list of reference documents.

[DDS_STBIO1_0002] Definition

| Current Version | Document | Author | Change Description |
| --- | --- | --- | --- |
| 1.0 | Main Controller IPOS (stbio1/FE/main_controller_top/Documentation/Specification/) | Aldo Torino, Sonia Andronaco, Giuliana De Milato |  |
| 3.0 | IPOS_I2C_SensorHub_Quokka_uP | Marco Leo |  |
|  | Documentation_Specification_FIFO_CTRL_dos | Roberta Priolo |  |
| 3.6 | Documentation_Specification_IP_Objective_Specification | Stefano Barbieri |  |
|  | ADSP IPOS | Alessandro Alaimo, Luca Gandolfi |  |
|  | H9A_MEM_OTP_ PUMP_85AL05_2 3A User Manual 2.0- 02 |  |  |
|  | https://developer.arm.com/documentation/ihi0033/latest/ | ARM |  |
| 0.2 | STRED-LP2 Architecture Reference Manual https://codex.cro.st.com/plugins/document/stbio1/folder/933984 | ASMS |  |
| 1.0 | ADSP micro Firmware | Alessandro Alaimo, Ugo Garozzo, Luca Gandolfi |  |

Table 2: Revision History

| IP | Git address | Git branch | Git tag |
| --- | --- | --- | --- |
| ADSP | git clone ssh://gitolite@codex.cro.st.com/ip-development/ADSP.git | adsp_vecma | ADSP_v0.3.22.OTP640 |
| i2c_spi_slave | git clone ssh://gitolite@codex.cro.st.com/ip-development/i2c_spi_slave.git | vecma | I2C_SPI_AHB_v3.1_061025_DOC |
| senshub_mst_i2c | ssh://gitolite@codex.cro.st.com/ip-development/senshub_mst_i2c.git | ahb_platf_ip | STBIO1_RTL100_v4.1 |

Table 3: Git Repository Table

| IP external | Description | Author |
| --- | --- | --- |
| ISPU | STRED-LP2-SVN_VERSION 9739 | SRA |
| BIST BLOCK | GenericMBIST | STMicroelectronics TRnD |
| ADSP ROM | H9A_M4_ST_ROM | STMicroelectronics TRnD |
| ADSP RAM | ST_SPSMALL_512x64m2_bM6_CUmr | STMicroelectronics TRnD |
| FIFO RAM | ST_SPSMALL_512x64m2_bM6_CUmr | STMicroelectronics TRnD |
| ISPU CODE | ST_SPUHD_2048x128m4_bM6_CUmr | STMicroelectronics TRnD |
| ISPU DATA | ST_SPUHD_2048x32m4_bM6_CUmr | STMicroelectronics TRnD |
| H9A_FU_OTPHV_85AL05_23A | H9A_FU_OTPHV_85AL05_23A_MIM5_6ML_ALUCU revision 2.6.b 640-bit |  |
| XBAR | CoreLink SIE-200 System IP for Embedded - Global Bundle r3p2-00rel0 |  |

Table 1:external IPs

# Conventions

[DDS_STBIO1_0003] Definition

If not otherwise noted, specification values are valid …

Abbreviations

[DDS_STBIO1_00304] Comment

The following abbreviations are used throughout this document:

[DDS_STBIO1_0006] Definition

| A |  |
| --- | --- |
| AGC | Automatic Gain Control |
| ASIL | Automotive Safety Integrity Level |
| ASIC | Application Specific Intergrated Circuit |
| B |  |
| BIST | Built In Self Test |
| … |  |
| C |  |
| CRM | Clock and Reset Management |
| F |  |
| FMEA | Failure Mode and Effect Analysis |
| FMEDA | Failure Mode, Effect and Diagnostic Analysis |
| FSM | Functional Safety Management |
| FTA | Fault Tree Analysis |
| CTRL | Controller |
| PPG | Photoplethysmography |
| EDA | Electro-Dermal Activity |
| GSR | Galvanic Skin Response |
| PMU | Power Management Unit |

Table 4: Abbreviations Table

[DDS_STBIO1_0007] Definition

Category Convention:

| Comment: | This category denotes that the content of the object text is a general comment. For example, this may be an explanation why the requirement demands certain items when there would also be other possibilities. A comment should not be necessary to understand the related requirements. |
| --- | --- |
| Definition: | This category is used for the definition of terms, wordings, technical expressions etc. It is a documentation of design decisions and general instructions that belong to the document itself, e.g. owner of the document, structure of the document. It is needed to understand a related requirement. It is not linked to any test case. |
| Assumption | This category is used for requirements this document provide upstream to another document, indicating what this IP/block/system needs to work properly. |
| Requirement: | This category denotes a requirement that has to be implemented and verified. Accordingly, it is necessary to establish traces from the different requirement levels to the test cases for this category. |

# STBIO1 toplevel overview

Aim of this document is to explain main features of the ASIC STBIO1, electronic interface of ECG, Bio-impedance, EDA and PPG (Photoplethysmography) embedded in a single die.

STBIO1 is an Analog Front End Device with embedded process capabilities.

STBIO1 collects and processes different types of Vital Signals:

- ECG

- PPG

- BIA and GSR

The optical channel (PPG) is designed as an optical transceiver, stimulating up to eight LEDs and measuring the return signal on up to six separate cent inputs. The signal chain rejects signal offsets and corruption from asynchronous modulated interference, typically from ambient light, eliminating the need for optical filters or externally controlled dc cancellation circuitry.

The ECG signal acquisition is designed to support low noise, diagnostic level measurement in the presence of a variety of interferers. The ECG signal chain has several complementary features supporting ECG measurement, such as driven reference for common-mode rejection and lead off detection to identify a fallen electrode.

The BIA signal chain is designed for body impedance and human breathing measurements with a configurable excitation path and measurement path. BIA channel delivers both the real and the imaginary parts of the body and the breathing impedance.

Finally, the last Electro-Dermal Activity (EDA), also called Galvanic Skin Response (GSR) refers to changes in sweat gland activity that are reflective of the intensity of our emotional state, otherwise known as emotional arousal.

STBIO1 is optimized ultra-low-power hardware circuitry for real-time execution of the algorithms is a state-of-the-art feature for any personal electronics, from wearable accessories to high-end applications.

The STBIO1 features an advanced digital engine, composed of multiple IPs able to reduce external host processing workload, reducing bus congestion, providing meta-data useful for final application, both in time and frequency domain.

The main IPs actors in digital domain are:

- ADSP

- Sensor-Hub

- ISPU 32-bit processor

- Smart FIFO data collector

ADSP is an embedded IP able to elaborate ultra-low-power custom DSP functions. The main purpose of ADSP is to reduce the computation workload on both ISPU and host processor, while saving power through the optimization of both hardware and algorithm.

Sensor hub IP can read external sensors by means of an I2C master protocol, enabling data collection in FIFO and embedded elaboration also on external data domain. The data collected by sensor hub is accessible from the Advanced DSP and ISPU, for sensor fusion algorithm elaboration.

The advanced ISPU (32bit proprietary core) is an ultra-low-power, high-performance programmable core which can execute signal processing and AI algorithms in the edge. The main benefits of the ISPU are C programming and an enhanced ecosystem with libraries and 3rd party tools/IDE. ISPU has 32 Kbyte program RAM, 24 Kbyte Data RAM capability and Floating-point support.

FIFO digital IPs oversees data collecting from multiple sources: internal and external data read by sensor hub, meta-data generated from both ADSP and ISPU.

The internal and external data that can be collected in FIFO are:

- ECG

- BIA

- GSR & 6 channels PPG

- up to 4 external sensors (expressed on 12 bytes each).

All the data are stored inside a Smart FIFO that can be configured as:

- Unique FIFO: Classic FIFO, the First Data that is written inside the Memory is also the First that is read.

- Multiple Fifo: The FIFO is split up to 6 Sub-FIFO (one for each Channels plus one for Sensor Hub and Elaboration results). The depth of each Sub-FIFO is configurable by regmap. Each Sub-FIFO behavior as a Classic FIFO.

[DDS_STBIO1_0101] Definition:

To understand how STBIO1 works, is necessary to define some basic concepts:

- TIME_SLOT= represents the interval within measurements are performed. It can be configured by the regmap field TIME_SLOT_SELECTION and its range is from 4096Hz to 64 Hz.

- In each TIME_SLOT we can distinguish two phases:

  - Operative: where all user configured measures are executed and ADSP and/or ISPU eventually runs its algorithms.

  - Sleep: where measures are finished, eventually only ADSP and/or ISPU is terminating its algorithms and writing elaborated data in FIFO.

Figure 1: Time Slot Definition

If all the selected operations do not end in a single Time Slot, an error shall be raised.

The Time Slot is composed of different Frame Slot representing the time-interval of a single operation (for example, ECG).

We also need to define the other 4 parameters: Mecg, Mbia, Mgsr and Mppg, also called Division Index.

These parameters are used to define the Data Rate Interval of a single Frame Slot.

Rate Interval= Time SlotMx

With x = ecg,ppg,bia and gsr.

Depending on the value of the Mx parameters, a Time_Slot can be composed by one or more Frame Slot or none. In the last case, the device will remain in SLEEP_OPERATIVE (as described in the next paragraphs). [END]

STBIO1 die size is 3700μm x 4000μm including scribe line of 80um. Technology used is HCMOS9A (130nm)

PDK_STM_HCMOS9A 11.8.b-03

PDK_STM_PLUS 04.04-02

Figure 2: STBIO1 Device Scheme

Figure 3: STBIO1 Reference Application

# Symbol

[DDS_STBIO1_0008] Definition

Figure 4: STBIO1 - Ball out. TOP View (Ball side down)

# I/O List

[DDS_STBIO1_0009] Definition

| Nr. | Name | Function | Pin No. | Type |
| --- | --- | --- | --- | --- |
| 1 | AV1V8 | 1.8 V Analog Power Supply | D5 | Power |
| 2 | AV3V3 | 3.3 V Analog Power Supply | D4 | Power |
| 3 | AGND | Analog Ground | D6 | Power |
| 4 | DVDD | 1.8 V Digital Power Supply | A3 | Power |
| 5 | GNDD | Digital Ground | B3 | Power |
| 6 | VDDIO | I/O Power Supply. | A2 | Power |
| 7 | GNDIO | I/O Ground | B2 | Power |
| 8 | LGND | LED Ground | F2 | Power |
| 9 | VREF | Voltage Reference | D7 | Analog |
| 10 | PDIN5N | Photodiode 5 N | A7 | Analog |
| 11 | PDIN1P | Photodiode 5 P | A6 | Analog |
| 12 | PDIN6N | Photodiode 6 N | A5 | Analog |
| 13 | PDIN6P | Photodiode 6 P | A4 | Analog |
| 14 | PDIN3N | Photodiode 3 N | B7 | Analog |
| 15 | PDIN3P | Photodiode 3 P | B6 | Analog |
| 16 | PDIN4N | Photodiode 4 N | B5 | Analog |
| 17 | PDIN4P | Photodiode 4 P | B4 | Analog |
| 18 | PDIN2P | Photodiode 2 P | C4 | Analog |
| 19 | PDIN2N | Photodiode 2 N | C5 | Analog |
| 20 | PDIN1P | Photodiode 1 P | C6 | Analog |
| 21 | PDIN1N | Photodiode 1 N | C7 | Analog |
| 22 | LED1 | LED driver 1 | G3 | Analog |
| 23 | LED2 | LED driver 2 | G2 | Analog |
| 24 | LED3 | LED driver 3 | G1 | Analog |
| 25 | LED4 | LED driver 4 | F1 | Analog |
| 26 | LED5 | LED driver 5 | F3 | Analog |
| 27 | LED6 | LED driver 6 | E1 | Analog |
| 28 | LED7 | LED driver 7 | E2 | Analog |
| 29 | LED8 | LED driver 8 | E3 | Analog |
| 30 | ECGIP | ECG Positive Input | E7 | Analog |
| 31 | ECGIN | ECG Negative Input | E6 | Analog |
| 32 | RLD | Right LEG Driver | E5 | Analog |
| 33 | IMPIP | BIA Positive Input | F7 | Analog |
| 34 | IMPIN | BIA Negative Input | F6 | Analog |
| 35 | GSRP | GSR Positive Input | G7 | Analog |
| 36 | GSRN | GSR Negative Input | G6 | Analog |
| 37 | CIP | BIA Injection Positive Output. | G5 | Analog |
| 38 | CIN | BIA Injection Negative Output. | F | Analog |
| 39 | GPIO3 | General-Purpose Input/Output 3 | G4 | Analog |
| 40 | GPIO4 | General-Purpose Input/Output 4 | F4 | Analog |
| 41 | SCLK | SPI Clock INPUT | A1 | Digital |
| 42 | CLKext | External Clock | C2 | Digital |
| 43 | SDA_SDI | I2C / SPI serial data line | B1 | Digital |
| 44 | SH_SDA | Sensor Hub Data | D1 | Digital |
| 45 | CS | Chip Select | C1 | Digital |
| 46 | SH_CK | Sensor Hub Clock | C3 |  |
| 47 | GPIO0 | General-Purpose Input/Output 0 | D2 | Digital |
| 48 | GPIO1 | General-Purpose Input/Output 1 | D3 | Digital |
| 49 | GPIO2 | General-Purpose Input/Output 2 | E4 | Digital |

Table 5: I/O List

# STBIO1: Functional Modes

Different measurement flow can be distinguished according to the operative mode set by the user:

Data Storage Mode

- Set DS_SEL bit in Device_Config_Channel register to 1 and PPG_ELAB to 0 in General_Parameters_PPG

  - This will cut the communication between Main Controller and ADSP

- Analog control Interface samples ECG and BIA signals from ADC, average and stores data sampled in appropriate in FIFO.

- Analog control Interface samples GSR signal from ADC and stores GSR data raw in FIFO.

- Analog control Interface samples PPG signal from ADC, average (according to the user's configuration) and stores data in FIFO.

- Raw data are always available in FIFO after stored. (Except when FIFO FULL occurs)

In this mode, the ADSP is turned off, but it’s possible to turn on the ISPU before the start of a measurement Cycle. In this case, the ISPU shall read the data from FIFO and can communicate with the ADSP.

Normal Mode

- Set DS_SEL bit in Device_Config_Channel register to 0

  - Set PPG_ELAB to 1 if also PPG data shall be elaborated by the ADSP

- Analog control Interface samples of ECG, BIA or GSR from ADC, average and stores data sampled in appropriate registers (SYSTEM REGISTERS from 0x007C to 0x0094) accessible by ADSP and other AHB Master.

- Analog control Interface samples PPG signal from ADC, average (according to the user's configuration) and stores data in FIFO.

- ECG, BIA or GSR data raw in SYSTEM REGISTERS are processed by ADSP.

  - If PPG_ELAB is set to 1, ADSP Shall read PPG Raw Data Directly from FIFO

- Finished processing phase ADSP stores elaborated data in FIFO.

In this mode is possible to turn on the ISPU to work with raw data and/or ADSP elaborated Data.

# Digital Architecture Overview

Figure 5: STBIO1 Digital Engine

TOP Pinout List

| Port name | Direction | POR State of Signal | Type | Description |  |
| --- | --- | --- | --- | --- | --- |
| i_comp_rx_out | input |  | 0 | wire | to u_main_controller_top, Description: "Inserire LS |  |
| o_BIDAC | output |  | 0 | wire [11:0] | from u_scan_out_mux |  |
| o_BDC_PPG | output |  | 0 | wire [7:0] | from u_scan_out_mux |  |
| o_BTIA_gain | output |  | 0 | wire [3:0] | from u_scan_out_mux |  |
| o_PD_PGA | output |  | 1 | wire | from u_scan_out_mux |  |
| o_PD_IDAC | output |  | 1 | wire | from u_scan_out_mux |  |
| o_PD_TIA | output |  | 1 | wire | from u_scan_out_mux |  |
| o_comp_rx_ck | output |  | 0 | wire | from u_scan_out_mux |  |
| BT_IDAC_N | output | 0 | wire [6:0] | from u_scan_out_mux |  |
| BT_IDAC_P | output |  | 0 | wire [9:0] | from u_scan_out_mux |  |
| o_b_cap_idac | output |  | 0 | wire [1:0] | from u_scan_out_mux, Description: IDAC Filter Cap |  |
| o_rx_ch_frame | output |  | 0 | wire [2:0] | from u_scan_out_mux, Description: BMUX_Selection |  |
| DIS_LOOP_TIA | output |  | 0 | wire | from u_scan_out_mux, Description: Disable TIA loop |  |
| T_RX_GPIO1 | output | 0 | wire [1:0] | from u_scan_out_mux, Description: TEST RX signal on |  |
| T_RX_GPIO2 | output |  | 0 | wire [1:0] | from u_scan_out_mux, Description: TEST RX signal on |  |
| TM_SEL_1V8 | output |  | 0 | wire | from u_scan_out_mux, Description: Force PGA input |  |
| o_en_tx_1V2 | output |  | 0 | wire [3:0] | from u_scan_out_mux |  |
| o_tx1_outlab_sel_frame_x_1V2 | output |  | 0 | wire | from u_scan_out_mux |  |
| o_tx2_outlab_sel_frame_x_1V2 | output |  | 0 | wire | from u_scan_out_mux |  |
| o_tx3_outlab_sel_frame_x_1V2 | output |  | 0 | wire | from u_scan_out_mux |  |
| o_tx4_outlab_sel_frame_x_1V2 | output |  | 0 | wire | from u_scan_out_mux |  |
| o_tx_preset_1V2 | output |  | 0 | wire | from u_scan_out_mux |  |
| BtrimIREF_TX_1V2 | output |  | 0 | wire [3:0] | from u_scan_out_mux, Description: TX Ramp Current |  |
| DFT_SEL_1V2 | output | 0 | wire | from u_scan_out_mux, Description: VFEED_1V2 Monitor |  |
| o_curr_sel_tx_frame_x_1V2 | output |  | 0 | wire [7:0] | from u_scan_out_mux, Description: Selezione corrente |  |
| o_Txi_RAMP_1V2 | output |  | 0 | wire [3:0] | from u_scan_out_mux, Description: B_RAMP_1V2<3:0> |  |
| o_ck_chop_imp | output |  | 0 | wire | from u_scan_out_mux |  |
| o_ck_hc2_10k | output |  | 0 | wire | from u_scan_out_mux |  |
| o_ck_m | output |  | 0 | wire | from u_scan_out_mux, Description: inserito new bit |  |
| o_ck_p | output |  | 0 | wire | from u_scan_out_mux, Description: inserito new bit |  |
| o_ck_q | output |  | 0 | wire | from u_scan_out_mux, Description: inserito new bit |  |
| o_short_on | output |  | 0 | wire | from u_scan_out_mux, Description: (codex 1417523 |  |
| o_resh_on | output |  | 0 | wire | from u_scan_out_mux, Description: (codex 1417523 |  |
| o_resl_on | output |  | 0 | wire | from u_scan_out_mux, Description: (codex 1417523 |  |
| o_pd_hc2 | output |  | 1 | wire | from u_scan_out_mux |  |
| o_pd_imp | output |  | 1 | wire | from u_scan_out_mux |  |
| o_rst_hc2 | output |  | 0 | wire | from u_scan_out_mux |  |
| o_rst_imp | output |  | 0 | wire | from u_scan_out_mux |  |
| o_rsti_hc2 | output |  | 0 | wire | from u_scan_out_mux |  |
| o_rsti_imp | output |  | 0 | wire | from u_scan_out_mux |  |
| o_su_hc2 | output |  | 0 | wire | from u_scan_out_mux |  |
| o_sug_imp | output |  | 0 | wire | from u_scan_out_mux |  |
| i_ADC_EOC | input |  | 0 | wire | to u_main_controller_top/u_pad_mux |  |
| i_data_ADC_out | input |  | 0 | wire [15:0] | to u_main_controller_top/u_pad_mux |  |
| o_adc_mux_bio_channel | output |  | 0 | wire [2:0] | from u_scan_out_mux |  |
| o_adc_mux_sel | output |  | 0 | wire | from u_scan_out_mux |  |
| o_gsr_curr_on | output |  | 0 | wire | from u_scan_out_mux |  |
| o_gsr_on | output |  | 0 | wire | from u_scan_out_mux |  |
| o_ADC_en | output |  | 0 | wire | from u_scan_out_mux |  |
| o_ADC_start | output |  | 0 | wire | from u_scan_out_mux |  |
| o_adc_error_cap8_1V2 | output |  | 0 | wire [6:0] | from u_scan_out_mux |  |
| o_adc_error_cap7_1V2 | output |  | 0 | wire [6:0] | from u_scan_out_mux |  |
| o_adc_error_cap6_1V2 | output |  | 0 | wire [6:0] | from u_scan_out_mux |  |
| o_adc_error_cap5_1V2 | output |  | 0 | wire [6:0] | from u_scan_out_mux |  |
| o_adc_error_cap4_1V2 | output |  | 0 | wire [6:0] | from u_scan_out_mux |  |
| o_adc_error_cap3_1V2 | output |  | 0 | wire [6:0] | from u_scan_out_mux |  |
| o_adc_error_cap2_1V2 | output |  | 0 | wire [6:0] | from u_scan_out_mux |  |
| o_adc_error_cap1_1V2 | output |  | 0 | wire [6:0] | from u_scan_out_mux |  |
| o_adc_error_cap0_1V2 | output |  | 0 | wire [6:0] | from u_scan_out_mux |  |
| o_pd_cm_buf | output |  | 1 | wire | from u_scan_out_mux |  |
| o_pd_rld | output |  | 1 | wire | from u_scan_out_mux |  |
| o_pd_avg1 | output |  | 1 | wire | from u_scan_out_mux |  |
| o_pd_avg2 | output |  | 1 | wire | from u_scan_out_mux |  |
| POR1V2_1V2 | input |  | 0 | wire | to u_pad_mux |  |
| clk_16m | input |  | 0 | wire | to u_pad_mux |  |
| clk_64k | input |  | 0 | wire | to u_pad_mux |  |
| START_CLK_16M | input |  | 0 | wire | to u_pad_mux |  |
| o_DIS_RC_VIREF | output |  | 0 | wire | from u_scan_out_mux |  |
| o_EN_LDO1V8 | output |  | 0 | wire | from u_scan_out_mux |  |
| o_PD_CLKF | output |  | 1 | wire | from u_scan_out_mux |  |
| ADC_CAL_CORR_DIS | output |  | 0 | wire | from u_scan_out_mux |  |
| ADC_ERROR_CAP9_DEBUG | output | 0 | wire [7:0] | from u_scan_out_mux |  |
| ADC_SR_ENHANCE_DIS | output |  | 0 | wire | from u_scan_out_mux |  |
| CCK_THRESHN_SEL | output | 0 | wire [3:0] | from u_scan_out_mux |  |
| CCK_THRESHP_SEL | output | 0 | wire [3:0] | from u_scan_out_mux |  |
| CURR_BG_TRIM | output | 8 | wire [3:0] | from u_scan_out_mux |  |
| CURR_PTAT_TRIM | output | 8 | wire [3:0] | from u_scan_out_mux |  |
| DCCK_CURRINJ | output | 0 | wire [1:0] | from u_scan_out_mux |  |
| DCCK_ISINK_IN | output |  | 0 | wire [2:0] | from u_scan_out_mux |  |
| DCCK_ISOURCE_IN | output |  | 0 | wire [2:0] | from u_scan_out_mux |  |
| ECG_PSDRES_DOWN | output |  | 0 | wire [2:0] | from u_scan_out_mux |  |
| ECG_PSDRES_UP | output | 0 | wire [2:0] | from u_scan_out_mux |  |
| EN_DEBUG_ADC_ERROR_CAP9 | output | 0 | wire | from u_scan_out_mux |  |
| EN_ECG_50m | output |  | 0 | wire | from u_scan_out_mux |  |
| EN_GRS_50m | output |  | 0 | wire | from u_scan_out_mux |  |
| EN_IMP_50m | output |  | 0 | wire | from u_scan_out_mux |  |
| EN_RLD_CM | output |  | 0 | wire | from u_scan_out_mux |  |
| EN_SW_IMP | output |  | 1 | wire | from u_scan_out_mux, Description: per selezionare |  |
| GAIN1_BUF | output |  | 0 | wire | from u_scan_out_mux |  |
| GSR_IN_TRIM | output |  | 0 | wire [7:0] | from u_scan_out_mux |  |
| GSR_IP_TRIM | output |  | 0 | wire [7:0] | from u_scan_out_mux |  |
| IMPP_PSDRES_DOWN | output |  | 0 | wire [2:0] | from u_scan_out_mux |  |
| IMPP_PSDRES_UP | output | 0 | wire [2:0] | from u_scan_out_mux |  |
| IMPQ_PSDRES_DOWN | output | 0 | wire [2:0] | from u_scan_out_mux |  |
| IMPQ_PSDRES_UP | output | 0 | wire [2:0] | from u_scan_out_mux |  |
| IMP_CURINJN_TRIM | output | 0 | wire [5:0] | from u_scan_out_mux |  |
| IMP_CURINJP_TRIM | output |  | 0 | wire [5:0] | from u_scan_out_mux |  |
| RLD_CAP_COMP | output |  | 0 | wire [3:0] | from u_scan_out_mux, Description: [1/2/3/…/15/16]*16pF |  |
| RLD_VREF | output |  | 0 | wire [1:0] | from u_scan_out_mux, Description: select 0.9V/ 0.6V |  |
| TRIM_LDOD_I | output |  | 0 | wire [3:0] | from u_scan_out_mux |  |
| TRIM_LDOD_V | output |  | 0 | wire [3:0] | from u_scan_out_mux |  |
| TRIM_OSC_FAST | output |  | 18 | wire [4:0] | from u_scan_out_mux |  |
| TRIM_OSC_SLOW | output |  | 0 | wire [3:0] | from u_scan_out_mux |  |
| TRIM_V1V8 | output |  | 7 | wire [2:0] | from u_scan_out_mux |  |
| VREF_TRIM | output |  | 10 | wire [4:0] | from u_scan_out_mux |  |
| CAPF_IMP_EN | output |  | 1 | wire | from u_scan_out_mux, Description: Enable filtro per |  |
| ECG0_GAIN_HC_0 | output |  | 1 | wire | from u_scan_out_mux |  |
| ECG0_GAIN_HC_1 | output |  | 1 | wire | from u_scan_out_mux |  |
| EN_EXT_CAP | output |  | 0 | wire | from u_scan_out_mux, Description: Mantenuta opzione |  |
| EN_ECG0_0P05HP | output |  | 0 | wire | from u_scan_out_mux, Description: aggiunto bit per |  |
| EN_ECG12_0P05HP | output |  | 0 | wire | from u_scan_out_mux, Description: aggiunto bit per |  |
| GAIN_IMP | output |  | 1 | wire [1:0] | from u_scan_out_mux |  |
| GIMP_DC | output |  | 1 | wire | from u_scan_out_mux |  |
| o_gsr_switchon | output |  | 0 | wire | from u_scan_out_mux, Description: spostato il bit |  |
| GSR_GAIN_SEL | output |  | 0 | wire [1:0] | from u_scan_out_mux |  |
| GSR_GAIN_X1 | output |  | 0 | wire | from u_scan_out_mux |  |
| IGSR_X10 | output |  | 0 | wire | from u_scan_out_mux |  |
| IMP_CURRINJ_CUR | output |  | 2 | wire [1:0] | from u_scan_out_mux |  |
| IMP_CURRINJ_x10 | output |  | 0 | wire | from u_scan_out_mux |  |
| SEL_GSR_CURR | output |  | 3 | wire [1:0] | from u_scan_out_mux |  |
| o_ck_bg | output |  | 0 | wire | from u_scan_out_mux |  |
| o_imp_curinj_pd | output |  | 1 | wire | from u_scan_out_mux |  |
| o_pd_p | output |  | 1 | wire | from u_scan_out_mux |  |
| o_pd_n | output |  | 1 | wire | from u_scan_out_mux |  |
| o_adc_error_cap9 | output |  | 0 | wire [7:0] | from u_scan_out_mux |  |
| o_en_bufxbio | output |  | 0 | wire | from u_scan_out_mux |  |
| o_ldo_ext | output |  | 0 | wire | from u_scan_out_mux |  |
| o_clk_ext_div | output |  | 0 | wire | from u_scan_out_mux |  |
| o_adc_test_cap | output |  | 0 | wire [9:0] | from u_scan_out_mux |  |
| if_cs_en | output |  |  | wire | from u_level_shifter_sel.if_cs_en_ls_out |  |
| if_cs_in | input |  |  | wire | to u_pad_mux |  |
| if_cs_out | output |  |  | wire | from u_level_shifter_sel.if_cs_out_ls_out |  |
| if_cs_tud | output |  | 1 | wire | from u_level_shifter_sel.if_cs_tud_ls_out, Description: |  |
| if_gpio0_en | output |  | wire | from u_level_shifter_sel.if_gpio0_en_ls_out |  |
| if_gpio0_in | input |  |  | wire | to u_pad_mux/u_pmu.if_sh_ck_in |  |
| if_gpio0_out | output |  |  | wire | from u_level_shifter_sel.if_gpio0_out_ls_out |  |
| if_gpio0_tud | output |  | 1 | wire | from u_level_shifter_sel.if_gpio0_tud_ls_out |  |
| if_gpio1_en | output |  | wire | from u_level_shifter_sel.if_gpio1_en_ls_out |  |
| if_gpio1_in | input |  |  | wire | to u_pad_mux |  |
| if_gpio1_out | output |  |  | wire | from u_level_shifter_sel.if_gpio1_out_ls_out |  |
| if_gpio1_tud | output |  | 1 | wire | from u_level_shifter_sel.if_gpio1_tud_ls_out |  |
| if_gpio2_in | input |  | wire | to u_pad_mux |  |
| if_gpio2_en | output |  |  | wire | from u_level_shifter_sel.if_gpio2_en_ls_out |  |
| if_gpio2_out | output |  | 1 | wire | from u_level_shifter_sel.if_gpio2_out_ls_out |  |
| if_gpio2_tud | output |  | 1 | wire | from u_level_shifter_sel.if_gpio2_tud_ls_out |  |
| if_gpio3_en | output |  | wire | from u_level_shifter_sel.if_gpio3_en_ls_out |  |
| if_gpio3_in | input |  |  | wire | to u_pad_mux, Description: decidere se mantenere l'IN |  |
| if_gpio3_out | output |  | 1 | wire | from u_level_shifter_sel.if_gpio3_out_ls_out |  |
| if_gpio3_tud | output |  | 1 | wire | from u_level_shifter_sel.if_gpio3_tud_ls_out |  |
| if_gpio4_en | output |  | wire | from u_level_shifter_sel.if_gpio4_en_ls_out |  |
| if_gpio4_in | input |  |  | wire | to u_pad_mux, Description: decidere se mantenere l'IN |  |
| if_gpio4_out | output |  | 1 | wire | from u_level_shifter_sel.if_gpio4_out_ls_out |  |
| if_gpio4_tud | output |  | 1 | wire | from u_level_shifter_sel.if_gpio4_tud_ls_out |  |
| if_sclk_en | output |  | wire | from u_level_shifter_sel.if_sclk_en_ls_out |  |
| if_sclk_in | input |  |  | wire | to u_pad_mux |  |
| if_sclk_out | output |  | 1 | wire | from u_level_shifter_sel.if_sclk_out_ls_out |  |
| if_sclk_tud | output |  | 1 | wire | from u_level_shifter_sel.if_sclk_tud_ls_out |  |
| if_sda_sdi_en | output |  | wire | from u_level_shifter_sel.if_sda_sdi_en_ls_out |  |
| if_sda_sdi_in | input |  |  | wire | to u_pad_mux |  |
| if_sda_sdi_out | output |  | 1 | wire | from u_level_shifter_sel.if_sda_sdi_out_ls_out |  |
| if_sda_sdi_tud | output |  | 1 | wire | from u_level_shifter_sel.if_sda_sdi_tud_ls_out |  |
| if_sh_ck_en | output |  | wire | from u_level_shifter_sel.if_sh_ck_en_ls_out |  |
| if_sh_ck_in | input |  |  | wire | to u_pad_mux |  |
| if_sh_ck_out | output |  |  | wire | from u_level_shifter_sel.if_sh_ck_out_ls_out |  |
| if_sh_ck_tud | output |  | 1 | wire | from u_level_shifter_sel.if_sh_ck_tud_ls_out |  |
| if_sh_sda_en | output |  | wire | from u_level_shifter_sel.if_sh_sda_en_ls_out |  |
| if_sh_sda_in | input |  |  | wire | to u_pad_mux |  |
| if_sh_sda_out | output |  | 1 | wire | from u_level_shifter_sel.if_sh_sda_out_ls_out |  |
| if_sh_sda_tud | output |  | 1 | wire | from u_level_shifter_sel.if_sh_sda_tud_ls_out |  |
| o_trim_ldo_d | output | 0 | wire [1:0] | from u_level_shifter_sel.o_trim_ldo_d_ls_out |  |
| ADC_BUFF_REFRESH_DIS | output |  | 0 | wire | from u_scan_out_mux |  |
| ADC_EXTRAPERIOD_DIS | output |  | 0 | wire | from u_scan_out_mux |  |
| ADC_INPUT_FROM_GPIO | output |  | 0 | wire | from u_scan_out_mux |  |
| ADC_REF_FROM_GPIO | output |  | 0 | wire | from u_scan_out_mux |  |
| AFE_SPARE_0_1V2 | output |  | 0 | wire [7:0] | from u_scan_out_mux |  |
| AFE_SPARE_1 | output |  | 0 | wire [7:0] | from u_scan_out_mux, Description: modificato l'utilizzo |  |
| DIS_LDOD_HP | output |  | 0 | wire | from u_scan_out_mux |  |
| DIS_LDOD_IP | output |  | 0 | wire | from u_scan_out_mux |  |
| EN_LDOD_FIFO | output |  | 0 | wire | from u_scan_out_mux |  |
| EN_LDOD_STREDL | output |  | 0 | wire | from u_scan_out_mux |  |
| OTP_SPARE | output |  | 00007F00 | wire [31:0] | from u_scan_out_mux, Description: i PMB_EXT sono collegati |  |
| OVERCURR_N_IN_i | input |  | 0 | wire | to u_regmap_otp |  |
| OVERCURR_P_IN_i | input |  | 0 | wire | to u_regmap_otp |  |
| O_DCCK_N_i | input |  | 0 | wire | to u_pad_mux/u_regmap_otp/u_regmap_otp.DCCK_N_IRQ_i/u_N_not_logic.A, |  |
| O_DCCK_P_i | input |  | 0 | wire | to u_pad_mux/u_regmap_otp/u_regmap_otp.DCCK_P_IRQ_i/u_P_not_logic.A, |  |
| t_en_gpio1 | output |  | 0 | wire | from u_scan_out_mux, Description: spostato il bit |  |
| t_en_gpio2 | output |  | 0 | wire | from u_scan_out_mux, Description: spostato il bit |  |
| t_en_gpio3 | output |  | 0 | wire | from u_scan_out_mux |  |
| t_en_gpio4 | output |  | 0 | wire | from u_scan_out_mux |  |
| t_ecg0_out_pre_buf | output |  | 0 | wire | from u_scan_out_mux |  |
| t_offset_gpio12 | output |  | 0 | wire | from u_scan_out_mux |  |
| t_in_bio_buf34 | output |  | 0 | wire | from u_scan_out_mux |  |
| t_ecg1_out_pre_bufp_3 | output |  | 0 | wire | from u_scan_out_mux |  |
| t_ecg1_out_pre_bufn_4 | output |  | 0 | wire | from u_scan_out_mux |  |
| t_ecg2_out_pre_bufn_3 | output |  | 0 | wire | from u_scan_out_mux |  |
| t_ecg2_out_pre_bufp_4 | output |  | 0 | wire | from u_scan_out_mux |  |
| T_CM_HLT_2 | output |  | 0 | wire | from u_scan_out_mux |  |
| T_CM_RLD_1 | output |  | 0 | wire | from u_scan_out_mux |  |
| T_ECG0_AC_1 | output |  | 0 | wire | from u_scan_out_mux |  |
| T_ECG0_DC_2 | output |  | 0 | wire | from u_scan_out_mux |  |
| T_EXT_HPF_ECG0 | output |  | 0 | wire | from u_scan_out_mux |  |
| T_FORCE_1V2 | output |  | 0 | wire | from u_pad_mux.o_t_force_1v2 |  |
| T_IBG_1u_2 | output | 0 | wire | from u_scan_out_mux |  |
| T_IMPP_AC_1 | output |  | 0 | wire | from u_scan_out_mux |  |
| T_IMPP_DC_2 | output |  | 0 | wire | from u_scan_out_mux |  |
| T_IMPQ_AC_2 | output |  | 0 | wire | from u_scan_out_mux |  |
| T_IMPQ_DC_1 | output |  | 0 | wire | from u_scan_out_mux |  |
| T_IN_BIO_BUF | output |  | 0 | wire | from u_scan_out_mux |  |
| T_IPTAT_1u_1 | output |  | 0 | wire | from u_scan_out_mux |  |
| T_OP55_1 | output |  | 0 | wire | from u_scan_out_mux |  |
| T_OP90_2 | output |  | 0 | wire | from u_scan_out_mux |  |
| T_OUT_BIO_BUF | output |  | 0 | wire | from u_scan_out_mux |  |
| T_OUT_GSR | output |  | 0 | wire | from u_scan_out_mux |  |
| T_OUT_PPG_BUF | output |  | 0 | wire | from u_scan_out_mux |  |
| T_SENSE_1V2 | output |  | 0 | wire | from u_scan_out_mux |  |
| T_SENSEIBG_1V2 | output | 0 | wire | from u_scan_out_mux |  |
| T_CM_HLT_1 | output |  | 0 | wire | from u_scan_out_mux |  |
| T_SENSEVBG_1V2 | output |  | 0 | wire | from u_scan_out_mux |  |
| o_end_calibration_1V2 | output |  | 0 | wire | from u_scan_out_mux |  |
| pd_osc_clk32k | output |  | 0 | wire | from u_scan_out_mux, Description: se si seleziona |  |
| o_pd_buf_gpio1 | output |  | 1 | wire | from u_scan_out_mux |  |
| o_pd_buf_gpio2 | output |  | 1 | wire | from u_scan_out_mux |  |
| EN_CLK_EXT | output |  | 0 | wire | from u_scan_out_mux |  |
| WHO_AM_I | input |  | D7 | wire [7:0] | to u_regmap_otp.DEV_ID_i, Description: ? OK ? |  |
| if_sclk_i3c_in | input |  |  | wire | to u_pad_mux |  |
| if_sda_sdi_i3c_in | input |  |  | wire | to u_pad_mux |  |
| VREF | input |  |  | wire | to u_otp |  |
| IREF | input |  |  | wire | to u_otp |  |
| OTP_IREF_WAKEUP | output |  | 0 | wire | from u_scan_out_mux, Description: Enable Mirror OTP |  |

Table 6: Digital I/O List

# Power Domains

Introduction

This document paragraph describes the Power Domain architecture implemented in the STBIO1. The goal is to provide a detailed overview of the power domain partitioning, their functionalities, control modes, and interactions between the various functional blocks.

Definitions

Power Domain (PD): A set of logic blocks sharing the same power supply, which can be powered on/off independently.

Always-On Domain (AON): Domain that is always powered, even in low-power modes.

Retention Domain: Domain that retains register state even when the main logic is powered down.

Switchable Domain: Domain that can be completely powered off to save energy.

Power Domain Architecture

| Domain Name | Included Block(s) | Domain Type | Voltage [V] | Control Mode | Notes |
| --- | --- | --- | --- | --- | --- |
| PD_TOP1V2 | ADSP, ca_fifo_bist_c, ca_qk_bist_c, ca_qk_rom_bist, multi_channel_fifo, i2c_spi_slave, main_controller_top, senshub_mst_i2c, pad_mux, pmu, stbio1_regmap, scan_out_mux, ta_stbio1_bist_ctlr, xbar_afe | Always-On | 1.2 | HW | Always active |
| PD_STREDL | STREDL | Switchable | 1.2 | SW | Can be powered down in idle |
| PD_TOP3V3 | H9A_MEM_OTP_ PUMP_85AL05_2 3A | Always-On | 2.7÷4.7V | HW | Always active |
| PD_TOPIO | level_shifter_sel | Always-On | 1.8V÷3.6V | HW | Always active |

Power Domain Descriptions

### PD_TOP1V2

- Function: Ensures continuity of critical functions (Data Storage Mode, Sensor-Hub acquisition, FIFO read and write data, Elaboration Mode with ADSP only).

- Characteristics: Never powered down.

- Control: Managed by hardware.

### PD_STREDL

- Function: Powers the STREDL core and associated cache.

- Characteristics: Can be powered down when STREDL is not in use.

- Control: Managed via software (power management driver)(see 12.3.1)

### PD_TOP3V3

- Function: power supply to the OTP memory block.

- Characteristics: Never powered down. It provides 4.7V of power supply to the OTP memory block during programming (write) operations

- Control: Managed by hardware.

### PD_TOPIO

- Function: Powers communication PAD

- Characteristics: Never powered down.

- Control: Managed by hardware.

# Interconnect

Figure 6: STBIO1 Digital Scheme

XBAR Connection Matrix

| Destination Source | Regmap | SENSOR HUB | ISPU | FIFO | ADSP |
| --- | --- | --- | --- | --- | --- |
| Main Controller | [DDS_STBIO1_0201] |  |  | [DDS_STBIO1_0202] | [DDS_STBIO1_0219] |
| ADSP | [DDS_STBIO1_0203] | [DDS_STBIO1_0204] |  | [DDS_STBIO1_0206] |  |
| I2C_SPI_AHB | [DDS_STBIO1_0207] | [DDS_STBIO1_0208] | [DDS_STBIO1_0209] | [DDS_STBIO1_0210] | [DDS_STBIO1_0221] |
| SENSOR HUB |  |  |  | [DDS_STBIO1_0211] |  |
| ISPU | [DDS_STBIO1_0212] | [DDS_STBIO1_0213] |  | [DDS_STBIO1_0215] | [DDS_STBIO1_0222] |
| ISPU debug | [DDS_STBIO1_0216] | [DDS_STBIO1_0217] | [DDS_STBIO1_0218] | [DDS_STBIO1_0220] | [DDS_STBIO1_0223] |

Table 7: XBAR Connection Matrix

[DDS_STBIO1_XBAR_01] Assumption

Every master and slave supports AHBLite and none supports burst communication

[End]

XBAR Memory Mapping

| AHB base start address | AHB base end address | Address block | Direct Access | Req_ID |
| --- | --- | --- | --- | --- |
| 0x0_0000 | 0x0_7FFF | ISPU CODE RAM 32KB | N.A. | [DDS_STBIO1_10100] |
| 0x0_8000 | 0x3_FFFF | RESERVED | N.A. | [DDS_STBIO1_10101] |
| 0x4_0000 | 0x4_5FFF | ISPU DATA RAM 24KB | N.A. | [DDS_STBIO1_10102] |
| 0x4_6000 | 0x7_FFFF | RESERVED | N.A. | [DDS_STBIO1_10103] |
| 0x8_0000 | 0x8_010F | ISPU SYSTEM REGISTERS | N.A. | [DDS_STBIO1_10104] |
| 0x8_0110 | 0xB_FFFF | RESERVED |  | [DDS_STBIO1_20000] |
| 0xC_0000 | 0xC_0800 | ADSP PROG ROM | N.A. | [DDS_STBIO1_10105] |
| 0xC_0804 | 0xC_0FFF | RESERVED |  | [DDS_STBIO1_20001] |
| 0xC_1000 | 0xC_1DFF | ADSP DATA MEMORY | N.A. | [DDS_STBIO1_10106] |
| 0xC_1E00 | 0xC_1E20 | ADSP REGISTERS | N.A. | [DDS_STBIO1_10107] |
| 0xC_1E24 | 0xC_3FFF | NOT USED |  |  |
| 0xC_4000 | 0xC_4FFF | FIFO RAM MEMORY | N.A. | [DDS_STBIO1_10108] |
| 0xC_5000 | 0xC_5060 | FIFO REGISTERS | N.A. | [DDS_STBIO1_10109] |
| 0xC_5064 | 0xC_5FFF | NOT USED |  |  |
| 0xC_6000 | 0xC_6FFF | SENSOR HUB | N.A. | [DDS_STBIO1_10110] |
| 0xC_7000 | 0xC_7FFF | NOT USED |  |  |
| 0xC_8000 | 0xC_DFFF | AFE SYSTEM REGISTERS | N.A. with exception (see 13.4) | [DDS_STBIO1_10111] |
| 0xE_0000 | 0xF_FFFF | STOP CLOCK (Reserved) | N.A. | [DDS_STBIO1_10112] |

Table 8: Memory Map

# STBIO1 Operating Modes

Digital blocks status for different operating modes is reported in table, where ON indicates the clock is running and OFF that it is gated:

|  | PMU | ADSP | Regmap | FIFO_CTRL | Sensor hub | Main CTRL | ISPU | Req_ID |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Power Down | On | Off | Off | Off | Off | On | Off | DDS_STBIO1_10200 |
| Boot | On | On | On | Off | Off | On | Off | DDS_STBIO1_10201 |
| Configuration Phase | On | Off | On | Off | Off | On | Off | DDS_STBIO1_10202 |
| Operative data storage mode | On | Off | Off | On | On | On | On (optionally) | DDS_STBIO1_10203 |
| Operative elaboration mode | On | On | Off | On | On | On | On (optionally) | DDS_STBIO1_10204 |
| Sleep | On | Off | Off | On (Optionally) | Off | On | On (optionally) | DDS_STBIO1_10205 |

Table 9: Operating modes Table

# Modules

Instantiations

- u_pmu: pmu

- u_pad_mux: pad_mux

- u_regmap_otp: stbio1_regmap

- u_masterblaze_ahb: senshub_mst_i2c

- u_xbar_afe: xbar_afe: interconnect

- u_ADSP: ADSP

- u_fifo: Fifo Controller

- u_ispu: ispu

- u_i2c_spi_slave: i2c_spi_slave

- u_ca_fifo_bist_c: FIFO RAM (Hard Macro)

- u_ca_qk_bist_c: ADSP RAM (Hard Macro)

- u_ca_qk_rom_bist: ADSP ROM (Hard Macro)

- u_psw_ctrl: ISPU Power Island

- u_ta_afe_bist_crtl: STBIO1 Bist Controller (Hard Macro)

- u_otp: otp (Hard Macro)

- u_main_controller_top: main_controller_top

### PMU

#### PMU TOP Description

PMU oversees the Device Start-Up phase, the clock generation to the Digital and Analog Domain and Reset Sync to the different Clock Domain.

#### PMU TOP I/O List

| Port name | Direction | Type | Description |
| --- | --- | --- | --- |
| clk_16m | input | wire | Clock 16 MHz from analog ring oscillators |
| clk_64k | input | wire | Clock 64kHz from analog ring oscillators |
| POR1V2_1V2 | input | wire | POR Signal |
| START_CLK_16M | input | wire | Clock ready for 16MHz |
| PDClockF | input | wire | Power Down for 16MHz Ring Oscillator from main controller |
| i_bio_divider | input | wire | Enable 32kHz geneartion from 16MHz when a Bio-Channel is activated |
| EN_LDO | input | wire | enable LDO from main controller |
| PD_CLK_REG | input | wire | power down 16MHz from Regmap |
| EN_LDO1V8_REG | input | wire | Enable LDO from regmap |
| bist_mode | input | wire | enable bist mode |
| debug_mode | input | wire | enable debug |
| scan_mode | input | wire | enable scan mode |
| scan_enable | input | wire | start scan mode |
| scan_clk | input | wire | scan clk |
| scan_rstn | input | wire | scan rst |
| clk_bistctlr | input | wire | bist controller and collar clock |
| rst_bistctlr_n | input | wire | bist controller and collar resetn |
| clk_ext_en | input | wire | enable external clock from pad |
| clk_ext_slow | input | wire | external clock 32kHz |
| i_en_sh_clk | input | wire | enable sensor hub clock |
| i_senshub_clk_sel | input | wire [2:0] | selector for sensor hub clock |
| i_test_clk_fast | input | wire | enable test and trimming for 16MHz clock |
| i_half_charge_ldo | input | wire | enable the low power mode for ldo when only ppg is selected |
| mask_en_ldo_pdclkf | input | wire | mask the control of pdf and ldo control during boot |
| clk_en_s0 | input | wire | clock gating logic inside stredl_afe_top |
| clk_en_s1 | input | wire | if present should be HSEL STMC AHB Master |
| clk_en_s2_i2c | input | wire | if present should be HSEL STMC AHB Master |
| clk_en_s2_on | input | wire | clk gating ON from AHB I2C/SPI Interface |
| clk_en_s2_off | input | wire | clk gating OFF from AHB I2C/SPI Interface |
| clk_en_s3 | input | wire | clk gating from AHB Master Main Controller |
| clk_en_s4 | input | wire | clock gating logic inside stredl_afe_top |
| clk_en_s5 | input | wire | clock gating logic inside masterblaze_ahb |
| clk_en_m0 | input | wire | clock gating logic inside stredl_afe_top |
| clk_en_m1 | input | wire | HSEL M1 from xbar to STMC AHB Slave |
| clk_en_m2 | input | wire | HSEL M2 from xbar to FIFO |
| clk_en_m22 | input | wire | HSEL M2 from xbar to MASTERBLAZE |
| clk_en_m3 | input | wire | HSEL M3 from xbar to Regmap |
| iso_i | input | wire | from register |
| vdden_i | input | wire | enable ISPU Power Island |
| i_start_operative_16MHz | input | wire | enable the lead of check |
| main_ctrl_sw_resetn | input | wire | soft reset for main controller |
| stmc_sw_resetn | input | wire | adsp soft reset |
| fifo_sw_resetn | input | wire | fifo soft reset |
| masterblaze_sw_resetn | input | wire | sensor hub soft reset |
| regmap_sw_resetn | input | wire | regmap soft reset |
| adsp_run | input | wire | adsp is running |
| adsp_xbar_sw_en | input | wire | adsp clk ahb |
| i_en_clk_gsr_16M | input | wire | enable the clock for gsr digital circuit |
| i_en_ck_m_p_q | input | wire | enable quadrature and phase clock |
| i_en_clk_mod_debug | input | wire | enable the clock modulation debug |
| i_en_chop_ref | input | wire | enable chopper clock |
| i_en_ck_chop_imp | input | wire | enable chopper clock for bia |
| i_en_ck_hc2_10k | input | wire | enable health channel clock |
| i_freq_bia_m_p_q | input | wire [2:0] | frequency selector for phase and quadrature |
| i_en_clk_16MHz_sleep | input | wire | enable clock in SLEEP mode when a bio-channel is selected |
| EN_ADC_TEST_LOW_NOISE | input | wire | enable adc test low noise |
| EN_ADC_TEST_FAST | input | wire | enable adc test fast |
| if_sh_ck_in | input | wire | sensor hub clock interface |
| o_EN_LDO1V8 | output | wire | enable ldo to analog domain |
| o_PD_CLKF | output | wire | power down 16MHz clock to analog domain |
| o_CLK_16M_READY | output | wire | clock ready to the digital domain |
| o_DIS_RC_VIREF | output | wire | dis rc viref to the analog domain |
| pd_osc_clk32k | output | wire | power down for clk 64kHz |
| tst_clk_32k | output | wire | clock slow 32k to main_controller |
| tst_clk_16m | output | wire | clock fast 16M |
| tst_clk_bistctlr | output | wire | test clock for bist ctrl |
| tst_rst_bistctlr_n | output | wire | test reset for bist ctrl |
| clk_stredl | output | wire | clock gating logic inside stredl_afe_top |
| clk_xbar | output | wire | xbar clock |
| clk_intf | output | wire | interface clock |
| clk_main_ctrl | output | wire | clock for main controller |
| clk_stmc | output | wire | adsp clock |
| clk_stmc_ahb | output | wire | adsp clk elaboration |
| clk_fifo | output | wire | fifo clock |
| clk_masterblaze_m | output | wire | sensor hub master clock |
| clk_masterblaze_s | output | wire | sensor hub slave clock |
| clk_regmap | output | wire | regmap clock |
| o_clk_ext_en | output | wire | enable external clock |
| resetn_stredl | output | wire | reset mngmt inside stredl_afe_top |
| resetn_xbar | output | wire | xbar reset |
| resetn_intf | output | wire | reset interface |
| resetn_main_ctrl | output | wire | reset main controller |
| resetn_32k_main_ctrl | output | wire | reset 32kHz domain main controller |
| resetn_stmc | output | wire | reset adsp |
| resetn_fifo | output | wire | reset FIFO |
| resetn_masterblaze | output | wire | reset sensor hub |
| resetn_regmap | output | wire | reset regmap |
| o_trimming_clk_fast | output | wire | trimming output clock fast |
| iso_o | output | wire | left open . to be connected in synthesis |
| vdden_o | output | wire | to be connected to psw_ctrl.VDDIEN |
| o_ck_bg | output | wire | bg clock to analog domain |
| o_sh_clk | output | wire | sensor hub clock |
| o_ck_chop_imp | output | wire | chopper bia clock to analog domain |
| o_ck_m | output | wire | modulation clock to analog domain |
| o_ck_hc2_10k | output | wire | healt channel 10kHz clock to analog domain |
| o_ck_p | output | wire | phase clock to analog domain |
| o_ck_q | output | wire | quadrature clock to analog domain |
| o_clk_16M_mc | output | wire | 16MHz clock for Main Controller |
| o_clk_dft_adc | output | wire | 16MHz clock for ADC DFT |
| o_clk_gsr_16M | output | wire | gsr clock to digital domain |
| o_ADC_clk_serial_dft | output | wire | adc serial clock in dft |
| o_sh_clk_gate | output | wire | status clk reg clk sh 16Mhz |

Table 10: PMU I/O List

#### PMU Peculiar Requirements

Figure 7: Device Start-Up Phase

[DDS_STBIO1_0102] Definition:

w_EN_LDO and w_PDClockF are as follows:

Figure 8: Mask Cell for LDO and Fast Ring Oscillator

w_EN_LDO_sync32 and w_PDClockF_sync32 are the relative 32kHz synchronized versions. [TO: IPOS_PMU] [END]

[DDS_STBIO1_0103] Definition:

clk_64k_divided (32kHz) is i_clk_64k (64kHz) divided by two. [TO: IPOS_PMU] [END]

[DDS_STBIO1_0104] Requirement:

When the device is turned on, i_POR1V2_1V2 shall be high. [TO: IPOS_PMU] [END]

[DDS_STBIO1_0105] Requirement:

When i_POR1V2_1V2 is low, o_ EN_LDO1V8 shall be low. [TO: IPOS_PMU] [END]

[DDS_STBIO1_0106] Requirement:

When i_POR1V2_1V2 is low, o_PD_CLKF shall be high. [TO: IPOS_PMU] [END]

[DDS_STBIO1_0107] Requirement:

When i_POR1V2_1V2 is low, o_CLK_16M_READY shall be low. [TO: IPOS_PMU] [END]

[DDS_STBIO1_0108] Requirement:

When i_POR1V2_1V2 is low, o_DIS_RC_VIREF shall be high. [TO: IPOS_PMU] [END]

[DDS_STBIO1_0109] Requirement:

When i_POR1V2_1V2 rises, after at least 10 cycles of i_clk_64k (64kHz), o_ EN_LDO1V8 shall be high and the clk_64k_divided is considered stable. [TO: IPOS_PMU] [END]

[DDS_STBIO1_0110] Requirement:

When clk_64k_divided is stable, after at least 352 cycles of clk_64k_divided (32kHz), o_DIS_RC_VIREF shall be low. [TO: IPOS_PMU] [END]

[DDS_STBIO1_0111] Requirement:

When i_mask_en_ldo_pdclkf_reg and o_EN_LDO1V8 are set to 1, o_PD_CLKF shall be low after at least 40 cycles of the clk_64k_divided. [TO: IPOS_PMU] [END]

[DDS_STBIO1_0112] Requirement:

When i_START_CLK_16M is high, after at least 20 cycles of i_clk_16M (16MHz), o_CLK_16M_READY shall be high. [TO: IPOS_PMU] [END]

[DDS_STBIO1_0113] Requirement:

When i_START_CLK_16M is high for the first time, after at least 20 cycles of i_clk_16M (16MHz), o_main_resetn shall be high. [TO: IPOS_PMU] [END]

#### Clocks and Reset

Clocks scheme is shown in the below figure

Figure 9: Clocks Scheme

Clocks scheme is shown in the above figure

| Req_ID | Source | Target | Path Delay | Clock Gating | Analog Control and Description | F Max |
| --- | --- | --- | --- | --- | --- | --- |
| Requirements |  |  |  |  | Definition |  |
| DDS_STBIO1_1020 | stbio1_top.clk_16m | stbio1_top.dig_wrapper_LV.u_pmu.clk_16m |  | No | Pwr_down controlled by pmu | 16 MHz |
| DDS_STBIO1_1021 | stbio1_top.clk_64k | stbio1_top.dig_wrapper_LV.u_pmu.clk_64k |  | No | No | 64 kHz |
| DDS_STBIO1_1027 | stbio1_top.dig_wrapper_LV.u_pmu.tst_clk_32k | stbio1_top.dig_wrapper_LV.u_main_controller_top.i_clk_32kHz |  | No | No | 32 kHz |
| DDS_STBIO1_1022 | stbio1_top.dig_wrapper_LV.u_pmu.clk_16m_dft | stbio1_top.dig_wrapper_LV.u_ADSP.stmc_clk | 1 | stbio1_top.dig_wrapper_LV.u_pmu.adsp_run ¦ stbio1_top.dig_wrapper_LV.u_pmu.all_test_mode ¦ ~stbio1_top.dig_wrapper_LV.u_pad_mux.scan_enable | No | 16 MHz |
| DDS_STBIO1_1023 | stbio1_top.dig_wrapper_LV.u_pmu.w_sh_clk | stbio1_top.dig_wrapper_LV.u_masterblaze_ahb.sys_clk | 1 | stbio1_top.dig_wrapper_LV.u_pmu.i_en_sh_clk ¦ stbio1_top.dig_wrapper_LV.u_pmu.all_test_mode ¦ ~stbio1_top.dig_wrapper_LV.u_pad_mux.scan_enable | No | 16 MHz |
| DDS_STBIO1_1024 | stbio1_top.dig_wrapper_LV.u_pmu.clk_16m_dft | stbio1_top.dig_wrapper_LV.u_xbar_afe.hclk | 1 | stbio1_top.dig_wrapper_LV.u_pmu.clk_xbar_en ¦ stbio1_top.dig_wrapper_LV.u_pmu.all_test_mode ¦ ~stbio1_top.dig_wrapper_LV.u_pad_mux.scan_enable | No | 16 MHz |
| DDS_STBIO1_2024 | stbio1_top.dig_wrapper_LV.u_pmuclk_xbar | stbio1_top.dig_wrapper_LV.u.fifo.HCLK | 1 | stbio1_top.dig_wrapper_LV.u_pmu.clk_en_m2 ¦ stbio1_top.dig_wrapper_LV.u_pmu.all_test_mode ¦ stbio1_top.dig_wrapper_LV.u_pmu.clk_en_m2_reg ¦ ~stbio1_top.dig_wrapper_LV.u_pad_mux.scan_enable | No | 16 MHz |
| DDS_STBIO1_2025 | stbio1_top.dig_wrapper_LV.u_pmu.clk_stredl | stbio1_top.dig_wrapper_LV.u_ispu.clk |  | No | Pwr_down controlled by pmu | 16 MHz |
| DDS_STBIO1_2026 | stbio1_top.dig_wrapper_LV.u_pmu.clk_xbar | stbio1_top.dig_wrapper_LV.u_regmap_otp.hclk | 1 | stbio1_top.dig_wrapper_LV.u_pmu.clk_en_m3 ¦ stbio1_top.dig_wrapper_LV.u_pmu.all_test_mode ¦ stbio1_top.dig_wrapper_LV.u_pmu.clk_en_m3_reg ¦ stbio1_top.dig_wrapper_LV.u_pmu.clk_en_s3 ¦ ~stbio1_top.dig_wrapper_LV.u_pad_mux.scan_enable | No | 16 MHz |
| DDS_STBIO1_1025 | u_stbio1_top.u_pmu.u_device_startup.tst_clk_32k | stbio1_top.dig_wrapper_LV.tst_clk_32k |  | No | No | 32 kHz |
| DDS_STBIO1_1026 | stbio1_top.dig_wrapper_LV.u_pmu.tst_clk_16m | stbio1_top.dig_wrapper_LV.u_main_controller_top.i_clk_16MHz | 1 | stbio1_top.dig_wrapper_LV.u_pmu.i_en_clk_16MHz_sleep ¦ stbio1_top.dig_wrapper_LV.u_pmu.scan_mode ¦ ~stbio1_top.dig_wrapper_LV.u_pad_mux.scan_enable | No | 16 MHz |

Table 11: Clock Recap Table

Reset scheme is shown in the below figure

Figure 10: STBIO1 Reset Scheme

| Req_ID | Source | Target |
| --- | --- | --- |
| DDS_STBIO1_1050 | u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_stmc | u_stbio1_top.dig_wrapper_LV.u_ADSP.HRESETn |
| DDS_STBIO1_1051 | u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_masterblaze | u_stbio1_top.dig_wrapper_LV.u_masterblaze_ahb.HRESETn |
| DDS_STBIO1_1052 | u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_regmap | u_stbio1_top.dig_wrapper_LV.u_regmap_otp.hresetn |
| DDS_STBIO1_2052 | stbio1_top.dig_wrapper_LV.u_pmu.u_device_startup.u_por_main_resetn.RST_N_FF[0] | u_stbio1_top.dig_wrapper_LV.u_xbar_afe.hresetn |
| DDS_STBIO1_2053 | u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_regmap | u_stbio1_top.dig_wrapper_LV.u_ispu.main_rst_n |
| DDS_STBIO1_1053 | u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_fifo | u_stbio1_top.dig_wrapper_LV.u_multi_channel_fifo_top.HRESETn |
| DDS_STBIO1_1054 | u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_main_ctrl | u_stbio1_top.dig_wrapper_LV.u_main_controller_top.i_rstn |
| DDS_STBIO1_2054 | u_stbio1_top.dig_wrapper_LV.u_pmu.resetn_32k_main_ctrl | u_stbio1_top.dig_wrapper_LV.u_main_controller_top.i_rstn_sync_32 |

Table 12: STBIO1 Functional reset Summary table

### PAD Mux

In the following table are reported the Pad Configuration for each operative mode of the device. The

BIST Mode, Scan Mode and Debug Mode are reserved for ST Testing Purpose

#### PAD MUX TOP I/O List

| Port name | Direction | Type | Description |
| --- | --- | --- | --- |
| i_write_protection_en | input |  | write_protection_en OTP regmap enable from regmap |
| o_write_protection_en | output |  | write_protection_en OTP regmap enable to regmap |
| i_tap_bist_mode | input |  | bist_mode signal used retun in functional mode |
| i_EN_0P05HP_ECG0 | input |  | bit enable select PDRES 05 ECG0 channel |
| i_EN_0P05HP_ECG12 | input |  | bit enable select PDRES 05 ECG12 channel |
| IMPQ_PSDRES_UP_02 | input | [2:0] | OTP value for Analog block |
| IMPQ_PSDRES_DOWN_02 | input | [2:0] | OTP value for Analog block |
| IMPP_PSDRES_UP_02 | input | [2:0] | OTP value for Analog block |
| IMPP_PSDRES_DOWN_02 | input | [2:0] | OTP value for Analog block |
| ECG_PSDRES_DOWN_02 | input | [2:0] | OTP value for Analog block |
| ECG_PSDRES_UP_02 | input | [2:0] | OTP value for Analog block |
| IMPQ_PSDRES_UP_05 | input | [2:0] | OTP value for Analog block |
| IMPQ_PSDRES_DOWN_05 | input | [2:0] | OTP value for Analog block |
| IMPP_PSDRES_UP_05 | input | [2:0] | OTP value for Analog block |
| IMPP_PSDRES_DOWN_05 | input | [2:0] | OTP value for Analog block |
| ECG_PSDRES_DOWN_05 | input | [2:0] | OTP value for Analog block |
| ECG_PSDRES_UP_05 | input | [2:0] | OTP value for Analog block |
| IMPQ_PSDRES_UP | output | [2:0] | OTP value for Analog block |
| IMPQ_PSDRES_DOWN | output | [2:0] | OTP value for Analog block |
| IMPP_PSDRES_UP | output | [2:0] | OTP value for Analog block |
| IMPP_PSDRES_DOWN | output | [2:0] | OTP value for Analog block |
| ECG_PSDRES_DOWN | output | [2:0] | OTP value for Analog block |
| ECG_PSDRES_UP | output | [2:0] | OTP value for Analog block |
| i_TS_IRQ_ST | input |  | time slot irq |
| i_DCCK_IRQ_ST | input |  | DCCK irq change status |
| o_en_tx_1V2 | input |  | input to LED pilot |
| o_tx_preset_1V2 | input |  | input to LED pilot |
| o_tx1_outlab_sel_frame_x_1V2 | input |  | input to LED pilot |
| sel_irq_gpio1 | input | [3:0] | sel irq gpi1 {000:NC, 001:i_fifo_irq, 010:i_LED_Pilot, 011:i_DCCK_irq, 100:i_error_time_slot, 101:i_time_slot_irq} |
| sel_irq_gpio2 | input | [3:0] | sel irq gpi2 {000:NC, 001:i_fifo_irq, 010:i_LED_Pilot, 011:i_DCCK_irq, 100:i_error_time_slot, 101:i_time_slot_irq} |
| sel_irq_gpio3 | input | [3:0] | sel irq gpi3 {000:NC, 001:i_fifo_irq, 010:i_LED_Pilot, 011:i_DCCK_irq, 100:i_error_time_slot, 101:i_time_slot_irq} |
| sel_irq_gpio4 | input | [3:0] | sel irq gpi4 {000:NC, 001:i_fifo_irq, 010:i_LED_Pilot, 011:i_DCCK_irq, 100:i_error_time_slot, 101:i_time_slot_irq} |
| DCCK_P_en | input |  | Manages lead off mode DCCK_P, Source: Regmap |
| DCCK_N_en | input |  | Manages lead off mode DCCK_P, Source: Regmap |
| MODE_OPERATION | input |  | Start Divice operations status |
| o_pd_p | output |  | enable lead off mode DCCK_P, Destination:Analog block |
| o_pd_n | output |  | enable lead off mode DCCK_N, Destination:Analog block |
| O_DCCK_P_i | input |  | status lead off mode DCCK_P {0:low impedance,1:floating electrode}, Source: Analog block |
| O_DCCK_N_i | input |  | status lead off mode DCCK_N {0:low impedance,1:floating electrode}, Source: Analog block |
| if_gpio1_in_reg | output |  | ZI value from regmap |
| if_gpio1_en_reg | input |  | EN value from regmap |
| if_gpio1_out_reg | input |  | OUT value from regmap |
| if_gpio2_in_reg | output |  | ZI value from regmap// ZI |
| if_gpio2_en_reg | input |  | EN value from regmap |
| if_gpio2_out_reg | input |  | OUT value from regmap |
| if_gpio3_in_reg | output |  | ZI value from regmap |
| if_gpio3_en_reg | input |  | EN value from regmap |
| if_gpio3_out_reg | input |  | OUT value from regmap |
| if_gpio4_in_reg | output |  | ZI value from regmap |
| if_gpio4_en_reg | input |  | EN value from regmap |
| if_gpio4_out_reg | input |  | OUT value from regmap |
| o_ext_reg_dt_afe | input | [7:0] | i2c/spi read data u_regmap_otp |
| o_ext_reg_dt | output | [7:0] | i2c/spi read data output |
| i_ext_reg_add | input | [7:0] | i2c/spi address |
| i_ext_reg_dt | input | [7:0] | i2c/spi write data |
| i_ext_reg_ck_wr | input |  | i2c/spi clk |
| i_ext_reg_we | input |  | i2c/spi regmap direct write enable |
| if_sclk_in | input |  | ZI_I2C |
| if_sclk_i3c_in | input |  | ZI_I3C |
| if_sclk_out | output |  | A |
| if_sclk_en | output |  | EN |
| if_sclk_tud | output |  | TUD |
| if_sda_sdi_in | input |  | ZI_I2C |
| if_sda_sdi_i3c_in | input |  | ZI_I3C |
| if_sda_sdi_out | output |  | A |
| if_sda_sdi_en | output |  | EN |
| if_sda_sdi_tud | output |  | TUD |
| if_cs_in | input |  | ZI CS |
| if_cs_out | output |  | A |
| if_cs_en | output |  | EN |
| if_cs_tud | output |  | TUD |
| if_gpio0_in | input |  | ZI GPIO0 |
| if_gpio0_out | output |  | A |
| if_gpio0_en | output |  | EN |
| if_gpio0_tud | output |  | TUD |
| if_gpio1_in | input |  | ZI GPIO1 |
| if_gpio1_out | output |  | A |
| if_gpio1_en | output |  | EN |
| if_gpio1_tud | output |  | TUD |
| if_gpio2_in | input |  | ZI GPIO2 |
| if_gpio2_out | output |  | A |
| if_gpio2_en | output |  | EN |
| if_gpio2_tud | output |  | TUD |
| if_gpio3_in | input |  | ZI GPIO3 |
| if_gpio3_out | output |  | A |
| if_gpio3_en | output |  | EN |
| if_gpio3_tud | output |  | TUD |
| if_gpio4_in | input |  | ZI GPIO4 |
| if_gpio4_out | output |  | A |
| if_gpio4_en | output |  | EN |
| if_gpio4_tud | output |  | TUD |
| if_sh_ck_in | input |  | ZI sensor hub ck |
| if_sh_ck_out | output |  | A |
| if_sh_ck_en | output |  | EN |
| if_sh_ck_tud | output |  | TUD |
| if_sh_sda_in | input |  | ZI sensor hub SDA |
| if_sh_sda_out | output |  | A |
| if_sh_sda_en | output |  | EN |
| if_sh_sda_tud | output |  | TUD |
| POR1V2_1V2 | input |  | POR |
| ispu_tck | output |  | jtag clk port ispu in ispu debug mode |
| ispu_tdi | output |  | jtag data in port ispu in ispu debug mode |
| ispu_tdo | input |  | jtag data out port ispu in ispu debug mode |
| ispu_tdo_en | input |  | jtag en port ispu in ispu debug mode |
| ispu_tms | output |  | jtag state in port ispu in ispu debug mode |
| ispu_trst_n | output |  | jtag reset in port ispu in ispu debug mode |
| i_if_ck | output |  | i2c/spi clk |
| i_if_cs | output |  | i2c/spi cs |
| i_if_sda_sdi_in | output |  | i2c/spi data |
| o_if_cs_en_n | input |  | i2c/spi cs en from i2c/spi ip |
| o_if_cs_out_n | input |  | i2c/spi cs out from i2c/spi ip |
| o_if_cs_tud | input |  | i2c/spi cs tud from i2c/spi ip |
| o_if_sda_sdi_en_n | input |  | i2c/spi data en from i2c/spi ip |
| o_if_sda_sdi_out | input |  | i2c/spi data out from i2c/spi ip |
| scl_master_in | output |  | from sensorhub |
| scl_master_out | input |  | from sensorhub |
| sda_master_in | output |  | to sensorhub |
| sda_master_out | input |  | from sensorhub |
| afe_bist_ctrl_tdi | output |  | bist jtag port |
| afe_bist_ctrl_trstn | output |  | bist jtag port |
| afe_bist_ctrl_tms | output |  | bist jtag port |
| afe_bist_ctrl_tdo | input |  | bist jtag port |
| afe_bist_ctrl_tdo_en_n | input |  | bist jtag port |
| afe_bist_ctrl_tck | output |  | bist jtag port |
| ck_16m | input |  | clk16MHz trimming in debug mode |
| ck_2m | input |  | clk2MHz trimming in debug mode |
| ck_64k | input |  | clk64MHz trimming in debug mode |
| scan_rstn | output |  | scan reset |
| scan_enable | output |  | scan enable |
| scan_clk | output |  | scan clock |
| ext_cap_en | input |  | extenal cap in debug mode |
| irq_fifo | input |  | fifo irq from main controller - regmap |
| error_time_slot | input |  | error timeslot from error time slot |
| gpio_in | output |  | gpio0 in in functional mode from regmap |
| gpio_out | input |  | gpio0 out in functional mode from regmap |
| gpio_en | input |  | gpio0 en in functional mode from regmap |
| irq_SENSHUB_STS | input |  | sensorhub status |
| irq_ISPU_MC_INTR_STS | input |  | irq main controller data-valid |
| scan_mode | output |  | stbio1 scan mode |
| bist_mode | output |  | stbio1 bist mode |
| debug_mode | output |  | stbio1 debug mode |
| memsafe | output |  | memsafe for bist |
| test_clk_fast | input |  | enable test clk 16MHz trimming |
| test_clk_slow | input |  | enable test clk 64kHz trimming |
| if_sclk_tud_reg | input |  | if sclk_tud from reg |
| if_sda_sdi_tud_reg | input |  | if sda_sdi_tud from re |
| if_sh_sda_tud_reg | input |  | if sh_sda_tud from reg |
| if_gpio0_tud_reg | input |  | if gpio0_tud from reg |
| if_gpio1_tud_reg | input |  | if gpio1_tud from reg |
| if_gpio2_tud_reg | input |  | if gpio2_tud from reg |
| if_gpio3_tud_reg | input |  | if gpio3_tud from reg |
| if_gpio4_tud_reg | input |  | if gpio4_tud from reg |
| if_sh_ck_tud_reg | input |  | if sh_ck_tud from reg |
| clk_dft_adc | input |  | 16MHz adc dft clk |
| i_ADC_clk_serial_dft | input |  | serial clk for adc dft 16Mhz:Fast mode, if_sh_ck_in:Low noise mode |
| EN_ADC_TEST_FAST | input |  | enable adc test fast |
| EN_ADC_TEST_LOW_NOISE | input |  | enable adc test low noise |
| i_ADC_EOC | input |  | adc eoc input |
| i_ADC_start | input |  | adc star from main controller |
| i_data_ADC_out | input | [15:0] | adc data input |
| o_ADC_start | output |  | adc star output |
| i_N_clk | input | [6-1:0] | n clk start time high |
| i_saturation_irq | input |  | interrupt saturation from main controller |
| i_end_boot | input |  | end boot signal from adsp |
| quokka_run | input |  | status quokka elaboration 1:run 0:not elaboration |
| sensor_hub_status | input |  | status sensor_hob from regmap |
| SH_CKL_EN | input |  | sensorhub clk enable from regmap (master_end_op_clr or sleep_master_clr or ahb_master_on_clr) |
| i_t_force_1v2 | input |  | input force signal to IDDQ test |
| o_t_force_1v2 | output |  | force signal to IDDQ test |
| i_bfail_fifo | input |  | bist fail fifo ram |
| i_bfail_qk | input |  | bist fail adsp ram |
| i_bfail_ispu1 | input |  | bist fail ispu ram instruction mem |
| i_bfail_ispu2 | input |  | bist fail ispu ram data mem 1 |
| i_bfail_ispu3 | input |  | bist fail ispu ram data mem 2 |
| i_bfail_ispu4 | input |  | bist fail ispu ram data mem 3 |
| i_bend | input |  | bist end signal |
| i_bbad | input |  | signal fail bist test |
| o_bypass | output |  | bypass memory signal |

Table 1313: PAD MUX I/O List

#### Requirements

[DDS_STBIO1_300] Definition:

The operative mode is selected by a i2c/spi write on register

| MODE_CONFIG | MODE_CONFIG_0 register |
| --- | --- |

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RESERVED | bypass | memsafe | ispu_debug_mode | debug_mode | bist_mode | SCAN_COMPR_EN | scan_mode |
|  | rw | rw | rw | rw | w | w | w |

0xFE

RW

0x00

| [6] | bypass:; SPIRIT parameter _resetPort_:reset_no_scan. SPIRIT parameter _clockPort_:clk_no_scan |
| --- | --- |
| [5] | memsafe:; SPIRIT parameter _resetPort_:reset_no_scan. SPIRIT parameter _clockPort_:clk_no_scan |
| [4] | ispu_debug_mode:. |
| [3] | debug_mode:. |
| [2] | bist_mode:. |
| [1] | SCAN_COMPR_EN:; SPIRIT parameter _resetPort_:reset_no_scan. SPIRIT parameter _customType_:W1S SPIRIT parameter _clockPort_:clk_no_scan |
| [0] | scan_mode:; SPIRIT parameter _customType_:W1S. SPIRIT parameter _resetPort_:reset_no_scan SPIRIT parameter _clockPort_:clk_no_scan |

[Vpriority High] [End]

Table 1414: PAD Configuration Table

[DDS_STBIO1_301] Assumption

It will be enabled by two consecutive writes, pag. 3 to address 0x40 to set the page and 0x00 to address 0x3E

[End]

[DDS_STBIO1_302] Requirement

If MODE_CONFIG register is 0x00 the operative mode shall be functional.

[Vpriority High] [End]

[DDS_STBIO1_303] Requirement

If MODE_CONFIG register is 0x01 the operative mode shall be SCAN.

[End]

[DDS_STBIO1_304] Requirement

If MODE_CONFIG register is 0x04 the operative mode shall be BIST.

[End]

[DDS_STBIO1_305] Requirement

If MODE_CONFIG register is 0x08 the operative mode shall be DEBUG.

[End]

[DDS_STBIO1_306] Requirement

If MODE_CONFIG register is 0x10 the operative mode shall be ISPU_DEBUG.

[Vpriority High] [End]

[DDS_STBIO1_307] Requirement

If MODE_CONFIG register is 0x20 the operative mode shall be Bist MEMSAFE mode.

[Vpriority High] [End]

[DDS_STBIO1_308] Requirement

If MODE_CONFIG register is 0x40 the operative mode shall be Bist Bypass mode.

[Vpriority High] [End]

[DDS_STBIO1_314] Requirement

In function mode, the GPIO1,2,3,4_out shall be General-Purpose if sel irq gpi1,2,3,4 is 0x00 and ext_cap_en is “0”

[Vpriority High][End]

[DDS_STBIO1_309] Requirement

In function mode, the GPIO1,2,3,4_out shall be irq_fifo if sel irq gpio1,2,3,4 is 0x01 and ext_cap_en is “0”

[Vpriority High] [End]

[DDS_STBIO1_310] Requirement

In function mode, the GPIO1,2,3,4_out shall be i_LED_Pilot if sel irq gpi1,2,3,4 is 0x02

and ext_cap_en is “0”

[Vpriority High] [End]

[DDS_STBIO1_311] Requirement

In function mode, the GPIO1,2,3,4_out shall be i_DCCK_irq if sel irq gpi1,2,3,4 is 0x03 and ext_cap_en is “0”

[Vpriority High] [End]

[DDS_STBIO1_312] Requirement

In function mode, the GPIO1,2,3,4 output shall be i_error_time_slot if sel irq gpi1,2,3,4 is 0x04 and ext_cap_en is “0”

[Vpriority High] [End]

[DDS_STBIO1_313] Requirement

In function mode, the GPIO1,2,3,4 output shall be i_time_slot_irq if sel irq gpi1,2,3,4 is 0x05 and ext_cap_en is “0”

[Vpriority High] [End]

Table 15 functional mode Static Connectivity Checks

| ENABLE | SOURCE | DESTINATION | NAME |
| --- | --- | --- | --- |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio1_irq_sel == 1) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_fifo.irq_FIFO_U_M_IRQ_STATUS | stbio1_top.if_gpio1_out | DDS_STBIO1_GPIO1_309 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio2_irq_sel == 1) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_fifo.irq_FIFO_U_M_IRQ_STATUS | stbio1_top.if_gpio2_out | DDS_STBIO1_GPIO2_309 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio3_irq_sel == 1) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) } | stbio1_top.dig_wrapper_LV.u_fifo.irq_FIFO_U_M_IRQ_STATUS | stbio1_top.if_gpio3_out | DDS_STBIO1_GPIO3_309 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio4_irq_sel == 1) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1)} | stbio1_top.dig_wrapper_LV.u_fifo.irq_FIFO_U_M_IRQ_STATUS | stbio1_top.if_gpio4_out | DDS_STBIO1_GPIO4_309 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio1_irq_sel == 1) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | 1'b0 | stbio1_top.if_gpio1_en | DDS_STBIO1_GPIO1_EN_309 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio2_irq_sel == 1) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | 1'b0 | stbio1_top.if_gpio2_en | DDS_STBIO1_GPIO2_EN_309 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio3_irq_sel == 1) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) } | 1'b0 | stbio1_top.if_gpio3_en | DDS_STBIO1_GPIO3_EN_309 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio4_irq_sel == 1) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1)} | 1'b0 | stbio1_top.if_gpio4_en | DDS_STBIO1_GPIO4_EN_309 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio1_irq_sel == 5) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_regmap_otp.irq_TS_IRQ_ST | stbio1_top.if_gpio1_out | DDS_STBIO1_GPIO1_313 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio1_irq_sel == 5) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | 1'b0 | stbio1_top.if_gpio1_en | DDS_STBIO1_GPIO1_EN_313 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio2_irq_sel == 5) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_regmap_otp.irq_TS_IRQ_ST | stbio1_top.if_gpio2_out | DDS_STBIO1_GPIO2_313 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio2_irq_sel == 5) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | 1'b0 | stbio1_top.if_gpio2_en | DDS_STBIO1_GPIO2_EN_313 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio3_irq_sel == 5) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) } | stbio1_top.dig_wrapper_LV.u_regmap_otp.irq_TS_IRQ_ST | stbio1_top.if_gpio3_out | DDS_STBIO1_GPIO3_313 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio3_irq_sel == 5) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) } | 1'b0 | stbio1_top.if_gpio3_en | DDS_STBIO1_GPIO3_EN_313 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio4_irq_sel == 5) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1)} | stbio1_top.dig_wrapper_LV.u_regmap_otp.irq_TS_IRQ_ST | stbio1_top.if_gpio4_out | DDS_STBIO1_GPIO4_313 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio4_irq_sel == 5) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1)} | 1'b0 | stbio1_top.if_gpio4_en | DDS_STBIO1_GPIO4_EN_313 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio1_irq_sel == 4) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_main_controller_top.o_error_time_slot | stbio1_top.if_gpio1_out | DDS_STBIO1_GPIO1_312 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio1_irq_sel == 4) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | 1'b0 | stbio1_top.if_gpio1_en | DDS_STBIO1_GPIO1_EN_312 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio2_irq_sel == 4) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_main_controller_top.o_error_time_slot | stbio1_top.if_gpio2_out | DDS_STBIO1_GPIO2_312 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio2_irq_sel == 4) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | 1'b0 | stbio1_top.if_gpio2_en | DDS_STBIO1_GPIO2_EN_312 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio3_irq_sel == 4) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) } | stbio1_top.dig_wrapper_LV.u_main_controller_top.o_error_time_slot | stbio1_top.if_gpio3_out | DDS_STBIO1_GPIO3_312 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio3_irq_sel == 4) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) } | 1'b0 | stbio1_top.if_gpio3_en | DDS_STBIO1_GPIO3_EN_312 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio4_irq_sel == 4) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1)} | stbio1_top.dig_wrapper_LV.u_main_controller_top.o_error_time_slot | stbio1_top.if_gpio4_out | DDS_STBIO1_GPIO4_312 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio4_irq_sel == 4) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1)} | 1'b0 | stbio1_top.if_gpio4_en | DDS_STBIO1_GPIO4_EN_312 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio1_irq_sel == 2) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_main_controller_top.o_en_tx & stbio1_top.dig_wrapper_LV.u_main_controller_top.o_tx_preset & stbio1_top.dig_wrapper_LV.u_main_controller_top.o_tx1_outlab_sel_frame_x | stbio1_top.if_gpio1_out | DDS_STBIO1_GPIO1_310 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio1_irq_sel == 2) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | 1'b0 | stbio1_top.if_gpio1_en | DDS_STBIO1_GPIO1_EN_310 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio2_irq_sel == 2) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_main_controller_top.o_en_tx & stbio1_top.dig_wrapper_LV.u_main_controller_top.o_tx_preset & stbio1_top.dig_wrapper_LV.u_main_controller_top.o_tx1_outlab_sel_frame_x | stbio1_top.if_gpio2_out | DDS_STBIO1_GPIO2_310 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio2_irq_sel == 2) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | 1'b0 | stbio1_top.if_gpio2_en | DDS_STBIO1_GPIO2_EN_310 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio3_irq_sel == 2) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) } | stbio1_top.dig_wrapper_LV.u_main_controller_top.o_en_tx & stbio1_top.dig_wrapper_LV.u_main_controller_top.o_tx_preset & stbio1_top.dig_wrapper_LV.u_main_controller_top.o_tx1_outlab_sel_frame_x | stbio1_top.if_gpio3_out | DDS_STBIO1_GPIO3_310 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio3_irq_sel == 2) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) } | 1'b0 | stbio1_top.if_gpio3_en | DDS_STBIO1_GPIO3_EN_310 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio4_irq_sel == 2) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1)} | stbio1_top.dig_wrapper_LV.u_main_controller_top.o_en_tx & stbio1_top.dig_wrapper_LV.u_main_controller_top.o_tx_preset & stbio1_top.dig_wrapper_LV.u_main_controller_top.o_tx1_outlab_sel_frame_x | stbio1_top.if_gpio4_out | DDS_STBIO1_GPIO4_310 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio4_irq_sel == 2) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1)} | 1'b0 | stbio1_top.if_gpio4_en | DDS_STBIO1_GPIO4_EN_310 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio1_irq_sel == 3) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_regmap_otp.irq_DCCK_IRQ_ST | stbio1_top.if_gpio1_out | DDS_STBIO1_GPIO1_311 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio1_irq_sel == 3) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | 1'b0 | stbio1_top.if_gpio1_en | DDS_STBIO1_GPIO1_EN_311 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio2_irq_sel == 3) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_regmap_otp.irq_DCCK_IRQ_ST | stbio1_top.if_gpio2_out | DDS_STBIO1_GPIO2_311 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio2_irq_sel == 3) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | 1'b0 | stbio1_top.if_gpio2_en | DDS_STBIO1_GPIO2_EN_311 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio3_irq_sel == 3) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) } | stbio1_top.dig_wrapper_LV.u_regmap_otp.irq_DCCK_IRQ_ST | stbio1_top.if_gpio3_out | DDS_STBIO1_GPIO3_311 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio3_irq_sel == 3) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) } | 1'b0 | stbio1_top.if_gpio3_en | DDS_STBIO1_GPIO3_EN_311 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio4_irq_sel == 3) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1)} | stbio1_top.dig_wrapper_LV.u_regmap_otp.irq_DCCK_IRQ_ST | stbio1_top.if_gpio4_out | DDS_STBIO1_GPIO4_311 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio4_irq_sel == 3) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1)} | 1'b0 | stbio1_top.if_gpio4_en | DDS_STBIO1_GPIO4_EN_311 |
| { !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_fifo.irq_FIFO_U_M_IRQ_STATUS ¦ stbio1_top.dig_wrapper_LV.u_main_controller_top.o_error_time_slot | stbio1_top.if_gpio0_out | DDS_STBIO1_GPIO0_EXT_CAP_315 |
| { !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && pad_mux__path.ext_cap_en} | 1'b0 | stbio1_top.if_gpio0_en | DDS_STBIO1_GPIO0_EN_EXT_CAP_315 |
| { !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && pad_mux__path.ext_cap_en} | 1'b1 | stbio1_top.if_gpio1_en | DDS_STBIO1_GPIO1_EN_EXT_CAP_315 |
| { !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && pad_mux__path.ext_cap_en} | 1'b1 | stbio1_top.if_gpio2_en | DDS_STBIO1_GPIO2_EN_EXT_CAP_315 |
| { !pad_mux__path.scan_mode && ((pad_mux__path.mission_mode == 1) ¦¦ pad_mux__path.debug_mode ¦¦ pad_mux__path.ispu_debug_mode) } | stbio1_top.if_cs_in | stbio1_top.dig_wrapper_LV.u_i2c_spi_slave.i_if_cs | DDS_STBIO1_CS_316 |
| { !pad_mux__path.scan_mode && ((pad_mux__path.mission_mode == 1) ¦¦ pad_mux__path.debug_mode ¦¦ pad_mux__path.ispu_debug_mode) } | !stbio1_top.dig_wrapper_LV.u_i2c_spi_slave.o_if_cs_en_n | stbio1_top.if_cs_en | DDS_STBIO1_CS_EN_316 |
| { !pad_mux__path.scan_mode && ((pad_mux__path.mission_mode == 1) ¦¦ pad_mux__path.debug_mode ¦¦ pad_mux__path.ispu_debug_mode) } | !stbio1_top.dig_wrapper_LV.u_i2c_spi_slave.o_if_cs_out_n | stbio1_top.if_cs_out | DDS_STBIO1_CS_OUT_316 |
| { !pad_mux__path.scan_mode && ((pad_mux__path.mission_mode == 1) ¦¦ pad_mux__path.ispu_debug_mode) && stbio1_top.dig_wrapper_LV.u_regmap_otp.SH_CKL_EN } | stbio1_top.dig_wrapper_LV.u_masterblaze_ahb.sda_master_out | stbio1_top.if_sh_sda_en | DDS_STBIO1_SH_SDA_EN_317 |
| { !pad_mux__path.scan_mode && !pad_mux__path.w_bist_mode && !pad_mux__path.debug_mode} | 1'b0 | stbio1_top.if_sh_sda_out | DDS_STBIO1_SH_SDA_OUT_317 |
| { !pad_mux__path.scan_mode && ((pad_mux__path.mission_mode == 1) ¦¦ pad_mux__path.ispu_debug_mode)} | stbio1_top.if_sh_sda_in | stbio1_top.dig_wrapper_LV.u_masterblaze_ahb.sda_master_in | DDS_STBIO1_SH_SDA_IN_317 |
| { !pad_mux__path.scan_mode && ((pad_mux__path.mission_mode == 1) ¦¦ pad_mux__path.ispu_debug_mode) && stbio1_top.dig_wrapper_LV.u_regmap_otp.SH_CKL_EN } | stbio1_top.dig_wrapper_LV.u_masterblaze_ahb.scl_master_out | stbio1_top.if_sh_ck_en | DDS_STBIO1_SH_SCL_EN_317 |
| { !pad_mux__path.scan_mode && !pad_mux__path.w_bist_mode && !pad_mux__path.debug_mode} | 1'b0 | stbio1_top.if_sh_ck_out | DDS_STBIO1_SH_SCL_OUT_317 |
| { !pad_mux__path.scan_mode && ((pad_mux__path.mission_mode == 1) ¦¦ pad_mux__path.ispu_debug_mode)} | stbio1_top.if_sh_ck_in | stbio1_top.dig_wrapper_LV.u_masterblaze_ahb.scl_master_in | DDS_STBIO1_SH_SCL_IN_317 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio1_irq_sel == 0) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio1_out | stbio1_top.if_gpio1_out | DDS_STBIO1_GPIO1_314 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio1_irq_sel == 0) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio1_en | stbio1_top.if_gpio1_en | DDS_STBIO1_GPIO1_EN_314 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio1_irq_sel == 0) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.if_gpio1_in | stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio1_in_i | DDS_STBIO1_GPIO1_IN_314 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio2_irq_sel == 0) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio2_out | stbio1_top.if_gpio2_out | DDS_STBIO1_GPIO2_314 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio2_irq_sel == 0) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio2_en | stbio1_top.if_gpio2_en | DDS_STBIO1_GPIO2_EN_314 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio2_irq_sel == 0) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.if_gpio2_in | stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio2_in_i | DDS_STBIO1_GPIO2_IN_314 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio3_irq_sel == 0) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) } | stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio3_out | stbio1_top.if_gpio3_out | DDS_STBIO1_GPIO3_314 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio3_irq_sel == 0) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) } | stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio3_en | stbio1_top.if_gpio3_en | DDS_STBIO1_GPIO3_EN_314 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio3_irq_sel == 0) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) } | stbio1_top.if_gpio3_in | stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio3_in_i | DDS_STBIO1_GPIO3_IN_314 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio4_irq_sel == 0) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1)} | stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio4_out | stbio1_top.if_gpio4_out | DDS_STBIO1_GPIO4_314 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio4_irq_sel == 0) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1)} | stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio4_en | stbio1_top.if_gpio4_en | DDS_STBIO1_GPIO4_EN_314 |
| {(stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio4_irq_sel == 0) && !pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1)} | stbio1_top.if_gpio4_in | stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio4_in_i | DDS_STBIO1_GPIO4_IN_314 |
| {!pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio0_out | stbio1_top.if_gpio0_out | DDS_STBIO1_GPIO0_318 |
| {!pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio0_en | stbio1_top.if_gpio0_en | DDS_STBIO1_GPIO0_EN_318 |
| {!pad_mux__path.scan_mode && (pad_mux__path.mission_mode == 1) && !pad_mux__path.ext_cap_en} | stbio1_top.if_gpio0_in | stbio1_top.dig_wrapper_LV.u_regmap_otp.gpio0_in_i | DDS_STBIO1_GPIO0_IN_318 |

#### DCCK Requirement

[DDS_STBIO1_390] Assumption

O_DCCK_P_i(A2D input) will be “1” in floating electrode P state else it will be “0”

[End]

DDS_STBIO1_391] Assumption

O_DCCK_N_i(A2D input) will be “1” in floating electrode N state else it will be “0”

[End]

[DDS_STBIO1_392] Requirement

If O_DCCK_P_i changes its value from '0' to '1' and DCCK_IRQ_MASK is set to '1', then irq_DCCK_IRQ_ST, output of u_regmap_otp, shall be set to '1'

[End]

[DDS_STBIO1_393] Requirement

If O_DCCK_N_i changes its value from '0' to '1' and DCCK_IRQ_MASK is set to '1', then irq_DCCK_IRQ_ST, output of u_regmap_otp, shall be set to '1'

[End]

[DDS_STBIO1_394] Requirement

If O_DCCK_P_i changes its value from '1' to '0' and DCCK_IRQ_MASK is set to '1', then irq_DCCK_IRQ_ST, output of u_regmap_otp, shall be set to '1'

[End]

[DDS_STBIO1_395] Requirement

If O_DCCK_P_i changes its value from '1’ to '0' and DCCK_IRQ_MASK is set to '1', then irq_DCCK_IRQ_ST, output of u_regmap_otp, shall be set to '1'

[End]

### Main Controller TOP

Instantiations

- inst_main_controller_fsm: main_controller_fsm

- inst_configuration_register_update: configuration_register_update

- inst_ECG_S_U: ECG_S_U

- inst_BIA_S_U: BIA_S_U

- inst_ppg_fsm: ppg_fsm

- inst_Digital_ramp_case: Digital_ramp_case

- inst_ALC_compensation: ALC_compensation

- inst_ADC_phases: ADC_phases

- inst_average_block: average_block

- inst_mainctrl_ahb_master: mainctrl_ahb_master

- inst_main_controller_clock_gatin_unit: main_controller_clock_gatin_unit

Figure 11: Main Controller Block Diagram

#### Main Controller TOP Description

The Main Controller is in charge of generating and receiving all the control signals for the Analog

Domain, furthermore, performs DSP elaboration on ADC Output and writes it in different Address.

The IP is divided into different sub-blocks, all at the same hierarchy level and described in the next chapter. Together with the Analog Domain, this IP is interfaced with the FIFO Controller, the ADSP and the PMU, these last two will not be described in this document. The IP is divided into different sub-blocks, all at the same hierarchy level and described in the next chapter.

#### Main Controller TOP I/O List

| Generic name | Type | Value | Description |
| --- | --- | --- | --- |
| ppg_frame_bit_top |  | 4 |  |
| n_max_frame |  | 12 |  |
| ppg_rx_channel_bit_top |  | 3 |  |
| T_tx_led_set_bit_top |  | 9 |  |
| T_tx_led_off_max_bit_top |  | 11 |  |
| m_odr_max_bit_top |  | 12 |  |
| mode_operation_n_bit_top |  | 2 |  |
| idac_dc_ppg_frame_bit_top |  | 6 |  |
| time_slot_sel_bit_top |  | 3 |  |
| T_tx_startup_bit_top |  | 8 |  |
| T_rx_set_bit_top |  | 9 |  |
| Nmax_bit_top |  | 4 |  |
| base_address_bit_top |  | 12 |  |
| delta_bit_top |  | 4 |  |
| gain_max_bit_top |  | 4 |  |
| T1_max_bit_top |  | 9 |  |
| T2_max_bit_top |  | 5 |  |
| N_clk_max_bit_top |  | 6 |  |
| n_average_ppg_max_bit_top |  | 3 |  |
| rx_ch_gain_max_bit_top |  | 4 |  |
| BTIA_ALC_register_bit_top |  | 9 |  |
| n_channel_bit_top |  | 4 |  |
| n_channel_adc_bit |  | 4 |  |
| quokka_routine_bit_top |  | 16 |  |
| vcm_bit_top |  | 14 |  |
| gnc_cmm_top |  | 15 |  |
| id_frame_max_bit_top |  | 4 |  |
| data_in_size_bit_top |  | 16 |  |
| ppg_start_up_bit_top |  | 9 |  |
| data_out_size_bit_top |  | 20 |  |
| n_average_ecg_bia_gsr_max_bit_top |  | 8 |  |
| rx_ch_frame_n_bit_top |  | 3 |  |
| curr_sel_bit_top |  | 8 |  |
| bit_outab_sel_top |  | 1 |  |
| n_average_ppg_max_value |  | 128 |  |
| n_average_ecg_bia_gsr_max_value |  | 16 |  |
| max_frame_repetition_top |  | 2 |  |
| n_tx_channel |  | 4 |  |
| ahb_base_address | [31:0] | 32'h000C4000 |  |
| time_buffer_const_bit_top |  | 10 |  |
| T1_max_bit_from_decoder_top |  | 4 |  |
| T2_max_bit_from_decoder_top |  | 2 |  |
| ppg_frame_bit_ALC_BTIA_top |  | 12 |  |
| ecg_channel_bit |  | 2 |  |

Table 16: Main Controller Parameter

| Port name | Direction | Type | Output Reset Value | Description |
| --- | --- | --- | --- | --- |
| i_clk_32kHz | input |  |  | clk 32kHz from pmu |
| i_clk_16MHz | input |  |  | clk 16MHz from pmu |
| i_clk_gsr_16M | input |  |  | clk for gsr signal |
| i_clk_ahb_master | input |  |  | clock for ahb master |
| i_rstn | input |  |  | reset in 16MHz domain |
| i_rstn_sync_32 | input |  |  | reset in 32kz domain |
| i_wave_velocity_mode | input |  |  | enable of the wave velocity mode |
| i_ispu_end_elab | input |  |  |  |
| i_n_ppg_frame | input | [ppg_frame_bit_top-1:0] |  | number of frame selected |
| i_const_value_ioff | input | [data_in_size_bit_top-1:0] |  | constant value from regmap used for ppg ioff calibraton |
| i_rx_ch_frame | input | [n_max_frame-1:0] |  | ppg rx channel frame from regamp |
| i_T_tx_led_set | input | [n_max_frame-1:0] |  | tx led set time from regmap |
| i_T_tx_led_off | input | [n_max_frame-1:0] |  | tx led off time from regmap |
| i_curr_sel_tx_frame_x | input | [n_max_frame-1:0] |  | current injection |
| i_tx1_outlab_sel_frame_x | input | [n_max_frame-1:0] |  | led output selected from regmap for each frame |
| i_tx2_outlab_sel_frame_x | input | [n_max_frame-1:0] |  | led output selected from regmap for each frame |
| i_tx3_outlab_sel_frame_x | input | [n_max_frame-1:0] |  | led output selected from regmap for each frame |
| i_tx4_outlab_sel_frame_x | input | [n_max_frame-1:0] |  | led output selected from regmap for each frame |
| i_idac_dc_ppg_frame | input | [n_max_frame-1:0] |  | idac value for each frame selected from regmap |
| i_average_ppg | input | [n_max_frame-1:0] |  | average ppg for each frame selected from regmap |
| i_gain_alc_start | input | [gain_max_bit_top-1:0] |  | btia gain for alc from regmap |
| i_gain_start | input | [n_max_frame-1:0] |  | btia gain start for compensation from regmap |
| i_Tx_idac_Vref_debug | input | [Nmax_bit_top-1:0] |  | Idac value for debug from regmap |
| i_ppg_start_up_time | input | [ppg_start_up_decod_bit_top-1:0] |  | start up time for ppg from regmap |
| i_en_txi | input | [n_max_frame-1:0] |  | tx enabling for each frame from regmap |
| i_frame_repetition | input | [n_max_frame-1:0] |  | i frame repetition for each frame from regmap |
| i_T_preset | input | [1:0] |  | Preset time for digital ramp from regmap |
| i_m_ecg | input | [m_odr_max_bit_top-1:0] |  | m parameter for ecg from regmap |
| i_m_bia | input | [m_odr_max_bit_top-1:0] |  | m parameter for bia from regmap |
| i_m_gsr | input | [m_odr_max_bit_top-1:0] |  | m parameter for gsr from regmap |
| i_m_ppg | input | [m_odr_max_bit_top-1:0] |  | m parameter for ppg from regmap |
| i_debug_adc_error_cap8_0_bypass | input |  |  | error cap bypass for adc from regmap |
| i_base_address_ecg0 | input | [base_address_bit_top-1:0] |  | base address for ecg0 in multiple fifo |
| i_base_address_bia | input | [base_address_bit_top-1:0] |  | base address for bia/ecg1-2 in multiple fifo |
| i_base_address_gsr | input | [base_address_bit_top-1:0] |  | base address for gsr in multiple fifo |
| i_base_address_ppg | input | [base_address_bit_top-1:0] |  | base address for ppg in multiple fifo |
| i_frame_ppg_debug | input | [ppg_frame_bit_top-1:0] |  | frame selection for ppg data debug |
| i_en_buf_gpio1 | input |  |  | enable buffer on gpio1 |
| i_en_buf_gpio2 | input |  |  | enable buffer on gpio2 |
| i_en_biobuf | input |  |  | enable biobuffer |
| i_do_calibration | input |  |  | enabling for adc calibration |
| i_mode_operation | input | [mode_operation_n_bit_top-1:0] |  | mode operation selection |
| i_T_rx_set | input | [T1_max_bit_from_decoder_top-1:0] |  | rx channel time set |
| i_en_clkf | input |  |  | en 16 MHz clock |
| i_quokka_boot_end | input |  |  | from quokka |
| i_quokka_run | input |  |  | from quokka |
| i_clk_16MHz_ready | input |  |  | (from analog circuit) |
| i_imp_curinj_en | input |  |  | enabling current injection for bia |
| i_en_ldo_1v8 | input |  |  | enabling ldo 1v8 |
| i_debug_comp_rx_ck | input |  |  | enabling debug for comp rx ck |
| i_debug_gsr_on | input |  |  | gsr on debug mode |
| i_debug_gsr_curr_on | input |  |  | gsr curr on debug mode |
| i_alc_disabled | input | [n_max_frame-1:0] |  | disabling alc bit for each frame |
| i_en_cm_buf | input |  |  | enabling common buffer |
| i_en_rld | input |  |  | enabling rld |
| i_en_avg1 | input |  |  | enabling avg1 |
| i_en_avg2 | input |  |  | enabling avg1 |
| i_hc_en | input |  |  | enabling health channel |
| i_debug | input |  |  | enabling debug mode |
| i_imp_en | input |  |  | enabling bia channel |
| i_debug_en_tia | input |  |  | enabling debug tia |
| i_debug_en_buffer | input |  |  | enabling debug for buffer |
| i_debug_en_idac | input |  |  | enabling debug for IDAC |
| i_en_clk_chop10k | input |  |  | enabling chopper clock |
| i_debug_tx_preset | input |  |  | enabling tx preset |
| i_en_ldo_1v8_ext | input |  |  | enabling external ldo 1v8 |
| i_clk_imp_10k_en | input |  |  | enabling clock chopper for bia |
| i_b_cap_idac | input | [1:0] |  | enabling cap for idac |
| i_adc_mux_bio_channel_debug | input | [2:0] |  | enabling debug adc mux bio channel |
| i_adc_mux_sel_debug | input |  |  | enabling adc mux |
| i_ispu_core_sw_rstn | input |  |  | ispu core enabling |
| i_T_TIAON | input | [T1_max_bit_from_decoder_top-1:0] |  | TIA Time |
| i_scan_mode | input |  |  | scan mode enabling |
| i_reset_debug_adc_data | input |  |  | reset adc data debug |
| i_start_adc_debug | input |  |  | pilot adc start om debug |
| i_en_ADC_debug | input |  |  | enabling adc debug |
| i_debug_adc_readout_bypass | input |  |  | enabling adc readout bypass |
| i_su_hlt_dly | input |  |  | enabling start up delay for ecg |
| i_multiple_fifo_sel | input |  |  | enabling multiple fifo |
| i_debug_calibration | input |  |  | enabling debug calibraiton |
| i_en_gsr_dc_cds | input | [1:0] |  | enabling gsr cds mode |
| i_ADC_EOC | input |  |  | end of conversion from adc |
| i_ecg0_sel | input |  |  | select ecg0 channel |
| i_bia_sel | input |  |  | select bia channel |
| i_gsr_sel | input |  |  | select gsr channel |
| i_ppg_sel | input |  |  | select ppg channel |
| i_ppg_ioff_calibration | input |  |  | enabling ppg ioff calibration |
| i_ecg1_ecg2_sel | input |  |  | enabling ecg1 and ecg2 channel |
| i_rst_async_ecg | input |  |  | reset async for ecg |
| i_rst_async_bia | input |  |  | reset async for bia |
| i_ppg_quokka_elab_sel | input |  |  | enabling adsp elab for ppg |
| i_data_ADC_out | input | [data_in_size_bit_top-1:0] |  | (from analog circuit) |
| i_time_slot_selection | input | [time_slot_sel_bit_top-1:0] |  | odr selection from regmap |
| i_Nmax_to_reg | input | [Nmax_bit_top-1:0] |  | Max value for digital ramp |
| i_nstart_case | input | [Nmax_bit_top-1:0] |  | n_start value for frist sterp |
| i_debug_rx_ch | input | [rx_ch_frame_n_bit_top-1:0] |  | enabling debug for rx channel |
| i_T_delta1_to_reg | input | [delta_bit_top-1:0] |  | delta 1 time for digital ramp |
| i_gain_max_set | input | [gain_max_bit_top-1:0] |  | gain max for alc |
| i_BTIA_ALC_debug | input | [BTIA_ALC_register_bit_top-1:0] |  | BTIA ALC Debug enable |
| i_comp_rx_out | input |  |  | input from analog comparator |
| i_adc_error_cap8_debug | input | [6:0] |  | adc cap error debug |
| i_adc_error_cap7_debug | input | [6:0] |  | adc cap error debug |
| i_adc_error_cap6_debug | input | [6:0] |  | adc cap error debug |
| i_adc_error_cap5_debug | input | [6:0] |  | adc cap error debug |
| i_adc_error_cap4_debug | input | [6:0] |  | adc cap error debug |
| i_adc_error_cap3_debug | input | [6:0] |  | adc cap error debug |
| i_adc_error_cap2_debug | input | [6:0] |  | adc cap error debug |
| i_adc_error_cap1_debug | input | [6:0] |  | adc cap error debug |
| i_adc_error_cap0_debug | input | [6:0] |  | adc cap error debug |
| i_BIDAC_debug_register | input | [IDAC_offset-1:0] |  |  |
| i_sleep_master | input |  |  |  |
| i_lpm | input | [1:0] |  | low power mode for ldo |
| i_mpm | input | [1:0] |  | medium power mode for ldo |
| i_hpm | input | [1:0] |  | high power mode for ldo |
| i_dis_rc_viref | input |  |  | dis rc viref |
| i_T1 | input | [T1_max_bit_from_decoder_top-1:0] |  | 4 to 10 us=160nclk |
| i_T2 | input | [T2_max_bit_from_decoder_top-1:0] |  | 0.5 to 2 us=~32 |
| i_N_clk_ppg | input | [N_clk_max_bit_top-1:0] |  | Sample time for ppg |
| i_N_clk_ecg | input | [N_clk_max_bit_top-1:0] |  | Sample time for ecg |
| i_N_clk_cal | input | [N_clk_max_bit_top-1:0] |  | Sample time for adc calibration |
| scan_enable | input |  |  | scan enable for scan mode |
| i_timer_buffer_config | input | [1:0] |  | timer buffer configuration |
| i_ppg_alc_frame_save | input | [ppg_frame_bit_top-1:0] |  | alc data frame to save |
| i_reset_ppg_ioff_calib | input |  |  |  |
| HRDATA | input | [31:0] |  | AHB Protocol signals |
| HREADY | input |  |  | AHB Protocol signals |
| HRESP | input | [1:0] |  | AHB Protocol signals |
| HGRANT | input |  |  | AHB Protocol signals |
| SCANENABLE | input |  |  | Scan Test Mode Enbl |
| SCANINHCLK | input |  |  | Scan Chain Input |
| SCANOUTHCLK | output |  |  | Scan Chain Output |
| HSEL | output |  | 0 | AHB Protocol signals |
| HADDR | output | [31:0] | 0 | AHB Protocol signals |
| HTRANS | output | [1:0] | 0 | AHB Protocol signals |
| HWRITE | output |  | 0 | AHB Protocol signals |
| HSIZE | output | [2:0] | 0 | AHB Protocol signals |
| HBURST | output | [2:0] | 0 | AHB Protocol signals |
| HPROT | output | [3:0] | 0 | AHB Protocol signals |
| HWDATA | output | [31:0] | 0 | AHB Protocol signals |
| HBUSREQ | output |  | 0 | AHB Protocol signals |
| HLOCK | output |  | 0 | AHB Protocol signals |
| o_bio_clock_div | output |  | 0 | output for clock chopper |
| o_BIDAC | output | [IDAC_offset-1:0] | 0 | IDAC for top |
| o_rst_imp | output |  | 1 | bia start up rst signal |
| o_rsti_imp | output |  | 1 | bia start up rsti signal |
| o_sug_imp | output |  | 1 | bia start up sug channel |
| o_pd_imp | output |  | 1 | bia start up pd channel |
| o_start_operative_16MHz | output |  | 0 | start operative 16MHz |
| o_su_hc2 | output |  | 1 | ecg start up hc2 signal |
| o_rst_hc2 | output |  | 1 | ecg start up rst signal |
| o_rsti_hc2 | output |  | 1 | ecg start up rsti signal |
| o_pd_hc2 | output |  | 1 | ecg start up pd signal |
| o_reset_reg_rst | output |  | 1 | end of reset async phase |
| o_reset_reg_rst_imp_async | output |  | 1 | end of reset async phase |
| o_imp_curinj_pd | output |  | 0 | bia current injection output |
| o_Txi_IDAC_Vref | output | [Nmax_bit_top-1:0] | 0 | TX IDAC VREF TO analog domain |
| o_comp_rx_ck | output |  | 0 | output for comparator debug |
| o_BTIA_gain | output | [gain_max_bit_top-1:0] | 0 | BTIA gain output to analog domain |
| o_data_out_debug_adc | output | [15:0] | 0 | o data output for adc debug |
| o_en_mask_ldo_pdclkf_reg | output |  | 1 | mask for en_ldo and pdf |
| o_pd_clkf | output |  | 1 | pd_clkf |
| o_trim_ldo_d | output | [1:0] | 3 | trim ldo value |
| o_BDC_PPG | output | [idac_dc_ppg_frame_bit_top-1:0] | 0 | idac dc ppg frame output |
| o_adc_mux_bio_channel | output | [2:0] | 0 | adc mux bio channel |
| o_adc_mux_sel | output |  | 0 | adc mux sel output |
| o_error_time_slot | output |  | 0 | error time slot irq |
| o_curr_sel_tx_frame_x | output | [curr_sel_bit_top-1:0] | 0 | current selection for tx channel output |
| o_tx1_outlab_sel_frame_x | output | [bit_outab_sel_top-1:0] | 0 | tx outlab sel frame x |
| o_tx2_outlab_sel_frame_x | output | [bit_outab_sel_top-1:0] | 0 | tx outlab sel frame x |
| o_tx3_outlab_sel_frame_x | output | [bit_outab_sel_top-1:0] | 0 | tx outlab sel frame x |
| o_tx4_outlab_sel_frame_x | output | [bit_outab_sel_top-1:0] | 0 | tx outlab sel frame x |
| o_tx_preset | output |  | 0 | tx prese to analog domain |
| o_en_tx | output | [n_tx_channel-1:0] | 0 | tx enable to analog domain |
| o_PD_TIA | output |  | 1 | enabling tia to analog domain |
| o_PD_PGA | output |  | 1 | enabling buffer to analog domain |
| o_PD_IDAC | output |  | 1 | enabling idac to analog domain |
| o_gsr_curr_on | output |  | 0 | gsr curr on pilot to analog domain |
| o_gsr_on | output |  | 0 | grs on on pilot to analog domain |
| o_ADC_en | output |  | 0 | ADC enabling |
| o_half_charge_ldo | output |  | 0 |  |
| o_ADC_start | output |  | 0 | ADC Start sampling |
| o_N_step2_to_reg | output | [rx_ch_gain_limit-1:0] | 0 | N_Step2 calibration value |
| o_adc_test_cap | output | [9:0] | 0 | adc test cap |
| o_adc_error_cap9 | output | [7:0] | 0 | adc calibration result |
| o_adc_error_cap8 | output | [6:0] | 0 | adc calibration result |
| o_adc_error_cap7 | output | [6:0] | 0 | adc calibration result |
| o_adc_error_cap6 | output | [6:0] | 0 | adc calibration result |
| o_adc_error_cap5 | output | [6:0] | 0 | adc calibration result |
| o_adc_error_cap4 | output | [6:0] | 0 | adc calibration result |
| o_adc_error_cap3 | output | [6:0] | 0 | adc calibration result |
| o_adc_error_cap2 | output | [6:0] | 0 | adc calibration result |
| o_adc_error_cap1 | output | [6:0] | 0 | adc calibration result |
| o_adc_error_cap0 | output | [6:0] | 0 | adc calibration result |
| o_pd_cm_buf | output |  | 1 | power down common buffer |
| o_pd_rld | output |  | 1 | power down rld |
| o_pd_avg1 | output |  | 1 | power down avg1 |
| o_pd_avg2 | output |  | 1 | power down avg2 |
| o_mc_ahb_clk_en | output |  | 0 | enable for master ahb clock |
| o_main_controller_device_status_register | output | [2:0] | 0 | device status register |
| o_main_controller_elab_status_register | output | [3:0] | 0 | device status register |
| o_en_ldo_1v8 | output |  | 0 | enable ldo |
| o_en_bufxbio | output |  | 0 | enable biobuffer to analog domain |
| o_rx_ch_frame | output | [ppg_rx_channel_bit_top-1:0] | 0 | selected rx ch frame to analog domain |
| o_ldo_ext | output |  | 0 | enable external ldo |
| o_ppg_status_register | output | [n_state_bit-1:0] | 0 | ppg state status register |
| o_b_cap_idac | output | [1:0] | 0 | b cap idac to analog domain |
| o_ispu_irq_ctrl | output | [3:0] | 0 | ispu irq |
| o_irq_start_measurement | output |  | 0 | irq start measurement to GPIO |
| o_end_calibration | output |  | 1 | end calibration to analog domain |
| o_alc_data | output | [data_out_size_bit_top+2:0] | 0 | alc data to regmap |
| o_ppg_data_raw | output | [data_out_size_bit_top+2:0] | 0 | ppg raw data to regmap |
| o_pd_buf_gpio1 | output |  | 1 | power down for buffer gpio1 |
| o_pd_buf_gpio2 | output |  | 1 | power down for buffer gpio2 |
| o_adc_data_debug_alc_1 | output | [frame_ppg_debug_alc_top-1:0] | 0 | alc 1 registers data debug |
| o_adc_data_debug_alc_2 | output | [frame_ppg_debug_alc_top-1:0] | 0 | alc 2 registers data debug |
| o_adc_data_debug_ppg_raw | output | [frame_ppg_debug_ppg_raw_top-1:0] | 0 | ppg raw registers data debug |
| o_en_lead_off | output |  | 0 | mask lead off |
| o_mc_busy | output |  | 0 | main controller is working |
| o_en_ck_m_p_q | output |  | 0 | clock m and q bia |
| o_en_ck_chop_imp | output |  | 0 | clock chopper bia |
| o_saturation_flag | output | [n_max_frame-1:0] | 0 | ppg saturation flag |
| o_en_ck_hc2_10k | output |  | 0 | clock chipper ecg |
| o_en_clk_16MHz_sleep | output |  | 1 | sleep signal for pmu |
| o_gsr_clk_en_16M | output |  | 0 | enable clock for gsr channel |

Table 17: Main Controller I/O List

#### Main Controller Peculiar Requirements

Main Controller manage all the Analog domain, so all its requirements shall be considered as High Priority. To check correctly the requirements, it’s important to cover all the different phase described, partially, in chapter 12 and its IPOS.

IPs

### ADSP integration

| Generic name | Type | Value | TOP CONNECTION |
| --- | --- | --- | --- |
| ADDRWIDTH |  | 12 |  |

Table 18: ADSP Parameter Table

| Port name |  | Direction | Type | TOP CONNECTION |
| --- | --- | --- | --- | --- |
| stmc_clk |  | input | wire | u_pmu.tst_clk_16m |
| HCLK |  | input | wire | u_pmu.clk_stmc |
| HRESETn |  | input | wire | u_pmu.resetn_stmc |
| HSELS |  | input | wire | u_xbar_afe. |
| HADDRS |  | input | wire [ADDRWIDTH-1:0] | u_xbar_afe.haddr_M1 |
| HTRANSS |  | input | wire [ 1:0] | u_xbar_afe.htrans_M1 |
| HSIZES |  | input | wire [ 2:0] | u_xbar_afe. |
| HWRITES |  | input | wire | u_xbar_afe. |
| HREADYS |  | input | wire | u_xbar_afe. |
| HWDATAS |  | input | wire [ 31:0] | u_xbar_afe. |
| HREADYOUTS |  | output | wire | u_xbar_afe. |
| HRESPS |  | output | wire | u_xbar_afe. |
| HRDATAS |  | output | wire [ 31:0] | u_xbar_afe. |
| HRDATAM |  | input | wire [ 31:0] |  |
| HREADYM |  | input | wire |  |
| HRESPM |  | input | wire [ 1:0] |  |
| HGRANTM |  | input | wire |  |
| HSELM |  | output | wire |  |
| HADDRM |  | output | wire [ 31:0] |  |
| HTRANSM |  | output | wire [ 1:0] |  |
| HWRITEM |  | output | wire |  |
| HSIZEM |  | output | wire [ 2:0] |  |
| HBURSTM |  | output | wire [ 2:0] |  |
| HPROTM |  | output | wire [ 3:0] |  |
| HWDATAM |  | output | wire [ 31:0] |  |
| HBUSREQM |  | output | wire |  |
| HLOCKM |  | output | wire |  |
| stmc_data_CSN |  | output | wire |  |
| stmc_data_WEN |  | output | wire |  |
| stmc_data_A |  | output | wire [ 8:0] |  |
| stmc_data_D |  | output | wire [ 63:0] |  |
| stmc_data_M |  | output | wire [ 63:0] |  |
| stmc_data_Q |  | input | wire [ 63:0] |  |
| stmc_rom_CLK |  | output | wire |  |
| stmc_rom_CSN |  | output | wire |  |
| stmc_rom_A |  | output | wire [ 10:0] |  |
| stmc_rom_Q |  | input | wire [ 22:0] |  |
| quokka_run |  | output | wire | u_pmu.adsp_run main_controller_top.i_quokka_run |
| quokka_boot_end |  | output | wire | main_controller_top.i_quokka_boot_end |
| ahb_master_clk_en |  | output | wire | U_pmu.clk_en_s1 |
| scan_mode |  | input | wire |  |
| scan_en |  | input | wire |  |
| scan_clk |  | input | wire |  |
| scan_rst_n |  | input | wire |  |

Table 19: ADSP I/O List

#### ADSP Peculiar Requirements

[DDS_STBIO1_0010] Definition:

ADSP is responsible for managing signals for the OTP memory H9A_FU_OTPHV_85AL05_23A_MIM5_6ML_ALUCU/2.6.b-00, which are required for its reading, programming and testing procedures.

The selected OTP memory has a data bit cut size of 640 bits with an 8-bit word size.

Each data bit is physically redundant to optimize memory yield. Consequently, the OTP memory is organized into 2 banks, each containing 80 addresses of 8-bit size.

[End]

##### OTP I/O List

[DDS_STBIO1_0011] Definition:

| Pin Name | Function |
| --- | --- |
| PDN | Power down command. Memory is in low power mode when PDN is low. |
| A<6:0> | A<6:0> is the word address for read or programming operation |
| BANKEN<1:0> | Bank selection bits. Used to program and read each bank separately. Or to activate redundancy when reading (2 banks read simultaneously, correction enabled) |
| BITSEL<2:0> | To define the bit to be programmed in the 8 bit word. Decimal value of (BITSEL<2:0>) |
| SELECTWLBL | To select antifuse block to be programmed |
| PRG | To select programming mode (active high) |
| FUSE | Fuse command to program bit cell. |
| TREAD<3:0> | For read condition trimming (sense current and read voltage). |
| VREF | Analog 1.25V+/-5% bandgap reference voltage |
| IREF | Pin requiring external current sink of 10uA+/-30% |

Table 20: OTP I/O List

[End]

##### Requirements

[DDS_STBIO1_0012] Definition:

There are three routine that the ADSP use to interact with the OTP memory, which are the Write, Boot and Test routine. In ADSP registers map there are 3 specific bits used to trigger these OTP routines:

| AHB ADDRESS | AHB REGISTER | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C1E02 | CONTROL_A | // | // | // | // | OTP_TEST | OTP_WRITE | OTP_BOOT | SOFT_RESET |

- OTP BOOT Bit used to trigger Boot Routine.

- OTP WRITE Bit used to trigger Write routine.

- OTP TEST  Bit used to trigger Test routine.

The same register also contain a SOFT_RESET bit used to trigger the corresponding soft reset routine.

[End]

[DDS_STBIO1_0013] Requirement:

To move OTP signal, ADSP shall writes new value in a regbank by AHB protocol.

| Register Name | Register Title | Address Offset | Reset value | Bits | Field name | Access |
| --- | --- | --- | --- | --- | --- | --- |
| OTP_PDN | OTP_PDN register | 0xc9000 | 0x0000 | 0 | OTP_PDN | RW |
| OTP_A | OTP_A register | 0x c9004 | 0x0000 | 6:0 | OTP_A | RW |
| OTP_BANKEN | OTP_BANKEN register | 0x c9008 | 0x0000 | 1:0 | OTP_BANKEN | RW |
| OTP_BITSEL | OTP_BITSEL register | 0x c900C | 0x0000 | 2:0 | OTP_BITSEL | RW |
| OTP_SELECTWLBL | OTP_SELECTWLBL register | 0x c9010 | 0x0000 | 0 | OTP_SELECTWLBL | RW |
| OTP_PRG | OTP_PRG register | 0x c9014 | 0x0000 | 0 | OTP_PRG | RW |
| OTP_FUSE | OTP_FUSE register | 0x c9018 | 0x0000 | 0 | OTP_FUSE | RW |
| OTP_TREAD | OTP_TREAD register | 0x c9020 | 0x08 | 3:0 | OTP_TREAD | RW |
| OTP_D | OTP_D register | 0x c9024 | 0x0000 | 7:0 | OTP_D | R |
| OTP_IREF_WAKEUP | OTP_IREF_WAKEUP register | 0x c9028 | 0x0000 | 0 | OTP_IREF_WAKEUP | RW |
| OTP_D_TEST | OTP_D_TEST register | 0x c902C | 0x0000 | 7:0 | OTP_D_TEST | RW |

[Vpriority High] [End]

[DDS_STBIO1_0014] Requirement:

The BOOT routine shall starts when the OTP_BOOT bit rises and shall completes when quokka_boot_end rises.

[Vpriority High] [End]

[DDS_STBIO1_0015] Requirement:

During OTP boot operation, the ADSP shall copy the content of the OTP memory into the OTP registers, following the procedure to access OTP memory

[Vpriority High] [End]

[DDS_STBIO1_0016] Requirement:

During BOOT routine, ADSP shall first read the last location of the OTP memory, at address 0x7F. If the content is 0xFF, called “lifecycle” byte, then the READ operation proceeds from address 0 to address 0x2F; otherwise, ADSP shall not perform any further reads.

Figure 12: Boot Routine

[Vpriority High] [End]

[DDS_STBIO1_0017] Requirement:

To perform a BOOT routine, OTP_BOOT bit shall be written when quokka_run is low.

[Vpriority High] [End]

[DDS_STBIO1_0018] Requirement:

The WRITE routine shall start when OTP Write Bit rises and completes when quokka_run falls.

[Vpriority High] [End]

[DDS_STBIO1_0019] Requirement:

During WRITE routine ADSP shall copy the content of the OTP regmap into the OTP memory, following the procedure to burn the fuses as expressed in the OTP datasheet

[Vpriority High] [End]

[DDS_STBIO1_0020] Requirement:

During WRITE routine, ADSP shall allow copying all OTP Registers into the OTP Memory only if the location 0x24 of OTP Memory (OTP[24] in the following diagram) is not set to 0xFF.

Figure 13: Write Routine Start

[Vpriority High] [End]

[DDS_STBIO1_0021] Requirement:

During the WRITE routine and after copying all OTP Registers into the OTP Memory, ADSP shall compare each value inside the OTP memory with the corresponding value in OTP Registers to check if the WRITE routine has been succesfull. In that case ADSP shall write 0xFF into the last location of the OTP memory.

Figure 14: WRITE routine and TEST regmap

[Vpriority High] [End]

[DDS_STBIO1_0022] Requirement:

OTP_WRITE, OTP_BOOT and OTP_TEST bits shall be cleared by ADSP at the end of their respective operation

[Vpriority High] [End]

[DDS_STBIO1_0023] Requirement:

A new operation shall start only when quokka_run is low, and one between OTP_TEST bit, OTP_WRITE bit or OTP_BOOT bit is written to 1

[Vpriority High] [End]

[DDS_STBIO1_0024] Requirement:

OTP_IREF_WAKEUP shall be set to one by an external I2C/SPI to AHB write before initiating a new OTP_TEST or OTP_WRITE operation

[Vpriority High] [End]

[DDS_STBIO1_0025] Requirement:

OTP_IREF_WAKEUP shall be setted to zero by an external I2C/SPI to AHB write at the end of OTP_TEST or OTP_WRITE operation

[Vpriority High] [End]

[DDS_STBIO1_0026] Requirement:

After startup phase, once started with an operation, the IP shall ignore any further start incoming transactions.

[Vpriority High] [End]

[DDS_STBIO1_0027] Requirement:

The TEST routine shall start when OTP Test Bit rises and completes when quokka_run falls.

[Vpriority High] [End]

[DDS_STBIO1_0028] Requirement:

The TEST routines shall allow reading an 8-bit word on OTP of a specific address written in OTP_A register, and write this value in OTP_D_TEST register.

[Vpriority High] [End]

[DDS_STBIO1_0029] Definition:

| Timing parameter | Value |
| --- | --- |
| ts_pdnr_tread | 3 µs |
| ts_pdnr_fuse | 3 µs |
| th_pdnr_selectwlbl | 8 µs |
| ts_selectwlblr_a | 6 µs |
| ts_selectwlblr_prg | 5 µs |
| ts_fuser_selectwlbl | 5 µs |
| tfuse_high | 100 µs |
| th_fusef_selectwlbl | 5 µs |
| t_selectwlbl_high_read | 10 µs |
| td_selectwlblr_dout | 8 µs |
| tselectwbl_low | 7 µs |
| th_selectwlblf_a | 2*clk_period |
| ts_pdnf_selectwlbl | 500 ns |
| th_pdnf_tread | 150 ns |
| th_pdnf_fuse | 150 ns |
| th_selectwlblf_prg | 2*clk_period |
| clk_period | 31ns |
| n_mirror_reg | 6 |

Table 21 Chosen OTP Timings

[End]

[DDS_STBIO1_0030] Definition:

| Timing parameter | Value |
| --- | --- |
| t_state1_cycles | ts_pdnr_tread_fuse_cycles + th_pdnr_selectwlbl_cycles |
| t_s1_int1_cycles | ts_pdnr_tread_fuse_cycles |
| t_s1_int2_cycles | t_state1_cycles - ts_selectwlblr_a_cycles |
| t_s1_int3_cycles | t_state1_cycles - ts_selectwlblr_prg_cycles |
| t_state2_write_cycles | ts_fuser_selectwlbl_cycles + tfuse_high_cycles + th_fusef_selectwlbl_cycles |
| t_s2_int1_write_cycles | ts_fuser_selectwlbl_cycles |
| t_s2_int2_write_cycles | t_state2_write_cycles - th_fusef_selectwlbl_cycles |
| t_state2_read_cycles | t_selectwlbl_high_read_cycles |
| t_s2_int1_read_cycles | td_selectwlblr_dout_cycles |
| t_state3_cycles | tselectwbl_low_cycles |
| t_s3_int1_cycles | th_selectwlblf_a_cycles |
| t_s3_int2_cycles | tselectwbl_low_cycles - ts_selectwlblr_a_cycles |
| t_state4_cycles | ts_pdnf_selectwlbl_cycles + th_pdnf_tread_fuse_cycles |
| t_s4_int1_cycles | th_selectwlblf_a_cycles |
| t_s4_int2_cycles | th_selectwlblf_prg_cycles |
| t_s4_int3_cycles | ts_pdnf_selectwlbl_cycles |

Table 22 Operation phases timings

[End]

[DDS_STBIO1_0031] Definition:

Figure 15 : OTP Reading timing diagram

[End]

[DDS_STBIO1_0032] Requirement:

After each Boot routine performed by the ADSP, a value of 0x04 shall be written once to address 0x0CA00C to ensure correct operation of the AHB transactions.

[Vpriority High] [End]

[DDS_STBIO1_0033] Requirement:

The Soft Reset routine shall start when SOFT_RESET Bit (in CONTROL_A register) rises and completes when quokka_run falls.

[Vpriority High] [End]

[DDS_STBIO1_0034] Requirement:

During the Soft Reset routine, ADSP shall write the ADSP RAM with the value 0. Only the first 3.5Kbyte of ADSP RAM shall be cleared, from address 0x000 to address 0xDFF

[Vpriority High] [End]

[DDS_STBIO1_0035] Requirement:

If during the start of a new operation, more than one bit is written into CONTROL_A reg in the same write operation, ADSP shall execute only the routine at higher priority and discard the others.

Priority is listed below, with first routine at higher priority:

- SOFT RESET routine

- OTP WRITE routine

- OTP TEST routine

- OTP BOOT routine

[Vpriority High] [End]

[DDS_STBIO1_0036] Requirement:

If bit OTP_PRG10[31] in OTP Registers, called write_protection_en, is set to 1, and STBIO1 is not in debug_mode, register map shall refuse any incoming write request if the transaction address is within the OTP Registers space (from 0xCD000 to 0xCD02F).

[End]

[DDS_STBIO1_0037] Definition:

Figure 16: OTP Writing timing diagram

[End]

### I2C_SPI_AHB integration

| Generic name |  | Type | Value | TOP CONNECTION |  |
| --- | --- | --- | --- | --- | --- |
| fifo_dpt |  |  | 8 | FIFO depth. Suggested val 8 | [DDS_STBIO1_8100] |
| rd_to_max_val |  |  | 6000 | Time available between double byte write to communicate the number of bytes to be read and the following read. If timer expires the operation is aborted and proper flag is activated. Suggested val. 1200 | [DDS_STBIO1_8101] |
| i2c_hs_mode |  |  | 1'b0 | To activate HS I2C | [DDS_STBIO1_8102] |
| i2c_master_code |  |  | 8'h00 | Master code for HS I2C | [DDS_STBIO1_8103] |
| ser_mode |  |  | "i2c_spi" | To choose if the interface is SPI only, I2C only or both. Suggested : i2c_spi | [DDS_STBIO1_8104] |
| ip_version |  |  | 8'haa | IP version | [DDS_STBIO1_8105] |
| i2c_dev_id |  |  | 7'h5f | I2C device address | [DDS_STBIO1_8106] |

Table 23: I2C/SPI AHB Parameters

| Port name | Direction | Type | TOP CONNECTION |
| --- | --- | --- | --- |
| rst_n | input |  | POR |
| i_if_cs | input |  | CS for SPI interface |
| o_if_cs_tud | output |  | Control for CS PAD. 1 in scan mode 0 otherwise |
| o_if_cs_en_n | output |  | Set to 0 for CS pad id bidir pad is used. Could be inverted depending on the PAD |
| o_if_cs_out_n | output |  | Set to 0 for CS pad id bidir pad is used. Could be inverted depending on the PAD |
| i_if_ck | input |  | Interface clock |
| i_if_sda_sdi_in | input |  | SDA/SDI from pad |
| o_if_sda_sdi_en_n | output |  | SDA/SDI pad enable |
| o_if_sda_sdi_out | output |  | SDA/SDI to pad |
| o_ctrl_reg_7 | output | [7:0] | 8 configuration bit set by local register (addr. 0x5e) |
| i_ext_reg_dt | input | [7:0] | Data from external register block (if present) |
| o_ext_reg_wr_add | output | [7:0] | Write address to external register block (if present) |
| o_ext_reg_rd_add | output | [7:0] | Read address to external register block (if present) |
| o_ext_reg_dt | output | [7:0] | Data to external register block (if present) |
| o_ext_reg_ck_wr | output |  | Write enable for external register block (if present) |
| o_ext_reg_we | output |  | Write direct enable for external register block (if present) |
| rst_16m_n |  |  | Sw reset for AHB domain |
| sys_16m_clk | input |  | AHB clock |
| o_hselm | output |  | AHB_M |
| o_haddrm | output | [31:0] | AHB_M |
| o_htransm | output | [1:0] | AHB_M |
| o_hmasterm | output | [3:0] | AHB_M |
| o_hwritem | output |  | AHB_M |
| o_hsizem | output | [2:0] | AHB_M |
| o_hmastlockm | output |  | AHB_M |
| o_hwdatam | output | [31:0] | AHB_M |
| o_hburstm | output | [2:0] | AHB_M |
| o_hprotm | output | [3:0] | AHB_M |
| i_hrdatam | input | [31:0] | AHB_M |
| i_hreadym | input |  | AHB_M |
| i_hrespm | input |  | AHB_M |
| o_gt_ck_on | output |  | Pulse on interface clock to inform system that AHB clock must be switched on |
| o_gt_ck_off | output |  | Pulse on AHB clock to inform system that AHB clock can be switched off |
| o_ahb_error | output |  | Error on AHB side. Active if bit 1 or of register at address 0x4f are active. |
| o_rd_byte_nr_err | output |  | Active when |
| o_req_error | output |  | Active when number of byte is missing for read or timeout between double yte write and read is expired |
| scan_mode | input |  | Scan mode |
| scan_enable | input |  | Scan enable |
| scan_clk | input |  | Scan clk |
| scan_rst | input |  | Scan reset |

Table 24: I2C/SPI AHB I/O List

#### I2C_SPI_AHB Peculiar Requirements

[DDS_STBIO1_8000] Requirement: I2C interface shall support all the specified modalities (Standard, FM, FM+) and, if configured, HS is also supported.

[End]

[DDS_STBIO1_8001] Requirement: Clock frequency ratio between system (AHB) and I2C/SPI interfaces domains shall be at least 2 times.

[TO: IPOS_I2C_SPI_AHB_0006]

[Vpriority High]

[End]

[DDS_STBIO1_8002] Requirement: The IP has a serial interface (I2C or SPI) and a parallel bus interface AHB.

[TO: IPOS_I2C_SPI_AHB_0007]

[End]

[DDS_STBIO1_8003] Requirement: An I2C/SPI write access to the address 0x60-0x67 shall be translated in an AHB write access to the configured AHB address (contained in registers SAM_ADDR_BYTE0/1/2/3, address 0x50/0x51/0x52/0x53).

[TO: IPOS_I2C_SPI_AHB_0009]

[Vpriority High]

[End]

[DDS_STBIO1_8004] Requirement: A double byte write access to the address 0x68-0x6f (with data that is the number of bytes that the master wants to read) shall start the data preload from AHB in order to have the internal fifo not empty when the read request will be done by the master.

[TO: IPOS_I2C_SPI_AHB_0010]

[Vpriority High]

[End]

[DDS_STBIO1_8005] Requirement: An I2C/SPI read access to the address 0x70-0x7f shall be translated in an AHB read access to the configured AHB address (contained in registers SAM_ADDR_BYTE0/1/2/3, address 0x50/0x51/0x52/0x53).

[TO: IPOS_I2C_SPI_AHB_0011]

[Vpriority High]

[End]

[DDS_STBIO1_8006] Requirement: On AHB interface, data size shall be limited to BYTE and WORD.

[TO: IPOS_I2C_SPI_AHB_0361]

[End]

[DDS_STBIO1_8007] Requirement: On AHB interface, BURST shall not be supported as well as LOCKED transfers.

[TO: IPOS_I2C_SPI_AHB_0362]

[End]

### SensorHub integration

| Generic name | Type | Value | TOP CONNECTION |
| --- | --- | --- | --- |
| ADDRWIDTH |  |  |  |
| EBMwriteAddr |  |  |  |
| TAGid |  |  |  |

Table 25: Sensor Hub Parameter

| Port name | Direction | Type | TOP CONNECTION |
| --- | --- | --- | --- |
| scan_mode | input |  |  |
| scan_enable | input |  |  |
| scan_rstn | input |  |  |
| HCLKM | input |  |  |
| HCLKS | input |  |  |
| sys_clk | input |  |  |
| HRESETn | input |  |  |
| DEBUG_MODE_i | input |  |  |
| scl_master_in | input |  |  |
| sda_master_in | input |  |  |
| scl_master_out | output |  |  |
| sda_master_out | output |  |  |
| dvalid_masterblaze_ext | input |  |  |
| dvalid_masterblaze_ext_enable | input |  |  |
| sleep_master | output |  |  |
| master_end_op_pulse | output |  |  |
| SH_SMARTFIFO_OCCUPATION | Input |  |  |
| fifo_mode | input |  |  |
| fifo_offset | input | [11:0] |  |
| HSELS | input |  |  |
| HADDRS | input | [ADDRWIDTH-1:0] |  |
| HTRANSS | input | [1:0] |  |
| HSIZES | input | [2:0] |  |
| HWRITES | input |  |  |
| HREADYS | input |  |  |
| HWDATAS | input | [31:0] |  |
| HREADYOUTS | output |  |  |
| HRESPS | output |  |  |
| HRDATAS | output | [31:0] |  |
| ahb_master_on | output |  |  |
| HRDATAM | input | [31:0] |  |
| HREADYM | input |  |  |
| HRESPM | input | [1:0] |  |
| HGRANTM | input |  |  |
| HSELM | output |  |  |
| HADDRM | output | [31:0] |  |
| HTRANSM | output | [1:0] |  |
| HWRITEM | output |  |  |
| HSIZEM | output | [2:0] |  |
| HBURSTM | output | [2:0] |  |
| HPROTM | output | [3:0] |  |
| HWDATAM | output | [31:0] |  |
| HBUSREQM | output |  |  |
| HLOCKM | output |  |  |

Table 26: Sensor Hub I/O List

[DDS_STBIO1_XXXX] Description

Sensor hub related data flow is composed by the following phases :

  - Initial configuration of STBIO1 registers through I2C/SPI interface

  - I2C master trigger generation through I2C/SPI interface

  - External I2C sensors are read by sensor hub

  - Data read is sent to FIFO

  - Data read can be read through I2C/SPI interface

[End]

[DDS_STBIO1_XXXX] Description

Sensor hub configuration sequence at system level to be added

[End]

#### SensorHub Peculiar Requirements

[DDS_STBIO1_9000] Description

Sensor hub IP is an IP used to read up-to 4 external I2C devices.

Sensor hub IP is compliant with FM+ I2C standard.

Both read and write operation are possible:

- Write:

  - 1 byte on one I2C target

- Read:

  - up-to 12byte on each I2C target

  - up-to 4 I2C target

[End]

[DDS_STBIO1_9317] Requirement

If the sensor hub IP is not enabled, the SCL and SDA pad shall not toggle.

[Vpriority High] [TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9011] Requirement

Input scl_master_in of sensor hub shall be connected to input buffer from sensor hub’s SCL PAD.
[Vpriority High] [TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9012] Requirement

Input sda_master_in of sensor hub shall be connected to input buffer from sensor hub’s SDA PAD.
[Vpriority High] [TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9013] Requirement

Output scl_master_out of sensor hub shall be connected to enable of output buffer of sensor hub’s SCL PAD (in case it is a bi-directional PAD).
[Vpriority High] [TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9014] Requirement

Output sda_master_out of sensor hub shall be connected to enable of output buffer of sensor hub’s SDA PAD.
[Vpriority High] [TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9017] Requirement

The output port sleep_master shall be set to 1 when an I2C operation is not on-going.
[Vpriority High] [TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9004] Requirement

Input sys_clk of sensor hub shall be connected to 8MHz clock to generate I2C communications compliant with FM+ I2C standard.
[TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9018] Requirement

The trigger of sensor hub shall be possible in both ways, HW via dvalid_masterblaze_ext signal or SW using DVALID_REGISTER.
[TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9024] Requirement

Only I2C target addressed through SLAVE0_DEV_ADD register shall be the destination of an I2C Write Operation.
[TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9027] Requirement

The bit 0 of SLAVE1_DEV_ADD, SLAVE2_DEV_ADD, SLAVE3_DEV_ADD registers shall be ignored: ONLY read operations towards I2C target  1, 2 and 3 shall be performed.
[TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9029] Requirement

AUX_SENS_ON possible values shall be 0,1,2,3. All the values shall be tested since MasterBlaze supports up-to 4 I2C targets connected to I2C network.
[TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9030] Requirement

SLAVEx_NUMOP bit-fields of SLAVEx_CONFIG registers shall define how many bytes per slave are read as per effect of an I2C operation (x from 1 to 12).
[TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9031] Requirement

All the values from 1 to 12 shall be valid for SLAVEx_NUMOP bit-fields of SLAVEx_CONFIG registers.
[TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9032] Requirement

SLAVE0_NUMOP bitfield shall be ignored if an I2C Write operation is issued to the I2C Slave 0.
[TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9033] Requirement

All the SENSOR_HUB_x registers shall be reset every time a new I2C operation starts.
[TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9035] Requirement

When Quokka MasterBlaze µP runs (sleep_master signal = 0), any dvalid triggered (HW or SW) shall be ignored.
[TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9034] Requirement

The NACK_REGISTER register is reset every time a new I2C operation starts.
[TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9260] Requirement

Each data stored in FIFO shall be composed by 4 words (16bytes).
[TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9261] Requirement

3 words (12bytes) of the data stored in FIFO shall contain the data from each I2C slave.
[TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9262] Requirement

24-bits of the data stored in FIFO shall be reserved at ‘0’.
[TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9263] Requirement

The last 8-bits of the data stored in FIFO shall contain the TAG field.
[TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9300] Requirement

The AHB Write to registers has no effect if the IP masterblaze processor is busy (I2C operation on-going); the registers value shall not be updated.

[TO: IPOS_Senshub_MB_i2c]

[End]

[DDS_STBIO1_9500] Requirement

In case also other sources that writes into FIFO are active, then sensor hub shall work always in multi-fifo mode (fifo_mode input of sensor hub set to ‘1’).

[End]

### FIFO_CTRL integration

| Generic name | Type | Value | TOP CONNECTION |
| --- | --- | --- | --- |
| AW |  | 13 |  |

Table 27: Fifo Parameter

| Port name |  | Direction | Type | Description | TOP CONNECTION |
| --- | --- | --- | --- | --- | --- |
| TEST_MODE |  | input |  | ScanMode | u_pad_mux. scan_mode |
| HCLK |  | input |  | AHB Master Handler Clock 16MHz clock | u_pmu.clk_fifo |
| HRESETn |  | input |  |  | u_pmu.resetn_fifo |
| HSEL |  | input |  | AHB Slave Port | u_afa_xbar.hsel_M2 |
| HADDR |  | input | [AW-1:0] | AHB Slave Port | u_afa_xbar.haddr_M2 |
| HMASTER |  | input | [3:0] | AHB Slave Port | u_afa_xbar.hmaster_M2 |
| HTRANS |  | input | [1:0] | AHB Slave Port | u_afa_xbar.htrans_M2 |
| HSIZE |  | input | [2:0] | AHB Slave Port | u_afa_xbar.hsize_M2 |
| HWRITE |  | input |  | AHB Slave Port | u_afa_xbar. |
| HWDATA |  | input | [31:0] | AHB Slave Port | u_afa_xbar. |
| HREADY |  | input |  | AHB Slave Port | u_afa_xbar. |
| HREADYOUT |  | output |  | AHB Slave Port | u_afa_xbar. |
| HRDATA |  | output | [31:0] | AHB Slave Port | u_afa_xbar. |
| HRESP |  | output |  | AHB Slave Port | u_afa_xbar. |
| O_FIFO_RAM_A |  | output | [ADDR_WIDTH-1:0] |  | u_ca_fifo_bist_c.ags_1 |
| O_FIFO_RAM_D |  | output | [23:0] |  | u_ca_fifo_bist_c.csns_1 |
| O_FIFO_RAM_CSN |  | output |  |  | u_ca_fifo_bist_c.dgs_1 |
| O_FIFO_RAM_M |  | output | [23:0] |  | u_ca_fifo_bist_c.wens_1 |
| O_FIFO_RAM_WEN |  | output |  |  | u_ca_fifo_bist_c.mgs_1 |
| I_FIFO_RAM_Q |  | input | [23:0] |  | u_ca_fifo_bist_c.dgrs_1 |

Table 28: FIFO Controller I/O List

#### FIFO_CTRL Peculiar Requirements

[DDS_STBIO1_00000102] Requirement

The parameter MEM_LOC shall define the first Memory Location of the FIFO RAM: 0xC4000.

[TO: IPOS_FIFO_CTRL_00000102]

[End]

[DDS_STBIO1_00000202] Requirement

The FIFO register map first memory location shall be defined as MEM_LOC + 0x1000: 0xC5000.

[TO: IPOS_FIFO_CTRL_00000202]

[End]

[DDS_STBIO1_89456557] Requirement:

The FIFO Controller shall be able to address 1.5 kbyte divided into 32-bit address.

[TO: IPOS_FIFO_CTRL_89456557]

[End]

[DDS_STBIO1_89459082] Requirement:

The FIFO Controller shall work at 16 MHz clock frequency.

[TO: IPOS_FIFO_CTRL_89459082]

[End]

[DDS_STBIO1_89455261] Requirement:

The FIFO Controller shall have an AHB Slave Interface connected to the AHB BUS and a Memory Master Interface connected to the Physical Memory.

[TO: IPOS_FIFO_CTRL_89455261]

[End]

[DDS_STBIO1_89455449] Requirement:

The FIFO Controller shall have two working modes: Multiple FIFO and Unique FIFO. These two modes are mutually exclusive.

[TO: IPOS_FIFO_CTRL_89455449]

[End]

[DDS_STBIO1_89455232] Requirement:

As default, the FIFO shall work in Unique FIFO Mode.

[TO: IPOS_FIFO_CTRL_89455232]

[End]

[DDS_STBIO1_89456975] Requirement:

The user shall select the Multiple FIFO mode setting to 1 the first bit into the FIFO_CFG_MODE and 0 for the Unique FIFO mode.

[TO: IPOS_FIFO_CTRL_89456975]

[End]

[DDS_STBIO1_89457435] Requirement:

The FIFO Controller shall have a 6 i_depth input of 8 bit to be used in Multiple FIFO Mode to define the depth of each subFIFOs.

[TO: IPOS_FIFO_CTRL_89457435]

[End]

[DDS_STBIO1_89457790] Requirement:

The FIFO 6 memory locations to access each sub-FIFO in Multiple mode are obtained adding multiples of 256 to the memory base address.

Example: FIFO Memory Base Address = 0xC4000, then:

| FIFO 0 memory location | 0xC4000 |
| --- | --- |
| FIFO 1 memory location | 0xC4100 |
| FIFO 2 memory location | 0xC4200 |
| FIFO 3 memory location | 0xC4300 |
| FIFO 4 memory location | 0xC4400 |
| FIFO 5 memory location | 0xC4500 |

Table 29: SUB FIFO Memory Address

[TO: IPOS_FIFO_CTRL_89457790]

[End]

[DDS_STBIO1_00000008] Requirement:

If input signal HMASTER is set to 0 or 1, the FIFO Controller shall enter Memory mode.

[TO: IPOS_FIFO_CTRL_00000008]

[End]

[DDS_STBIO1_00000199] Requirement

| DATA TYPE | BIT: 23 - 20 | BIT: 19 - 16 | BIT: 15 - 0 |
| --- | --- | --- | --- |
| PPG FRAME 0 | 0000 | 20 BIT DATA |  |
| PPG FRAME 1 | 0001 | 20 BIT DATA |  |
| PPG FRAME 2 | 0010 | 20 BIT DATA |  |
| PPG FRAME 3 | 0011 | 20 BIT DATA |  |
| PPG FRAME 4 | 0100 | 20 BIT DATA |  |
| PPG FRAME 5 | 0101 | 20 BIT DATA |  |
| PPG FRAME 6 | 0110 | 20 BIT DATA |  |
| PPG FRAME 7 | 0111 | 20 BIT DATA |  |
| PPG FRAME 8 | 1000 | 20 BIT DATA |  |
| PPG FRAME 9 | 1001 | 20 BIT DATA |  |
| PPG FRAME 10 | 1010 | 20 BIT DATA |  |
| PPG FRAME 11 | 1011 | 20 BIT DATA |  |
| TIME SLOT DATA | 1111 | 0000 | 16 BIT DATA |
| ECG0_AC | 1100 | 0000 | 16 BIT DATA |
| ECG0_DC | 1101 | 0000 | 16 BIT DATA |
| ECG1_AC | 1100 | 0001 | 16 BIT DATA |
| ECG1_DC | 1101 | 0001 | 16 BIT DATA |
| ECG2_AC | 1100 | 0010 | 16 BIT DATA |
| ECG2_DC | 1101 | 0010 | 16 BIT DATA |
| BIA_AC_P | 1110 | 0001 | 16 BIT DATA |
| BIA_DC_P | 1110 | 0011 | 16 BIT DATA |
| BIA_AC_Q | 1110 | 0000 | 16 BIT DATA |
| BIA_DC_Q | 1110 | 0010 | 16 BIT DATA |
| GSR | 1111 | 1000 | 16 BIT DATA |
| DATA_QUOKKA_ELAB | 1111 | 0001 to 0111 | 16 BIT DATA |

Table 30: Data Tag

00000199

[End]

[DDS_STBIO1_00001199] Requirement

When in Multiple Fifo, the 6 sub-FIFO offset shall coincide with the following table :

| FIFO OFFSET | DATA TYPE |
| --- | --- |
| ECG FIFO OFFSET | ECG0_AC, ECG0_DC |
| BIA FIFO OFFSET | BIA_AC_P, BIA_DC_P, BIA_AC_Q, BIA_DC_Q, ECG1_AC, ECG2_DC |
| GSR FIFO OFFSET | GSR, ECG1_DC, ECG2_AC |
| PPG FIFO OFFSET | PPG |

Table 31 : FIFO Offset Data Table

[END]

ISPU integration

| Por name | Direction | Comment | TOP CONNECTION |
| --- | --- | --- | --- |
| clk | input | clk 16MHz from pmu | u_pmu_clk_stredl_sig |
| main_rst_n | input | reset in 16MHz domain | u_pmu_resetn_stredl_sig |
| POR_n | input | reset in 16MHz domain | u_pmu_resetn_stredl_sig |
| ispu_xbar_sw_resetn | input | Software reset ISPU xbar | u_regmap_otp_ispu_xbar_sw_resetn_sig |
| ispu_core_sw_resetn | input | Software reset ISPU core | u_regmap_otp_ispu_core_sw_resetn_sig |
| scan_mode | input | Scan mode | u_pad_mux_scan_mode_sig |
| scan_enable | input | Scan enable | u_pad_mux_scan_enable_sig |
| test_rst_n | input | Scan reset | u_pad_mux_scan_rstn_sig |
| bist_mode | input | Bist mode enable | u_pad_mux_bist_mode_sig |
| interrupt | input | Interrupt lines | u_ca_fifo_bist_c_oen_1, u_fifo_irq_FIFO_U_M_IRQ_STATUS_sig, u_regmap_otp_irq_SENSHUB_STS_sig, u_regmap_otp_irq_ISPU_MC_INTR_STS_sig, u_regmap_otp_sw_interrupt_sig, u_ca_fifo_bist_c_oen_1, u_ca_fifo_bist_c_oen_1, u_ca_fifo_bist_c_oen_1, u_ca_fifo_bist_c_oen_1, u_ca_fifo_bist_c_oen_1, u_ca_fifo_bist_c_oen_1, u_ca_fifo_bist_c_oen_1, u_ca_fifo_bist_c_oen_1, u_ca_fifo_bist_c_oen_1 |
| clk_en | output | Enable clock at ISPU core | u_ispu_clk_en_sig |
| S_HSEL | input | AHB HSEL Slave Port | u_xbar_afe_hsel_M0_sig |
| S_HREADY | input | AHB HREADY Slave Port | u_xbar_afe_hreadymux_M0_sig |
| S_HREADYOUT | output | AHB HREADYOUT Slave Port | u_ispu_S_HREADYOUT_sig |
| S_HTRANS | input | AHB HTRANS Slave Port | u_xbar_afe_htrans_M0_sig |
| S_HADDR | input | AHB HADRR Slave Port | u_xbar_afe_haddr_M0_sig |
| S_HSIZE | input | AHB HSIZE Slave Port | u_xbar_afe_hsize_M0_sig[1:0] |
| S_HWRITE | input | AHB HWRITE Slave Port | u_xbar_afe_hwrite_M0_sig |
| S_HWDATA | input | AHB HWDATA Slave Port | u_xbar_afe_hwdata_M0_sig |
| S_HRDATA | output | AHB HRDATA Slave Port | u_ispu_S_HRDATA_sig |
| S_HRESP | output | AHB HRESP Slave Port | u_ispu_S_HRESP_sig |
| M_HSEL | output | AHB HSEL Master Port | u_ispu_M_HSEL_sig |
| M_HREADY | input | AHB HREADY Master Port | u_xbar_afe_hready_S0_sig |
| M_HTRANS | output | AHB HTRANS Master Port | u_ispu_M_HTRANS_sig |
| M_HADDR | output | AHB HADDR Master Port | u_ispu_M_HADDR_sig |
| M_HSIZE | output | AHB HSIZE Master Port | u_ispu_M_HSIZE_sig |
| M_HWRITE | output | AHB HWRITE Master Port | u_ispu_M_HWRITE_sig |
| M_HWDATA | output | AHB HWDATA Master Port | u_ispu_M_HWDATA_sig |
| M_HRDATA | input | AHB HRDATA Master Port | u_xbar_afe_hrdata_S0_sig |
| M_HRESP | input | AHB HRESP Master Port | u_xbar_afe_hresp_S0_sig |
| DBG_HREADY | input | AHB HREADY Debug Master Port | u_xbar_afe_hready_S4_sig |
| DBG_HTRANS | output | AHB HTRANS Debug Master Port | u_ispu_DBG_HTRANS_sig |
| DBG_HADDR | output | AHB HADDR Debug Master Port | u_ispu_DBG_HADDR_sig |
| DBG_HSIZE | output | AHB HSIZE Debug Master Port | u_ispu_DBG_HSIZE_sig |
| DBG_HWRITE | output | AHB HWRITE Debug Master Port | u_ispu_DBG_HWRITE_sig |
| DBG_HBURST | output | AHB DHBURST ebug Master Port | NO CONN |
| DBG_HWDATA | output | AHB HWDATA Debug Master Port | u_ispu_DBG_HWDATA_sig |
| DBG_HRDATA | input | AHB HRDATA Debug Master Port | u_xbar_afe_hrdata_S4_sig |
| DBG_HRESP | input | AHB HRESP Debug Master Port | u_xbar_afe_hresp_S4_sig |
| trst_n | input | JTAG Test Reset | u_pad_mux_ispu_trst_n_sig |
| tck | input | JTAG Test Clock | u_pad_mux_ispu_tck_sig |
| tms | input | JTAG Test Mode Select | u_pad_mux_ispu_tms_sig |
| tdi | input | JTAG Test Data In | u_pad_mux_ispu_tdi_sig |
| tdo | output | JTAG Test Data Out | u_ispu_tdo_sig |
| tdo_en | output | JTAG Test Data Out Enable | u_ispu_tdo_en_sig |
| ca_stredl_bist_c_rst_bistctlr_n | input | BIST Asynchronous Reset Pin for collar Serial Interface and Controller | u_pmu_tst_rst_bistctlr_n_sig |
| ca_stredl_bist_c_clk_bistctlr | input | BIST Low Frequency Serial Shift Clock for the controller | u_pmu_tst_clk_bistctlr_sig |
| ca_stredl_bist_c_bist_mode | input | BIST Global BIST Mode Selection Bus | u_ta_afe_bist_crtl_bist_mode_ctlr_sig |
| ca_stredl_bist_c_march_shift | input | BIST March element serial data out enable to SP/ROM/RF/DP collars | u_ta_afe_bist_crtl_march_shiftSP_sig |
| ca_stredl_bist_c_collar_config | input | BIST Mode selection pins for SP/ROM/RF/DP memory collars (modefunc, modestab, modecoll) | u_ta_afe_bist_crtl_collar_configSP_sig |
| ca_stredl_bist_c_march_in | input | BIST March element serial data in | u_ta_afe_bist_crtl_march_outSP_sig |
| ca_stredl_bist_c_march_run | input | BIST March run to SP/ROM/RF/DP collars | u_ta_afe_bist_crtl_march_runSP_sig |
| ca_stredl_bist_c_memsafe | input | BIST Pin to globally deactivate the selected memories by making CSN high internally | u_ta_afe_bist_crtl_memsafe_ctlr_sig |
| ca_stredl_bist_c_sif_reg_en | input | BIST Enable for various types of data transfer (Bitmap, memory selection, programmable algorithms) through serial interface. | u_ta_afe_bist_crtl_sif_reg_en_ctlr_sig |
| ca_stredl_bist_c_sif_load_en | input | BIST Allows to control the data loading from collar registers to serial interface registers of the collar | u_ta_afe_bist_crtl_sif_load_en_ctlr_sig |
| ca_stredl_bist_c_shift_en | input | BIST Enable for Serial Input Data from Tester/External World | u_ta_afe_bist_crtl_shift_en_ctlr_sig |
| ca_stredl_bist_c_sif_update_en | input | BIST Allows to control the data loading from collar registers to serial interface registers of the collar | u_ta_afe_bist_crtl_sif_update_en_ctlr_sig |
| ca_stredl_bist_c_march_end | output | BIST March end signal from each SP / ROM / RF / DP collar type | u_ispu_ca_stredl_bist_c_march_end_sig |
| ca_stredl_bist_c_bbad_1 | output | BIST Indicates bbad for a specific memory. There are as many bbad signals as the number of memories. | NO CONN |
| ca_stredl_bist_c_bfail_1 | output | BIST Indicates bfail for a specific memory. There are as many bbad signals as the number of memories. | u_ispu_ca_stredl_bist_c_bfail_1_sig |
| ca_stredl_bist_c_repairable_1 | output | BIST Indicates repairable for a specific memory.There are as many repairable signals as the number of memories. For a shared collar there would be as many repairable outputs as the number of memories | NO CONN |
| ca_stredl_bist_c_bbad_2 | output | BIST Indicates bbad for a specific memory. There are as many bbad signals as the number of memories. | NO CONN |
| ca_stredl_bist_c_bfail_2 | output | BIST Indicates bfail for a specific memory. There are as many bbad signals as the number of memories. | u_ispu_ca_stredl_bist_c_bfail_2_sig |
| ca_stredl_bist_c_repairable_2 | output | BIST Indicates repairable for a specific memory.There are as many repairable signals as the number of memories. For a shared collar there would be as many repairable outputs as the number of memories | NO CONN |
| ca_stredl_bist_c_bbad_3 | output | BIST Indicates bbad for a specific memory. There are as many bbad signals as the number of memories. | NO CONN |
| ca_stredl_bist_c_bfail_3 | output | BIST Indicates bfail for a specific memory. There are as many bbad signals as the number of memories. | u_ispu_ca_stredl_bist_c_bfail_3_sig |
| ca_stredl_bist_c_repairable_3 | output | BIST Indicates repairable for a specific memory.There are as many repairable signals as the number of memories. For a shared collar there would be as many repairable outputs as the number of memories | NO CONN |
| ca_stredl_bist_c_bbad_4 | output | BIST Indicates bbad for a specific memory. There are as many bbad signals as the number of memories. | NO CONN |
| ca_stredl_bist_c_bfail_4 | output | BIST Indicates bfail for a specific memory. There are as many bbad signals as the number of memories. | u_ispu_ca_stredl_bist_c_bfail_4_sig |
| ca_stredl_bist_c_repairable_4 | output | BIST Indicates repairable for a specific memory.There are as many repairable signals as the number of memories. For a shared collar there would be as many repairable outputs as the number of memories | NO CONN |
| ca_stredl_bist_c_serial_out | output | BIST Serial Output Data from Tester / External World | u_ispu_ca_stredl_bist_c_serial_out_sig |
| ca_stredl_bist_c_bypass | input | BIST Memory Bypass Pin for RAM type memories. One top level port per collar. | u_pad_mux_scan_mode_sig |
| ca_stredl_bist_c_serial_in | input | BIST Serial Input Data from Tester / External World | u_ta_afe_bist_crtl_serial_out_ctlr_sig |
| ca_stredl_bist_c_test_se | input | BIST Scan Enable Signal Used internally by the BIST to prevent memory operation during scan shifting (a dedicated test_se (one test_se per collar) is provided at the top level | u_pad_mux_scan_enable_sig |
| ca_stredl_bist_c_tst_gatedclock | input | BIST Clock Gating Signal for the entire BIST. Used to force clock gating cell transparency in ATPG mode . | u_pad_mux_scan_enable_sig |
| ca_stredl_bist_c_tst_reset_mux | input | BIST Bypass for reset synchronizer. allows to force reset synchronizing circuit transparency in ATPG mode. | u_pad_mux_scan_mode_sig |
| ca_stredl_bist_c_oen_1 | input | BIST Memory Output Enable 1 | u_ca_fifo_bist_c_oen_1 |
| ca_stredl_bist_c_oen_2 | input | BIST Memory Output Enable 2 | u_ca_fifo_bist_c_oen_1 |
| ca_stredl_bist_c_oen_3 | input | BIST Memory Output Enable 3 | u_ca_fifo_bist_c_oen_1 |
| ca_stredl_bist_c_oen_4 | input | BIST Memory Output Enable 4 | u_ca_fifo_bist_c_oen_1 |
| ca_stredl_bist_c_mem_atpg_mode | input | BIST For ATPG purpose: Used to constraint few memory inputs and improve controllability and observability. | u_ca_fifo_bist_c_oen_1 |
| ca_stredl_bist_c_tst_mem_selck_1 | input | BIST To select the clock and memory ports in ATPG mode 0: Functional Clock / Functional Ports 1: Test Clock/ Test Ports | u_ca_fifo_bist_c_oen_1 |
| ca_stredl_bist_c_tst_mem_selck_2 | input | BIST To select the clock and memory ports in ATPG mode 0: Functional Clock / Functional Ports 1: Test Clock/ Test Ports | u_ca_fifo_bist_c_oen_1 |
| ca_stredl_bist_c_tst_mem_selck_3 | input | BIST To select the clock and memory ports in ATPG mode 0: Functional Clock / Functional Ports 1: Test Clock/ Test Ports | u_ca_fifo_bist_c_oen_1 |
| ca_stredl_bist_c_tst_mem_selck_4 | input | BIST To select the clock and memory ports in ATPG mode 0: Functional Clock / Functional Ports 1: Test Clock/ Test Ports | u_ca_fifo_bist_c_oen_1 |
| ca_stredl_bist_c_par_memsel | input | BIST Parallel Memory Selection Bus | { 1'b0, u_ca_fifo_bist_c_oen_1, u_ca_fifo_bist_c_oen_1, u_ca_fifo_bist_c_oen_1 } |

Table 32: ISPU I/O List

### Requirements

[DDS_STBIO1_0500] Requirement: ] Requirement:

Interrupt lines list:

| Interrupt | bit | Description |
| --- | --- | --- |
| irq_FIFO_U_M_IRQ_STsATUS | 29 | FIFO status [DDS_STBIO1_0501] |
| irq_SENSHUB_STS | 28 | SENSOR HUB status [DDS_STBIO1_0502] |
| irq_ISPU_MC_INTR_STS | 27 | ECG, BIA or PPG samples ready for elaboration[DDS_STBIO1_0503] |
| sw_interrupt | 26:11 | 16 sw interrupt used by ADSP or external user [DDS_STBIO1_0504] |
| irq_timer | 1 | Internal timer [DDS_STBIO1_0505] |

[END]

[DDS_STBIO1_0506] Description:

The ISPU is a dedicated IP for implementing complex processing algorithms. By default, it is turned off. This is located inside a power island

Figure 17: ISPU turn on phases

[End]

[DDS_STBIO1_0507] Requirement:

After the initial boot phase, to turn on the ISPU shall be:

- Select page 3 of the regmap (ADDRESS=0x04, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable user mode by writing to regmap (ADDRESS=0x5E, DATA=0x01)

- Enable the power island by writing to regmap POWER CONFIGURATION REGISTER via AHB interface (ADDRESS=0xCA000, DATA=0x01)

[End]

Figure 18: Timing turn on ISPU

[DDS_STBIO1_0508] Requirement:

To turn off the ISPU shall be:

- Disable the power island by writing to regmap POWER CONFIGURATION REGISTER via AHB interface (ADDRESS=0xCA000, DATA=0x00)

[End]

[DDS_STBIO1_0509] Requirement:

To put the ISPU into sleep mode, it is necessary to perform a read at address 0x80054.

[End]

[DDS_STBIO1_05010] Requirement:

To wake up the ISPU, it is necessary to trigger an interrupt.

[End]

[DDS_STBIO1_05011] Requirement:

When the ISPU is in sleep mode, it is possible to perform reads and writes from the data and code memory by triggering an AHB transition.

[End]

| Stato corrente | TMS = 0 | TMS = 1 |
| --- | --- | --- |
| **Test-Logic-Reset** | Run-Test/Idle | Test-Logic-Reset |
| **Run-Test/Idle** | Run-Test/Idle | Select-DR-Scan |
| **Select-DR-Scan** | Capture-DR | Select-IR-Scan |
| **Capture-DR** | Shift-DR | Exit1-DR |
| **Shift-DR** | Shift-DR | Exit1-DR |
| **Exit1-DR** | Pause-DR | Update-DR |
| **Pause-DR** | Pause-DR | Exit2-DR |
| **Exit2-DR** | Shift-DR | Update-DR |
| **Update-DR** | Run-Test/Idle | Select-DR-Scan |
| **Select-IR-Scan** | Capture-IR | Test-Logic-Reset |
| **Capture-IR** | Shift-IR | Exit1-IR |
| **Shift-IR** | Shift-IR | Exit1-IR |
| **Exit1-IR** | Pause-IR | Update-IR |
| **Pause-IR** | Pause-IR | Exit2-IR |
| **Exit2-IR** | Shift-IR | Update-IR |
| **Update-IR** | Run-Test/Idle | Select-DR-Scan |

STBIO1 Register Map

### STBIO1 Register Map

Here is described the list of registers in the register map. The AFE_BLOCK registers are accessible by I2C/SPI direct access and AHB interface, while the others are only by AHB.

The I2C/SPI direct access mode is done by the HOST INTF port (I2C/SPI AHB) and the address of the AFE_BLOCK registers are arranged in 4 pages of 64 registers.

| BASE START ADDRESS​ | ADDRESS BLOCK​ | REGISTER WIDTH | BASE END ADDRE VREF_TRIM SS | RESET | REQ_ID |
| --- | --- | --- | --- | --- | --- |
| 0xC8000​ | AFE_BLOCK​ | 8​ | 0xC80EB | tst_main_reset_n, regmap_sw_resetn | DDS_STBIO1_3000 |
| 0xC80EC | RESERVED |  | 0xC8FFF | tst_main_reset_n, regmap_sw_resetn | DDS_STBIO1_3001 |
| 0xC9000​ | OTP REGISTER​ | 8​ | 0xC902C | tst_main_reset_n | DDS_STBIO1_3002 |
| 0xC9030​ | RESERVED |  | 0xC9FFF | tst_main_reset_n, regmap_sw_resetn | DDS_STBIO1_3003 |
| 0xCA000​ | SYSTEM_REGISTER​ | 32​ | 0xCA140 | tst_main_reset_n, regmap_sw_resetn | DDS_STBIO1_3004 |
| 0xCA144 | RESERVED |  | 0xCCFFF | tst_main_reset_n, regmap_sw_resetn | DDS_STBIO1_3005 |
| 0xCD000​ | OTP (TRIMM REGS)​ | 32​ | 0xCD02F | tst_main_reset_n, regmap_sw_resetn | DDS_STBIO1_3006 |

Table 33: Address Register Map

The device is configurable/readable by the I2C/AHB bridge. This IP is in charge of take an I2C/SPI access and (eventually) transform it into an AHB transaction, to allow the configuration/reading of the device using the ISPU.

The only registers that are accessible also by an I2C/SPI direct access, are the ones of the AFE BLOCK section.

To reach a very Low-Power consumption, the AHB interface is disabled as default.

The I2C/SPI direct access mode is done by the HOST INTF port (I2C/SPI AHB) and the address of the AFE_BLOCK registers are arranged in 4 pages of 64 registers (Table 24).

In the following table are the base address and the size of each one page.

| BASE ADDRESS​ | SIZE | PAGE |
| --- | --- | --- |
| 0x00​ | 0x3F |  | 0​ |  |  |
| 0x40​ | 0x3F |  | 1​ |  |  |
| 0x80​ | 0x3F​ |  | 2​ |  |  |
| 0xC0​ | 0x3F​ |  | 3​ |  |  |

Table 34: Location AFE_BLOCK Registers on the I2C/SPI pages.

For direct access operation two I2C/SPI operations need for reading/writing an AFE_BLOCK register:

- Page Selection -> I2C write operation for writing the 2 bits of the PAGE_SEL (address 0x40 of I2C/SPI AHB Bridge)

- AFE_BLOCK register R/W -> Read/Write the AFE_BLOCK register with the address reported in one of 4 pages.

  - e.g.: for reading the DEVICE_STATUS_REG (Register Map address 0x0096) the PAGE_SEL=0x02, I2C/SPI direct access address=0x16

To enable AHB Access, it needs using I2C/SPI direct access mode, for:

- First turn-on the LDO1V8 and the Fast-Ring Oscillator (set EN_LDO1V8 and EN_CLKF of VIREF_LDO1V8_CLK16M, see address in the Table 34).

- Check the Fast Clock(16MHz),  is stable  (when the Fast-Ring Oscillator is stable, the CLK_READY bit in DEVICE_CONFIG_OPERATION (0x0000) register is set to 1).

Now it’s possible to configure the registers that are mapped on AHB_Interface as explained in the following example:

- Run an I2C/SPI transaction, with the type of AHB access word/byte, at the address 0x59 (one of I2C/SPI Internal Configuration Registers).

- Run an I2C/SPI transaction, with the register address that you want to configure, at the address 0x50 (one of I2C/SPI Internal Configuration Registers), starting from the LSB. In other word, if you need to access at register VIREF_LDO1V8_CLK16M at AHB address 0x000C_00CF, you have to perform four I2C/SPI accesses to write:

  - At address 0x50  0xCF

  - At address 0x51  0x00

  - At address 0x52  0x0C

  - At address 0x53 à 0x00

- Run an I2C/SPI write, of the new data, at address 0x60 (one of I2C/SPI Internal Configuration Registers).

- Run an I2C/SPI read transition, with the number of byte that you read, at address 0x68 (one of I2C/SPI Internal Configuration Registers).

- Run an I2C/SPI transaction with the data that you want to read, at address loaded into the previous step, at the address 0x70 (one of I2C/SPI Internal Configuration Registers).

It’s possible remap the whole AFE register map with AHB addresses (column “AHB Start Address” of Table 34), included the AFE_BLOCK block: in this case each registers is accessible with own AHB address (AHB start address + offset) and the I2C/SPI access to these registers must follow the previous procedure. To do that:

- Set the user_mode at 1 (set first bit-field of CTRL_REG_7 (0x5E), one of I2C/SPI Internal Configuration Registers).

This procedure is useful when you are thinking of configuring the STBIO1 by ISPU (with a code previously charged in code memory), leaving to host interface (I2C/SPI) the rule to collect the data from FIFO after AFE sampling and processing phases.

The same procedure can be used also to consider the Regmap only for AHB, allowing the host interface to convert the I2C/SPI Transaction into AHB Transaction.

Pay Attention: is strictly recommended to change the user_mode only when the Device is in IDLE STATE (check DEVICE_STATUS_REGISTER in REGMAP).

### STBIO1 Register Map Peculiar Requirements

[DDS_STBIO1_4000] Requirement:

Write enable for each OTP REGISTER shall be 0, in functional mode, if bit 31 of  OTP_PRG10 is set 1.

[Vpriority High] [End]

#### AFE_BLOCK Register Summary

| Offset | Register Name | Register Title | Direct Access | PAGE SEL |
| --- | --- | --- | --- | --- |
| 0x0000 | DEVICE_CONFIG_OPERATION | DEVICE_CONFIG_OPERATION register | 0x00 | 0x00 |
| 0x0001 | DEVICE_CONFIG_CHANNEL | DEVICE_CONFIG_CHANNEL register | 0x01 |  |
| 0x0002 | SET_ECG_FREQ_l | SET_ ECG_FREQ_l register | 0x02 |  |
| 0x0003 | SET_ECG_FREQ_h | SET_ ECG_FREQ_h register | 0x03 |  |
| 0x0004 | SET_BIA_FREQ_l | SET_BIA_FREQ_l register | 0x04 |  |
| 0x0005 | SET_BIA_FREQ_h | SET_BIA_FREQ_h register | 0x05 |  |
| 0x0006 | SET_GSR_FREQ_l | SET_GSR_FREQ_l register | 0x06 |  |
| 0x0007 | SET_GSR_FREQ_h | SET_GSR_FREQ_h register | 0x07 |  |
| 0x0008 | SET_PPG_FREQ_l | SET_PPG_FREQ_l register | 0x08 |  |
| 0x0009 | SET_PPG_FREQ_h | SET_PPG_FREQ_h register | 0x09 |  |
| 0x000A | ADC_GENERAL_CONFIG | ADC_GENERAL_CONFIG register | 0x0A |  |
| 0x000B | ADC_CAL_CONFIG | ADC_CAL_CONFIG register | 0x0B |  |
| 0x000C | ADC_CONFIG_ECG | ADC_CONFIG_ECG register | 0x0C |  |
| 0x000D | ADC_CONFIG_PPG | ADC_CONFIG_PPG register | 0x0D |  |
| 0x000E | LDO1V2 | LDO1V2 register | 0x0E |  |
| 0x000F | DEVICE_ID_REG | DEVICE ID REGISTER | 0x0F |  |
| 0x0010 | RLD_0 | RLD_0 register | 0x10 |  |
| 0x0011 | RLD_1_DCCK_0 | RLD_1_DCCK_0 register | 0x11 |  |
| 0x0012 | DCCK_1 | DCCK_1 register | 0x12 |  |
| 0x0013 | CURR_INJ | CURR_INJ register | 0x13 |  |
| 0x0014 | CONTROL_CIP_CIN | CONTROL_CIP_CIN register | 0x14 |  |
| 0x0015 | IMP_OVERCURR | IMP_OVERCURR register | 0x15 |  |
| 0x0016 | BIO_BUF_AND_BIO_ASYNC_RST | BIO_BUF_AND_BIO_ASYNC_RST register | 0x16 |  |
| 0x0017 | IN_MUX | IN_MUX register | 0x17 |  |
| 0x0018 | ECG_CTRL | ECG_CTRL register | 0x18 |  |
| 0x0019 | IMP_CTRL | IMP_CTRL register | 0x19 |  |
| 0x001A | GSR_CTRL | GSR_CTRL register | 0x1A |  |
| 0x001B | EN_CHOP_FREQ_MOD_DEMOD | EN_CHOP_FREQ_MOD_DEMOD register | 0x1B |  |
| 0x001C | PPG_ALC_CONFIG_PARAM_1 | PPG_ALC_CONFIG_PARAM_1 register | 0x1C |  |
| 0x001D | PPG_ALC_CONFIG_PARAM_2 | PPG_ALC_CONFIG_PARAM_2 register | 0x1D |  |
| 0x001E | DIGITAL_RAMP_CONFIG_PARAM_1 | DIGITAL_RAMP_CONFIG_PARAM_1 register | 0x1E |  |
| 0x001F | DIGITAL_RAMP_CONFIG_PARAM_2 | DIGITAL_RAMP_CONFIG_PARAM_2 register | 0x1F |  |
| 0x0020 | DIGITAL_RAMP_CONFIG_PARAM_3 | DIGITAL_RAMP_CONFIG_PARAM_3 register | 0x20 |  |
| 0x0021 | GENERAL_PPG_PARAMETERS_1 | GENERAL_PPG_PARAMETERS_1 register | 0x21 |  |
| 0x0023 | GENERAL_PPG_PARAMETERS_2 | GENERAL_PPG_PARAMETERS_2 register | 0x23 |  |
| 0x0024 | GENERAL_PPG_PARAMETERS_3 | GENERAL_PPG_PARAMETERS_3 register | 0x24 |  |
| 0x0025 | FRAME_1_PARAMETERS_1 | FRAME_1_PARAMETERS_1 register | 0x25 |  |
| 0x0026 | FRAME_2_PARAMETERS_1 | FRAME_2_PARAMETERS_1 register | 0x26 |  |
| 0x0027 | FRAME_3_PARAMETERS_1 | FRAME_3_PARAMETERS_1 register | 0x27 |  |
| 0x0028 | FRAME_4_PARAMETERS_1 | FRAME_4_PARAMETERS_1 register | 0x28 |  |
| 0x0029 | FRAME_5_PARAMETERS_1 | FRAME_5_PARAMETERS_1 register | 0x29 |  |
| 0x002A | FRAME_6_PARAMETERS_1 | FRAME_6_PARAMETERS_1 register | 0x2A |  |
| 0x002B | FRAME_7_PARAMETERS_1 | FRAME_7_PARAMETERS_1 register | 0x2B |  |
| 0x002C | FRAME_8_PARAMETERS_1 | FRAME_8_PARAMETERS_1 register | 0x2C |  |
| 0x002D | FRAME_9_PARAMETERS_1 | FRAME_9_PARAMETERS_1 register | 0x2D |  |
| 0x002E | FRAME_10_PARAMETERS_1 | FRAME_10_PARAMETERS_1 register | 0x2E |  |
| 0x002F | FRAME_11_PARAMETERS_1 | FRAME_11_PARAMETERS_1 register | 0x2F |  |
| 0x0030 | FRAME_12_PARAMETERS_1 | FRAME_12_PARAMETERS_1 register | 0x30 |  |
| 0x0031 | FRAME_1_PARAMETERS_2 | FRAME_1_PARAMETERS_2 register | 0x31 |  |
| 0x0032 | FRAME_2_PARAMETERS_2 | FRAME_2_PARAMETERS_2 register | 0x32 |  |
| 0x0033 | FRAME_3_PARAMETERS_2 | FRAME_3_PARAMETERS_2 register | 0x33 |  |
| 0x0034 | FRAME_4_PARAMETERS_2 | FRAME_4_PARAMETERS_2 register | 0x34 |  |
| 0x0035 | FRAME_5_PARAMETERS_2 | FRAME_5_PARAMETERS_2 register | 0x35 |  |
| 0x0036 | FRAME_6_PARAMETERS_2 | FRAME_6_PARAMETERS_2 register | 0x36 |  |
| 0x0037 | FRAME_7_PARAMETERS_2 | FRAME_7_PARAMETERS_2 register | 0x37 |  |
| 0x0038 | FRAME_8_PARAMETERS_2 | FRAME_8_PARAMETERS_2 register | 0x38 |  |
| 0x0039 | FRAME_9_PARAMETERS_2 | FRAME_9_PARAMETERS_2 register | 0x39 |  |
| 0x003A | FRAME_10_PARAMETERS_2 | FRAME_10_PARAMETERS_2 register | 0x3A |  |
| 0x003B | FRAME_11_PARAMETERS_2 | FRAME_11_PARAMETERS_2 register | 0x3B |  |
| 0x003C | FRAME_12_PARAMETERS_2 | FRAME_12_PARAMETERS_2 register | 0x3C |  |
| 0x003D | FRAME_1_PARAMETERS_3 | FRAME_1_PARAMETERS_3 register | 0x3D |  |
| 0x003E | FRAME_2_PARAMETERS_3 | FRAME_2_PARAMETERS_3 register | 0x3E |  |
| 0x003F | FRAME_3_PARAMETERS_3 | FRAME_3_PARAMETERS_3 register | 0x3F |  |
| 0x0040 | FRAME_4_PARAMETERS_3 | FRAME_4_PARAMETERS_3 register | 0x00 | 0x01 |
| 0x0041 | FRAME_5_PARAMETERS_3 | FRAME_5_PARAMETERS_3 register | 0x01 |  |
| 0x0042 | FRAME_6_PARAMETERS_3 | FRAME_6_PARAMETERS_3 register | 0x02 |  |
| 0x0043 | FRAME_7_PARAMETERS_3 | FRAME_7_PARAMETERS_3 register | 0x03 |  |
| 0x0044 | FRAME_8_PARAMETERS_3 | FRAME_8_PARAMETERS_3 register | 0x04 |  |
| 0x0045 | FRAME_9_PARAMETERS_3 | FRAME_9_PARAMETERS_3 register | 0x05 |  |
| 0x0046 | FRAME_10_PARAMETERS_3 | FRAME_10_PARAMETERS_3 register | 0x06 |  |
| 0x0047 | FRAME_11_PARAMETERS_3 | FRAME_11_PARAMETERS_3 register | 0x07 |  |
| 0x0048 | FRAME_12_PARAMETERS_3 | FRAME_12_PARAMETERS_3 register | 0x08 |  |
| 0x0049 | FRAME_1_PARAMETERS_4 | FRAME_1_PARAMETERS_4 register | 0x09 |  |
| 0x004A | FRAME_2_PARAMETERS_4 | FRAME_2_PARAMETERS_4 register | 0x0A |  |
| 0x004B | FRAME_3_PARAMETERS_4 | FRAME_3_PARAMETERS_4 register | 0x0B |  |
| 0x004C | FRAME_4_PARAMETERS_4 | FRAME_4_PARAMETERS_4 register | 0x0C |  |
| 0x004D | FRAME_5_PARAMETERS_4 | FRAME_5_PARAMETERS_4 register | 0x0D |  |
| 0x004E | FRAME_6_PARAMETERS_4 | FRAME_6_PARAMETERS_4 register | 0x0E |  |
| 0x004F | FRAME_7_PARAMETERS_4 | FRAME_7_PARAMETERS_4 register | 0x0F |  |
| 0x0050 | FRAME_8_PARAMETERS_4 | FRAME_8_PARAMETERS_4 register | 0x10 |  |
| 0x0051 | FRAME_9_PARAMETERS_4 | FRAME_9_PARAMETERS_4 register | 0x11 |  |
| 0x0052 | FRAME_10_PARAMETERS_4 | FRAME_10_PARAMETERS_4 register | 0x12 |  |
| 0x0053 | FRAME_11_PARAMETERS_4 | FRAME_11_PARAMETERS_4 register | 0x13 |  |
| 0x0054 | FRAME_12_PARAMETERS_4 | FRAME_12_PARAMETERS_4 register | 0x14 |  |
| 0x0055 | FRAME_1_PARAMETERS_5 | FRAME_1_PARAMETERS_5 register | 0x15 |  |
| 0x0056 | FRAME_2_PARAMETERS_5 | FRAME_2_PARAMETERS_5 register | 0x16 |  |
| 0x0057 | FRAME_3_PARAMETERS_5 | FRAME_3_PARAMETERS_5 register | 0x17 |  |
| 0x0058 | FRAME_4_PARAMETERS_5 | FRAME_4_PARAMETERS_5 register | 0x18 |  |
| 0x0059 | FRAME_5_PARAMETERS_5 | FRAME_5_PARAMETERS_5 register | 0x19 |  |
| 0x005A | FRAME_6_PARAMETERS_5 | FRAME_6_PARAMETERS_5 register | 0x1A |  |
| 0x005B | FRAME_7_PARAMETERS_5 | FRAME_7_PARAMETERS_5 register | 0x1B |  |
| 0x005C | FRAME_8_PARAMETERS_5 | FRAME_8_PARAMETERS_5 register | 0x1C |  |
| 0x005D | FRAME_9_PARAMETERS_5 | FRAME_9_PARAMETERS_5 register | 0x1D |  |
| 0x005E | FRAME_10_PARAMETERS_5 | FRAME_10_PARAMETERS_5 register | 0x1E |  |
| 0x005F | FRAME_11_PARAMETERS_5 | FRAME_11_PARAMETERS_5 register | 0x1F |  |
| 0x0060 | FRAME_12_PARAMETERS_5 | FRAME_12_PARAMETERS_5 register | 0x20 |  |
| 0x0061 | FRAME_1_PARAMETERS_6 | FRAME_1_PARAMETERS_6 register | 0x21 |  |
| 0x0062 | FRAME_2_PARAMETERS_6 | FRAME_2_PARAMETERS_6 register | 0x22 |  |
| 0x0063 | FRAME_3_PARAMETERS_6 | FRAME_3_PARAMETERS_6 register | 0x23 |  |
| 0x0064 | FRAME_4_PARAMETERS_6 | FRAME_4_PARAMETERS_6 register | 0x24 |  |
| 0x0065 | FRAME_5_PARAMETERS_6 | FRAME_5_PARAMETERS_6 register | 0x25 |  |
| 0x0066 | FRAME_6_PARAMETERS_6 | FRAME_6_PARAMETERS_6 register | 0x26 |  |
| 0x0067 | FRAME_7_PARAMETERS_6 | FRAME_7_PARAMETERS_6 register | 0x27 |  |
| 0x0068 | FRAME_8_PARAMETERS_6 | FRAME_8_PARAMETERS_6 register | 0x28 |  |
| 0x0069 | FRAME_9_PARAMETERS_6 | FRAME_9_PARAMETERS_6 register | 0x29 |  |
| 0x006A | FRAME_10_PARAMETERS_6 | FRAME_10_PARAMETERS_6 register | 0x2A |  |
| 0x006B | FRAME_11_PARAMETERS_6 | FRAME_11_PARAMETERS_6 register | 0x2B |  |
| 0x006C | FRAME_12_PARAMETERS_6 | FRAME_12_PARAMETERS_6 register | 0x2C |  |
| 0x006D | FRAME_1_PARAMETERS_7 | FRAME_1_PARAMETERS_7 register | 0x2D |  |
| 0x006E | FRAME_2_PARAMETERS_7 | FRAME_2_PARAMETERS_7 register | 0x2E |  |
| 0x006F | FRAME_3_PARAMETERS_7 | FRAME_3_PARAMETERS_7 register | 0x2F |  |
| 0x0070 | FRAME_4_PARAMETERS_7 | FRAME_4_PARAMETERS_7 register | 0x30 |  |
| 0x0071 | FRAME_5_PARAMETERS_7 | FRAME_5_PARAMETERS_7 register | 0x31 |  |
| 0x0072 | FRAME_6_PARAMETERS_7 | FRAME_6_PARAMETERS_7 register | 0x32 |  |
| 0x0073 | FRAME_7_PARAMETERS_7 | FRAME_7_PARAMETERS_7 register | 0x33 |  |
| 0x0074 | FRAME_8_PARAMETERS_7 | FRAME_8_PARAMETERS_7 register | 0x34 |  |
| 0x0075 | FRAME_9_PARAMETERS_7 | FRAME_9_PARAMETERS_7 register | 0x35 |  |
| 0x0076 | FRAME_10_PARAMETERS_7 | FRAME_10_PARAMETERS_7 register | 0x36 |  |
| 0x0077 | FRAME_11_PARAMETERS_7 | FRAME_11_PARAMETERS_7 register | 0x37 |  |
| 0x0078 | FRAME_12_PARAMETERS_7 | FRAME_12_PARAMETERS_7 register | 0x38 |  |
| 0x0079 | SAVE_FRAME_DATA_RAW | SAVE_FRAME_DATA_RAW register | 0x39 |  |
| 0x0080 | DIGITAL_RAMP_DEBUG_REG | DIGITAL_RAMP_DEBUG_REG register | 0x00 | 0x02 |
| 0x0086 | PPG_DEBUG_REG_0 | PPG_DEBUG_REG_0 register | 0x06 |  |
| 0x0087 | PPG_DEBUG_REG_1 | PPG_DEBUG_REG_1 register | 0x07 |  |
| 0x0088 | PPG_DEBUG_REG_2 | PPG_DEBUG_REG_2 register | 0x08 |  |
| 0x0089 | ADC_GENERAL_DEBUG_0 | ADC_GENERAL_DEBUG_0 register | 0x09 |  |
| 0x008A | ADC_GENERAL_DEBUG_1 | ADC_GENERAL_DEBUG_1 register | 0x0A |  |
| 0x008B | ADC_ERROR_CAP_9_DEBUG | ADC_ERROR_CAP_9_DEBUG register | 0x0B |  |
| 0x008C | ADC_ERROR_CAP_8_DEBUG | ADC_ERROR_CAP_8_DEBUG register | 0x0C |  |
| 0x008D | ADC_ERROR_CAP_7_DEBUG | ADC_ERROR_CAP_7_DEBUG register | 0x0D |  |
| 0x008E | ADC_ERROR_CAP_6_DEBUG | ADC_ERROR_CAP_6_DEBUG register | 0x0E |  |
| 0x008F | ADC_ERROR_CAP_5_DEBUG | ADC_ERROR_CAP_5_DEBUG register | 0x0F |  |
| 0x0090 | ADC_ERROR_CAP_4_DEBUG | ADC_ERROR_CAP_4_DEBUG register | 0x10 |  |
| 0x0091 | ADC_ERROR_CAP_3_DEBUG | ADC_ERROR_CAP_3_DEBUG register | 0x11 |  |
| 0x0092 | ADC_ERROR_CAP_2_DEBUG | ADC_ERROR_CAP_2_DEBUG register | 0x12 |  |
| 0x0093 | ADC_ERROR_CAP_1_DEBUG | ADC_ERROR_CAP_1_DEBUG register | 0x13 |  |
| 0x0094 | ADC_ERROR_CAP_0_DEBUG | ADC_ERROR_CAP_0_DEBUG register | 0x14 |  |
| 0x0095 | DEVICE_STATUS_REG | DEVICE_STATUS_REG register | 0x15 |  |
| 0x0096 | ELAB_STATUS_REG | ELAB_STATUS_REG register | 0x16 |  |
| 0x0097 | ANA_TEST_0 | ANA_TEST_0 register | 0x17 |  |
| 0x0098 | ANA_TEST_1 | ANA_TEST_1 register | 0x18 |  |
| 0x0099 | ANA_TEST_2 | ANA_TEST_2 register | 0x19 |  |
| 0x009A | ANA_TEST_3 | ANA_TEST_3 register | 0x1A |  |
| 0x009B | ANA_TEST_4 | ANA_TEST_4 register | 0x1B |  |
| 0x009C | ADC_DATA_OUT_DEBUG_LOW | ADC_DATA_OUT_DEBUG_l register | 0x1C |  |
| 0x009D | ADC_DATA_OUT_DEBUG_HIGH | ADC_DATA_OUT_DEBUG_h register | 0x1D |  |
| 0x009E | DEBUG_GSR_REG | DEBUG GSR REGISTER | 0x1E |  |
| 0x009F | B_IDAC_DEBUG_LOW | B_IDAC_DEBUG_l register | 0x1F |  |
| 0x00A0 | B_IDAC_DEBUG_HIGH | B_IDAC_DEBUG_h register | 0x20 |  |
| 0x00A1 | SPARE_0_REG | AFE SPARE 0 REGISTER | 0x21 |  |
| 0x00A2 | SPARE_1_REG | AFE SPARE 1 REGISTER | 0x22 |  |
| 0x00A3 | TEST_GPIO | TEST_GPIO REGISTER | 0x23 |  |
| 0x00A4 | PPG_STATUS | PPG_STATUS REGISTER | 0x24 |  |
| 0x00CF | VIREF_LDO1V8_CLK16M | VIREF_LDO1V8_CLK16M register | 0x0F | 0x03 |
| 0x00D2 | GPIO_CTRL_OUT | GPIO_CONTROL_OUTPUT register | 0x12 |  |
| 0x00D3 | GPIO1_CTRL_IN | GPIO1_CONTROL _INPUT register | 0x13 |  |
| 0x00D4 | GPIO2_CTRL_IN | GPIO2_CONTROL_INPUT register | 0x14 |  |
| 0x00D5 | GPIO3_CTRL_IN | GPIO3_CONTROL _INPUT register | 0x15 |  |
| 0x00D6 | GPIO4_CTRL_IN | GPIO4_CONTROL_INPUT register | 0x16 |  |
| 0x00D7 | PAD_CTRL | PAD_CTRL register | 0x17 |  |
| 0x00D8 | PAD_CTRL_GPIO | PAD_CTRL_GPIO register | 0x18 |  |
| 0x00D9 | OFFSET_CALIBRATION_DATA1_LOW | OFFSET_CALIBRATION_DATA1_l register | 0x19 |  |
| 0x00DA | OFFSET_CALIBRATION_DATA1_HIGH | OFFSET_CALIBRATION_DATA1_h register | 0x1A |  |
| 0x00DB | OFFSET_CALIBRATION_DATA2_LOW | OFFSET_CALIBRATION_DATA2_l register | 0x1B |  |
| 0x00DC | OFFSET_CALIBRATION_DATA2_HIGH | OFFSET_CALIBRATION_DATA2_h register | 0x1C |  |
| 0x00DD | OFFSET_CALIBRATION_DATA3_LOW | OFFSET_CALIBRATION_DATA3_l register | 0x1D |  |
| 0x00DE | OFFSET_CALIBRATION_DATA3_HIGH | OFFSET_CALIBRATION_DATA3_h register | 0x1E |  |
| 0x00DF | OFFSET_CALIBRATION_DATA4_LOW | OFFSET_CALIBRATION_DATA4_l register | 0x1F |  |
| 0x00E0 | OFFSET_CALIBRATION_DATA4_HIGH | OFFSET_CALIBRATION_DATA4_h register | 0x20 |  |
| 0x00E1 | OFFSET_CALIBRATION_DATA5_LOW | OFFSET_CALIBRATION_DATA5_l register | 0x21 |  |
| 0x00E2 | OFFSET_CALIBRATION_DATA5_HIGH | OFFSET_CALIBRATION_DATA5_h register | 0x22 |  |
| 0x00E3 | OFFSET_CALIBRATION_DATA6_LOW | OFFSET_CALIBRATION_DATA6_l register | 0x23 |  |
| 0x00E4 | OFFSET_CALIBRATION_DATA6_HIGH | OFFSET_CALIBRATION_DATA6_h register | 0x24 |  |
| 0x00E5 | OFFSET_CALIBRATION_DATA7_LOW | OFFSET_CALIBRATION_DATA7_l register | 0x25 |  |
| 0x00E6 | OFFSET_CALIBRATION_DATA7_HIGH | OFFSET_CALIBRATION_DATA7_h register | 0x26 |  |
| 0x00E7 | OFFSET_CALIBRATION_DATA8_LOW | OFFSET_CALIBRATION_DATA8_l register | 0x27 |  |
| 0x00E8 | OFFSET_CALIBRATION_DATA8_HIGH | OFFSET_CALIBRATION_DATA8_h register | 0x28 |  |
| 0x00E9 | OFFSET_CALIBRATION_DATA9_LOW | OFFSET_CALIBRATION_DATA9_l register | 0x29 |  |
| 0x00EA | OFFSET_CALIBRATION_DATA9_HIGH | OFFSET_CALIBRATION_DATA9_h register | 0x2A |  |
| 0x00EB | RESET_ASYNC_BIO_CH | RESET_ASYNC_BIO_CH REGISTER | 0x2B |  |

Table 35:  AFE_BLOCK register list.

In the following are reported the configuration internal registers of HOST_IF I2C/SPI AHB. For more information and details regarding to IP functionalities and register it’s need to reference the IP documentation.

| Register Name | Address Offset | Reset Value | Access | Bits | Field name |
| --- | --- | --- | --- | --- | --- |
| PAGE_SEL | 0x40 | 0x00 | RW | 7:2 | Unused |
|  |  |  |  | 1:0 | In case of external 4 pages registers expansion these bits are used to choose the page |
| ERROR_BYTE | 0x41 | 0x00 | RW | 7:0 | Byte send to the I2C/SPI master in case of error |
| FLUSH_FIFO | 0x4B | 0x00 | RW | 7:2 | Unused |
|  |  |  |  | 1 | Force flush of write fifo. Write 0 and then 1 to activate the function |
|  |  |  |  | 0 | Force flush of read fifo. Write 0 and then 1 to activate the function |
| FIFO_STATUS | 0x4C | 0x00 | RO | 7:4 | Unused |
|  |  |  |  | 3 | Read fifo full |
|  |  |  |  | 2 | Read fifo not empty |
|  |  |  |  | 1 | Write fifo full |
|  |  |  |  | 0 | Write fifo not empty |
| ERR_ITF | 0x4E | 0x00 | RC | 7:6 | Unused |
|  |  |  |  | 5 | Unexpected read fifo empty |
|  |  |  |  | 4 | Unexpected write fifo full |
|  |  |  |  | 3 | Number of bytes write operation already done, read operation is expected |
|  |  |  |  | 2 | Read operation done without writing nr bytes before |
|  |  |  |  | 1 | Write fifo flushed since not empty before write operation |
|  |  |  |  | 0 | Read fifo flushed since not empty before read operation |
| ERR_AHB | 0x4F | 0x00 | RC | 7:5 | Unused |
|  |  |  |  | 4 | Time out reached from single write for BYTE_NR to following read |
|  |  |  |  | 3 | Unused |
|  |  |  |  | 2 | AHB error on read |
|  |  |  |  | 1 | AHB error on write |
|  |  |  |  | 0 | Unused |
| SAM_ADDRESS_BYTE0 | 0x50 | 0x00 | RW | 7:0 | First (LSB) byte of the AHB address |
| SAM_ADDRESS_BYTE1 | 0x51 | 0x00 | RW | 7:0 | Second byte of the AHB address |
| SAM_ADDRESS_BYTE2 | 0x52 | 0x00 | RW | 7:0 | Third byte of the AHB address |
| SAM_ADDRESS_BYTE3 | 0x53 | 0x00 | RW | 7:0 | Fourth (MSB) byte of the AHB address |
| CTRL_REG_0 | 0x57 | 0x00 | RW | 7:1 | Unused |
|  |  |  |  | 0 | Increment address flag. If ‘1’ the AHB address is incremented after each access. If ‘0’ the address is not incremented (fifo access) |
| CTRL_REG_1 | 0x58 | 0x00 | RW | 7:1 | Unused |
|  |  |  |  | 0 | I2C disable. If ‘1’ the i2c slave is switched off. Every request from master gets all ‘0’ |
| CTRL_REG_2 | 0x59 | 0x00 | RW | 7:1 | Unused |
|  |  |  |  | 0 | AHB access. If ‘1’ the access is by word, by byte otherwise |
| CTRL_REG_3 | 0x5A | 0x00 | RW | 7:0 | Unused. Available for future expansion |
| CTRL_REG_4 | 0x5A | 0x00 | RW | 7:0 | Unused. Available for future expansion |
| CTRL_REG_5 | 0x5A | 0x00 | RW | 7:0 | Unused. Available for future expansion |
| CTRL_REG_6 | 0x5A | 0x00 | RW | 7:0 | Unused. Available for future expansion |
| CTRL_REG_7 | 0x5A | 0x00 | RW | 7:0 | Connected to 8 bit output o_ctrl_reg_7. They can be used during integration |
| IP_VERSION | 0x5F | 0x00 | RO | 7:0 | Byte to specify IP version. To be used at integration time by means of IP parameter "ip_version" |

Table 36: I2C/SPI internal registers.

# Soft Reset Procedures

Main Controller Soft Reset

This procedure reset all the Main Controller FSMs and related IPs, reperforming the BOOT Phase without reset the Regmap.

[DDS_STBIO1_1114] Requirement:

To perform a SOFT Reset procedure, the user shall perform the following steps:

- If Mode Operation has been set to 1, shall be reset to 0

- Set the Soft Reset Main Controller Bit (SW_RESETN_REG[0]) to 0 for at least 1 ms

- Set the Soft Reset Main Controller Bit (SW_RESETN_REG[0]) to 1

- Wait for the DEVICE_STATUS_REGISTER to reach the IDLE Value

[END]

# Measurement Cycle Examples

The Device Activity can be summarized by the following Flowchart:

Figure 19: Device Activity Flowchart

In the next paragraphs, each phase will be explicated.

BOOT Phase

[DDS_STBIO1_0114] Requirement:

At the release of the POR signal from the analog domain, the PMU shall manage the turn-on of the LDO1V8 and starts clocks Start-Up Sequence (reported in PMU IPOS) [TO: IPOS_PMU] [END]

[DDS_STBIO1_0115] Requirement:

After At least 2 ms from the Release of the POR, the reset of the digital domain shall be de-asserted (an overlook on this part is described in the PMU IPOS) and the device shall start the Boot Phase [TO: IPOS_PMU] [END]

[DDS_STBIO1_0116] Requirement:

To check the end of the BOOT phase, the user shall read the main_controller_status_register (DEVICE_STATUS_REG register [2:0]) and follow these steps:

- Read the DEVICE_STATUS_REG [2:0]

- Check if the Value is 1 (BOOT phase still in process)

- If the value is 2, then the BOOT phase is ended and the device is in IDLE. [TO: IPOS_MAIN_CTRL] [END]

Figure 20: Boot Phase Flow

Configuration Phase

[DDS_STBIO1_ 0117] Requirement:

During the Configuration Phase (IDLE STATE of the Device_FSM), the user shall be able to:

- Configure Bio-Channel or Optical Channel, the ADC Calibration and then Start a measurement cycle.

- Put the device in Debug-Mode. [TO: IPOS_MAIN_CTRL] [END]

Figure 21: Configuration Phase Diagram

The start and the end of a Measurement Cycle is selected by the writing of the dedicated register as described in following examples.

Measurement Cycle

In this paragraph, different examples of Measurement Cycle will be analyzed to give the Application User an overview of the Device Workflow. All measurements will be performed in Data Storage Mode

### ECG Only

This configuration generates each time slot, two averaged ECG data, which are ECG AC and ECG DC, respectively.

- TIME SLOT LENGTH:

[DDS_STBIO1_0118] Requirement:

  - The user shall configure time slot duration in DEVICE_CONFIG_OPERATION register [4:2] [TO: IPOS_MAIN_CTRL] [END]

- SELECT CHANNEL:

[DDS_STBIO1_0119] Requirement:

  - The user shall select active channel, in this case only ECG0_SEL and DS_SEL (Data Storage Mode) in DEVICE_CONFIG_CHANNEL register writing  0x21. [TO: IPOS_MAIN_CTRL] [END]

- SELECT DIVISION INDEX FOR CONFIGURED CHANNELS:

[DDS_STBIO1_0120] Requirement:

  - The user shall select a division index for ECG, configuring M_ECG_l and M_ECG_h in SET_ECG_FREQ_l register and SET_ECG_FREQ_h register. [TO: IPOS_MAIN_CTRL] [END]

- CONFIGURE ADC SAMPLING PERIOD:

[DDS_STBIO1_0121] Requirement:

  - The user shall set the number of clocks for sampling ECG data from ADC in ADC_CONFIG_ECG register. [TO: IPOS_MAIN_CTRL] [END]

- GENERAL CONFIG:

[DDS_STBIO1_0122] Requirement:

  - The User shall set at 1 HC_EN in ECG_CTRL register. [TO: IPOS_MAIN_CTRL] [END]

- SELECT OPERATIVE MODE:

[DDS_STBIO1_0123] Requirement:

  - The user shall start measurement writing register DEVICE_CONFIG_OPERATION [0] at 1. [TO: IPOS_MAIN_CTRL] [END]

- CHECK DATA:

[DDS_STBIO1_0124] Requirement:

  - To check data in unique FIFO (default config) the user shall check the first MSB which will have to be equal to the table in DATA TAG Table. [TO: IPOS_MAIN_CTRL] [END]

Figure 22: ECG ONLY FLOWCHART

### ECG and BIA

This configuration generates each time slot, two averaged ECG data, which are ECG AC and ECG DC, respectively and four averaged BIA data which are BIA_AC_P, BIA_DC_P, BIA_AC_Q and BIA_DC_Q.

- TIME SLOT LENGTH:

[DDS_STBIO1_0125] Requirement:

- The user shall configure the time slot duration in DEVICE_CONFIG_OPERATION register [4:2]. [TO: IPOS_MAIN_CTRL] [END]

- SELECT CHANNEL:

[DDS_STBIO1_0126] Requirement:

- The user shall select active channel, in this case ECG0_SEL, BIA_SEL and DS_SEL (Data Storage Mode) in DEVICE_CONFIG_CHANNEL writing 0x23. [TO: IPOS_MAIN_CTRL] [END]

- SELECT DIVISION INDEX FOR CONFIGURED CHANNELS:

[DDS_STBIO1_0127] Requirement:

- The user shall select a division index for ECG, configuring M_ECG_l and M_ECG_h in SET_ECG_FREQ_l register and SET_ECG_FREQ_h register. [TO: IPOS_MAIN_CTRL] [END]

[DDS_STBIO1_0128] Requirement:

- The user shall select a division index for ECG, configuring M_BIA_l and M_BIA_h in

SET_BIA_FREQ_l register and SET_BIA_FREQ_h register. [TO: IPOS_MAIN_CTRL] [END]

- CONFIGURE ADC SAMPLING PERIOD:

[DDS_STBIO1_0129] Requirement:

- The user shall set number of clocks for sampling ECG and BIA data from ADC in ADC_CONFIG_ECG register. [TO: IPOS_MAIN_CTRL] [END]

- GENERAL CONFIG:

[DDS_STBIO1_0130] Requirement:

- The user shall set at 1 HC_EN in ECG_CTRL register. [TO: IPOS_MAIN_CTRL] [END]

[DDS_STBIO1_0140] Requirement:

- The user shall set at 1 IMP_EN in IMP_CTRL register. [TO: IPOS_MAIN_CTRL] [END]

- SELECT OPERATIVE MODE:

[DDS_STBIO1_0141] Requirement:

  - The user shall start measurement writing register DEVICE_CONFIG_OPERATION [0] at 1. [TO: IPOS_MAIN_CTRL] [END]

- CHECK DATA:

[DDS_STBIO1_0142] Requirement:

- To check data in unique FIFO (default config) the user shall check the first MSB which will have to be equal to the one described in the DATA TAG Table. [TO: IPOS_MAIN_CTRL] [END]

Figure 23: ECG and BIA Measurement Flowchart

### PPG Only

This configuration generates each time slot, a number of PPG DATA according to user configuration.

- TIME SLOT LENGTH:

[DDS_STBIO1_0143] Requirement:

  - The User shall configure time slot duration in DEVICE_CONFIG_OPERATION register [4:2].

- SELECT CHANNEL:

[DDS_STBIO1_0144] Requirement:

  - The user shall select active channel, in this case only PPG_SEL and DS_SEL (data Storage) in DEVICE_CONFIG_CHANNEL register shall be set to 0x30.

- SELECT DIVISION INDEX FOR CONFIGURED CHANNELS:

[DDS_STBIO1_0145] Requirement:

  - The user shall configure a division index PPG, writing M_PPG_l and M_PPG_h in SET_PPG_FREQ_l register and SET_PPG_FREQ_h register.

- CONFIGURE ADC SAMPLING PERIOD:

[DDS_STBIO1_0146] Requirement:

  - The user shall configure the number of clocks for sampling PPG data from ADC in ADC_CONFIG_PPG register.

- PPG FRAMES CONFIG:

[DDS_STBIO1_0147] Requirement:

The user shall configure each frame following these steps:

  - ALC PARAMETERS:

  - T1 in PPG_ALC_CONFIG_PARAM_1 register.

  - T2, GAIN_START_SET, GAIN_MAX_SET in PPG_ALC_CONFIG_PARAM_2 register.

  - DIGITAL RAMP PARAMETERS:

- N_MAX, T_DELTA in DIGITAL_RAMP_CONFIG_PARAM_1 register.

- DELTA, T_DELTA1 in DIGITAL_RAMP_CONFIG_PARAM_2 register.

- DELTA1 in DIGITAL_RAMP_CONFIG_PARAM_3 register.

  - GENERAL PPG PARAMETERS:

- N_PPG_FRAME, WAVE_VELOCITY_MODE, SEL_PPG_ELAB in GENERAL_PPG_PARAMETERS_1 register.

- T_TX_STARTUP in GENERAL_PPG_PARAMETERS_2 register.

- T_RX_SET, PPG_START_UP_TIME_l in GENERAL_PPG_PARAMETERS_3 register.

- PPG_START_UP_TIME_h in GENERAL_PPG_PARAMETERS_4 register.

  - PARAMETERS FOR EACH FRAME CONFIGURED BY USER:

- RX_CH_FRAME_{n}, FRAME_{n}_SAMPLES, FRAME_{n}_REPETITION in FRAME_n_PARAMETERS_1 register.

- BTIA_GAIN_FRAME_{n}, TX_LEDSET_FRAME_l_{n} in FRAME_n_PARAMETERS_2 register.

- TX_LEDSET_FRAME_h_{n}, TX_LEDOFF_FRAME_l_{n} in FRAME_n_PARAMETERS_3 register.

- TX_LEDOFF_FRAME_h_{n} in FRAME_n_PARAMETERS_4 register.

- IDAC_DC_PPG_FRAME_{n} in FRAME_n_PARAMETERS_5 register.

- EN_TX_i in FRAME_n_PARAMETERS_6 register.

- CURR_SEL_TX_FRAME{n} in FRAME_n_PARAMETERS_7 register.

- TX1_OUTAB_SEL_FRAME_{n}, TX2_OUTAB_SEL_FRAME_{n}, TX3_OUTAB_SEL_FRAME_{n}, TX4_OUTAB_SEL_FRAME_{n} in FRAME_n_PARAMETERS_10 register.

- SELECT OPERATIVE MODE:

[DDS_STBIO1_0148] Requirement:

- The user shall write DEVICE_CONFIG_OPERATION register [0] at 1.

- CHECK DATA:

[DDS_STBIO1_0149] Requirement:

- To check data in unique FIFO (default config) the user shall check the first MSB. PPG DATA has 20 bits of data and 4 of TAG. The 4 bits, from 0 to 11, are used to identify the frame (as described in the table in DATA TAG table).

Figure 24: PPG Measurement Flowchart

# SCAN

Top Level Approach

OUTPUT

- Scan out 1 (GPIO3/PMBF)

- Scan out 2 (SDA_SDI)

INPUT FROM PADS

- Scan in 1 (GPIO4/PMBS)

- Scan in 2 (SCLK)

- Scan clk (SH_CK)

- Scan en (GPIO0)

- Scan reset (CS)

INPUT FROM REGISTER

- Scan mode (ADDRESS=0x3E, BIT=0x01)

- SCAN_COMPR_EN (ADDRESS=0x3E, BIT=0x02)

ANALOG PADS:

- sense internal V1V2 (GPIO1)

- force = V1V2 SUPPLY (GPIO2)

- T_force_v1v2 (SH_SDA)

# BIST

Reference documents

| Current Version | Document | Author | Change Description |
| --- | --- | --- | --- |
| 1.0-17.00 | /prj/ams_DELIVERIES/H9A_STBIO1/memorycuts/H9A_STBIO1--GenericMBIST_CONFIG--BIST_STBIO1/1.0-17.00/ /doc/ref_manual/generic_mbist_um.pdf | STMicroelectronics TRnD | User manual, release notes and KPS for the GenericMBIST |
| 1.0-17.00 | /prj/ams_DELIVERIES/H9A_STBIO1/memorycuts/H9A_STBIO1--GenericMBIST_CONFIG--BIST_STBIO1/1.0-17.00/ /doc/user_manual/generic_mbist_rm.pdf | STMicroelectronics TRnD | Reference manual for the GenericMBIST |

Memory BIST Architecture

The BIST solution consists of memory-wrapper “collar” and common master “controller” and an IEEE 1149.1 compliant Test Access Port (TAP).

### MBIST Collar

The BIST collar is a wrapper which is placed closest to memory. It executes the instructions received from controller in the form of march elements to test the memory. It runs at the speed of the memory under test.

After testing it reports the execution status to the controller and manages all the BIST status, diagnosis, repair information and debug data. The collar also provides support for testing multiple memories in parallel or in sequential order.

### MBIST Controller

The controller acts as an interface between the collars and the user. All user BIST control is done through the controller. The controller controls various testing and reporting activities of the memory test. It allows various mode selections and programs the pre-stored synthesized algorithms instructions into collars attached to it.

Main functionalities of controller are:

•	Mode management (RunBIST mode, Retention mode etc.)

•	Program the configurable collar with default algorithms, march element by march element and initiate the activities on memory through collars by launching the march run.

•	Compute the Global status based on the individual status from the collar.

TAP

•	It includes a TAP controller, a 7-bit instruction register, three test data registers, and the bypass register.

•	This responds to the control sequences supplied through the test access port (TAP) and generates the control signals required for BIST operation

### Top Level Approach

Figure 66: Top Level Controller Configuration

In this configuration, there is only one controller for the entire chip. All associated collars are spread into the design (at top level and within subblocks)

Figure 25 Top Level Controller Configuration

OUTPUT

- Bend: Indicates that all the activities of the test run of all Memories in the current BIST mode are complete

- Bfail: Indicates bfail for a specific memory. There are as many bfail signals as the number of memories. Bfail also goes high at the first cycle of each march element run. This pulse should be used as a reference in clk_m domain to calculate the number of cycles after which bfail went high in case of a failure in a specific march sequence run.

- Tdo:TAP serial output Updated on falling edge of tck

INPUT FROM PAD

- Tck: TAP clock

- Tdi: TAP serial input Sampled on rising edge of tck

- Tms: TAP test mode select Sampled on rising edge of tck

- Trstn: TAP reset

INPUT FROM REGISTER

- Memsafe (ADDRESS=0x3E, BIT=0x05): to globally deactivate the memory by making CSN high internally

- Mem_atpg_mode (ADDRESS=0x3E, BIT=0x06): Used to constraint few memory inputs and improve controllability and observability.

ANALOG PADS:

- sense internal V1V2 (GPIO1)

- force = V1V2 SUPPLY (GPIO2)

# Digital DFT

Scan mode

[DDS_STBIO1_0699] Requirement:

After the initial boot phase, only with SPI interface, to turn on the Scan mode shall be:

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable debug mode by writing to regmap (ADDRESS=0x3E, DATA=0x01)

Figure 26 scan mode activation timing

[End]

Debug mode

[DDS_STBIO1_0700] Requirement:

After the initial boot phase, to turn on the Debug mode shall be:

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable debug mode by writing to regmap (ADDRESS=0x3E, DATA=0x08)

[End]

Figure 27 debug mode activation timing

ADC TEST LOW NOISE

[DDS_STBIO1_0701] Requirement:

To turn on the ADC TEST LOW NOISE mode shall be:

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0xF, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable ADC in debug mode (ADDRESS=0x09, DATA=0x06)

- Select page 2 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable ADC TEST LOW NOISE and input of ADC from GPIO (ADDRESS=0x0A, DATA=0x14)

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable debug mode by writing to regmap (ADDRESS=0x3E, DATA=0x08)

[End]

[DDS_STBIO1_0702] Requirement:

To turn on the ADC TEST LOW NOISE mode shall be:

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0xF, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable ADC in debug mode (ADDRESS=0x09, DATA=0x06)

- Select page 2 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable ADC TEST LOW NOISE and input of ADC from GPIO (ADDRESS=0x0A, DATA=0x14)

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable debug mode by writing to regmap (ADDRESS=0x3E, DATA=0x08)

[End]

ADC TEST FAST

[DDS_STBIO1_1507] Requirement:

To turn on the ADC TEST LOW NOISE mode shall be:

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0xF, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable ADC in debug mode (ADDRESS=0x09, DATA=0x06)

- Select page 2 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable ADC TEST LOW NOISE and input of ADC from GPIO (ADDRESS=0x0A, DATA=0x0C)

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x02)

- Enable debug mode by writing to regmap (ADDRESS=0x3E, DATA=0x08)

[End]

BIST MODE

[DDS_STBIO1_2507] Requirement:

After the initial boot phase, to turn on the Debug mode shall be:

- Select page 3 of the regmap (ADDRESS=0x40, DATA=0x03)

- Enable LDO 1V8 by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x02)

- Enable clock by writing to regmap VIREF_LDO1V8_CLK16M register (ADDRESS=0x0F, DATA=0x12)

- Enable the power island [DDS_STBIO1_0507]

- Enable bist mode by writing to regmap (ADDRESS=0x3E, DATA=0x04)

[End]

Figure 28 bist mode activation timing

# References

- ADSP Document: ADSP micro Firmware